3GPP TSG-RAN WG2 #118-e	R2-2205880

# Online, May 9 - 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0728 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.306 CR for sidelink relay UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In 4.2.16.1.1, the use of ‘basic’ term in the definitions of relayUE-Operation-L2-r17 and remoteUE-Operation-L2-r17 parameters is misleading. There is no differentiation of basic and any other feature set for L2 Relay operation. Only one mode of operation is defined for L2 Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove ‘basic’ term from the definitions of relayUE-Operation-L2-r17 and remoteUE-Operation-L2-r17 parameters in 4.2.16.1.1 and Annex on Feature list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For L2 relay operation capabilities unclear operation is remained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.16.1.1, Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 4.2.16	Sidelink Parameters

#### 4.2.16.1	Sidelink Parameters in NR

##### 4.2.16.1.1	Sidelink General Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumReleaseSidelink-r16 Indicates the access stratum release for NR sidelink communication the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| relayUE-Operation-L2-r17 Indicates whether NR L2 sidelink relay UE operation is supported by the UE. | UE | No | No | No |
| remoteUE-Operation-L2-r17 Indicates whether NR L2 sidelink remote UE operation is supported by the UE. | UE | No | No | No |
| remoteUE-PathSwitchToIdleInactiveRelay-r17 Indicates whether L2 sidelink remote UE supports direct to indirect path switch with target relay in RRC_IDLE or RRC_INACTIVE state. | UE | No | No | No |

END OF CHANGES

# Annex: RAN2 determined UE capabilities in the feature list format

Table X: Layer-2 and Layer-3 feature list for NR_SL_Relay-Core

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. NR_SL_Relay-Core. | X-1 | Band combination list supporting transmission and reception of relay discovery | <To be added after R2 agreement> |  | BandCombinationListSidelinkRelayDiscovery-r17 | RF-Parameters | No | No |  | Optional |
|  | X-2 | Band combination list supporting transmission and reception of non-relay discovery | <To be added after R2 agreement> |  | BandCombinationListSidelinkNonRelayDiscovery-r17 | RF-Parameters | No | No |  | Optional |
|  | X-3 | NR L2 sidelink relay UE operation | <To be added after R2 agreement> |  | relayUE-Operation-L2-r17 | SidelinkParametersNR-r16 | No | No |  | Optional |
|  | X-4 | NR L2 sidelink remote UE operation | <To be added after R2 agreement> |  | remoteUE-Operation-L2-r17 | SidelinkParametersNR-r16 | No | No |  | Optional |
|  | X-5 | Remote UE performs handover to idle/inactive relay UE | <To be added after R2 agreement> |  | remoteUE-PathSwitchToIdleInactiveRelay-r17 | SidelinkParametersNR-r16 | No | No |  | Optional |