3GPP TSG-RAN WG2 #118                                                                                         R2-220xxxx

eMeeting, 09th May – 20th May, 2022

Agenda Item:	    6.24.1

Source: 	    Apple

Title:		    Summary of [AT118-e][041][NR17] FR2 UL gap (Apple)

Document for:	    Discussion and decision

# 1 Introduction

This is the summary of the following email discussion.

[AT118-e][041][NR17] FR2 UL gap (Apple)

Scope: Treat R2-2205666, R2-2204507, R2-2205659, R2-2205667, R2-2205392

Ph1 Determine agreeable parts, Ph2 agree CRs

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

# 2 Discussion

## 2.1 On RAN4 LS in R2-2204507

Rapporteur thinks this LS can be just noted as RAN4 has no questions to RAN2.

Question 1: Do companies agree to note the RAN4 LS R2-2204507?

| Company | Yes / No | Comments |
| --- | --- | --- |

## 2.2 RIL(s) and Class 0 issues (R2-2205666, R2-2205667)

The conclusion from R2-2205666 is on FR2 UL gap RIL(s) and Class 0 issues is copied below:

Proposal 1: Agree the following RIL(s) and Class 0 issues and reflect them in rapporteur CR.

- RIL: Z151, Z152, A803, A804, A807, A808

- Class 0 issues: 155, 156, 193, 425

Question 2: Do companies agree the proposal 1 in [1]?

| Company | Yes / No | Comments |
| --- | --- | --- |

Question 3: Do companies have comments on the CR R2-2205667 to address the RIL(s) and Class 0 issues?

It is also fine to add the comments into the CR.

| Company | Comments |
| --- | --- |

## 2.3 MAC CR in R2-2205659

The reason for change is copied below:

RAN4 agreed with the following UE behavior in FR2 UL gap in R4-2206511. During UL gaps, except for the signals used for random access procedure according to TS 38.321, CG-PUSCH (type 1 and 2) and PUCCH allocations for SR and LRR [and for the signals used for other RAN4 agreed procedures], UE is not required to conduct transmission to the corresponding NR serving cells in FR2 single CC, intra-band CA. For inter-band FR2-FR2 CA/DC, UE may or may not be required to conduct transmission to the corresponding NR serving cells based on UE capability whether UL transmission within a gap is feasible. Note that CR 1191 (R2-2204231) already captured that RACH procedure is prioritized during FR2 UL gap.

The summary of change:

Capture that during FR2 UL gap, if UE does not support UL Tx transmission, UE should not perform certain actions. CG-PUSCH, PUCCH for SR and LRR are the exceptions.

Question 4: Do companies have comments on R2-2205659?

It is also fine to add comments to the CR.

| Company | Comments on the change |
| --- | --- |

## 2.4 Discussions on release of UAI preference in R2-2205392

The discussion in R2-2205392 is not limited to FR2 UL gap, but a rather generic one on UAI message.

Observation 1: There is some ambiguity with the release of FR2 UL gap preference indication in the UAI. Observation 2: The issue with the release of UAI preference indications exist for several UAI fields. And proposed the following: Proposal 1: Add NOTE to 5.7.4.3 indicating the following: "When UE includes an indication that requires not including certain fields, UE is still required to include the parent field to ensure network comprehends the message as requesting release of a previously indicated configuration."

Question 5: Do companies agree with Proposal 1 in R2-2205392 to add a note.

| Company | Yes / No | Comments on the Note in Proposal 1 |
| --- | --- | --- |

# 3 Conclusions

[TBA]

# 4 References

[1] R2-2204507	LS to RAN2 on UL gap in FR2 RF enhancement (R4-2206608; contact: Apple)	RAN4	LS in	Rel-17	NR_RF_FR2_req_enh2-Core	To:RAN2	Cc:RAN1

[2] R2-2205666	Summary of [Pre118-e][004][NR17 FR2 UL Gap] 38331 CR and rapporteur resolutions (Apple)	Apple	discussion	Rel-17	NR_RF_FR2_req_enh2	Late

[3] R2-2205659	Correction on FR2 UL gap	Apple	CR	Rel-17	38.321	17.0.0	1279	-	F	NR_RF_FR2_req_enh2

[4] R2-2205667	Addressing FR2 UL gap RILs [Z151, Z152, A803, A804, A807, A808]	Apple	CR	Rel-17	38.331	17.0.0	3110	-	F	NR_RF_FR2_req_enh2	Late

[5] R2-2205392	[N129] Corrections to FR2 UL gaps	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_RF_FR2_req_enh2-Core	Late

Chair: General?