3GPP TSG-RAN WG2 #119e                                                                                        R2-220xxxx

eMeeting, 17th – 26th August, 2022

Agenda Item:	    5.1.3.3

Source: 	    MediaTek Inc.

Title:		    [Draft] Report of [AT119-e][012][NR1516] UE capabilities (MediaTek)

Document for:	    Discussion and decision

# 1 Introduction

This is report for the following AT119-e mail discussion.

[AT119-e][012][NR1516] UE capabilities (MediaTek)

Scope: Treat R2-2206911, R2-2208501, R2-2208502, R2-2208503, R2-2208504, R2-2207640, R2-2207641, R2-2207049, R2-2207085, R2-2207086, R2-2207094, R2-2207095, R2-2207113, R2-2207114, R2-2208027, R2-2208028, R2-2207331, R2-2207332, R2-2208505, R2-2208506. Determine agreeable parts, For agreeable parts, agree CRs.

Intended outcome: Report, Agreed CRs, LS out if applicable

Deadline: Schedule 1

This discussion follows Schedule 1, which is organized as follow:

Discussions with Deadline Schedule 1:

A first round with Deadline for comments W1 Friday Aug 19th 1400 UTC to settle scope what is agreeable etc

A Final round with Final deadline W2 Thursday Aug 25th 1200 UTC to settle details / agree CRs etc.

For all discussions: Additional deadlines check points etc if needed are defined by the Rapporteur of each discussion respectively. In case some parts of an email discussion need more time, doesn’t converge, need on-line treatment, then please contact the chair.

# 2 Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| MediaTek (Rapp) | Mutai Morton Lin | morton.lin@mediatek.com |
| Huawei, HiSilicon | Tong Sha | shatong3@hisilicon.com |
| Qualcomm Incorporated | Masato Kitazoe | mkitazoe@qti.qualcomm.com |

# 3 Discussion

## 3.1 First round: Intended to determine agreeable parts

### Power Class 1.5

[1] R2-2207049	On early implementation and capability signaling of Power Class 1.5	MediaTek Inc.	discussion	Rel-16	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

[2] R2-2207094	Make PC1.5 an early implementation candidate	MediaTek Inc.	CR	Rel-16	38.331	16.9.0	3454	-	F	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

[3] R2-2207095	Make PC1.5 an early implementation candidate	MediaTek Inc.	CR	Rel-17	38.331	17.1.0	3455	-	A	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

The discussion paper [1] indicates the necessity in the early implementation of PC1.5 and discloses a special power class reporting strategy which is unclear how would network treat such a UE capability. It proposed:

- Add PC 1.5 in the Table C-1 of TS 38.331 as the earliest implementable release is Rel-15.

- The principle “Network regards the highest supported power class among all advertised ones as the supported capability” shall be captured in RAN2 specification.

Therefore CRs [2][3] propose to make PC1.5 an early implementable feature to resolve prerequisite absence issue.

Q1: Do companies agree with the intention of the CRs [2][3]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm Incorporated | Yes, but | Why is the proposed text only limit the early implementability to intra-band EN-DC, while the PC1.5 was introduced also for n41 NR SA? |

[4] R2-2207085	PC1.5 and legacy power class capability reporting clarification	MediaTek Inc.	CR	Rel-16	38.306	16.9.0	0795	-	F	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

[5] R2-2207086	PC1.5 and legacy power class capability reporting clarification	MediaTek Inc.	CR	Rel-17	38.306	17.1.0	0796	-	A	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

Based on the proposal in discussion paper [1], CRs [4][5] propose to clarify network interpretation when UE reports more than one PC (to gain better UL coverage in legacy NW).

Q2: Do companies agree with the intention of the CRs [4][5]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm Incorporated | Yes |  |

### CSI-RS capability signaling

[6] R2-2207113	Clarification on codebookParametersPerBC parameter for extension of CSI-RS capabilities reporting	MediaTek Inc.	CR	Rel-16	38.331	16.9.0	3452	-	F	NR_newRAT-Core, TEI16

[7] R2-2207114	Clarification on codebookParametersPerBC parameter for extension of CSI-RS capabilities reporting	MediaTek Inc.	CR	Rel-17	38.331	17.1.0	3453	-	A	NR_newRAT-Core, TEI16

CRs [6][7] point out a conflict that UE is required to report codebookParametersPerBC parameter under CA-ParametersNR for non-CA band combination and propose to clarify reporting condition.

Q3: Do companies agree with the intention of the CRs [6][7]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | This is NBC. In current specification, it is required that both codebookParametersPerBC and codebookParametersPerBand should be reported together by UE if supported, regardless whether the BC is for CA or not. Then for the legacy NW, the NW would consider a UE not reporting codebookParametersPerBC as not supporting the corresponding enhanced codebook capability. If a UE is implemented according to the CR, then the enhanced codebooks can never be configured for a non-CA BC. We should avoid such a NBC change for a Rel-16 capability at this stage. |
| Qualcomm Incorporated | Yes | But backward compatibility to legacy network must be assessed by infra-vendors. |

### Beam management

[8] R2-2207331	Correction on beamManagementSSB-CSI-RS	Qualcomm Incorporated	CR	Rel-16	38.306	16.9.0	0765	-	F	TEI16

[9] R2-2207332	Correction on beamManagementSSB-CSI-RS	Qualcomm Incorporated	CR	Rel-17	38.306	17.1.0	0766	-	A	TEI16

CRs [8][9] indicate that the current R2 specification does not correctly capture the RAN1 conclusion in RAN1#101-e regarding to the beam management capability and propose corresponding corrections.

Q4: Do companies agree with the intention of the CRs [8][9]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | We do not see the description in the CR reflected in the RAN1 feature list nor in the LS to RAN2, thus it is not necessary to be captured in the 38.306. Besides, for FR2, it is confused how the UE indicates the capability according to the smallest SCS configured for PDSCH, since UE has no idea what is the smallest SCS of the serving cell(s) configured by the NW when reporting the capability information. Assuming that UE reports the capability according to the supported smallest SCS for FR2 band (e.g. 60kHz), then it is contradictory with the conclusion of the smallest SCS configured for PDSCH. We suggest to send RAN1 a LS, asking to clarify what does it mean by “For FR2, the parameter indicates the total number of resources across serving cells within 1 slot of the smallest subcarrier spacing configured for PDSCH in FR2”. |
| Qualcomm Incorporated | Yes | At least the current text is incorrect and must be corrected. As for the FR2 capability (Huawei’s question), it is our understanding that the UE capability is semi-static regardless of the size of the smallest SCS that may be configured. We are OK sending an LS to RAN1, if majority prefer. |

### MMSE-IRC

[10] R2-2207640	CR to TS 38.306 on UE capability of MMSE-IRC receiver	China Telecom	CR	Rel-15	38.306	15.17.0	0775	-	F	NR_newRAT-Core

[11] R2-2207641	CR to TS 38.306 on UE capability of MMSE-IRC receiver	China Telecom	CR	Rel-16	38.306	16.9.0	0776	-	A	NR_newRAT-Core

CRs [10][11] propose new MMSE-IRC receiver capability shall be specified in TS 38.306 for Rel-15 based on RAN4 LS.

Q5: Do companies agree with the intention of the CRs [10][11]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes, but | Fine in principle. We do not see the need to talk about FDD/TDD differentiation for mandatory feature without signalling. Ideally, reference to RAN4 specification / section should be added so that it is clear what the corresponding feature is. |

### Aperiodic CSI-RS

[12] R2-2208027	Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306)	Ericsson	CR	Rel-17	38.306	17.1.0	0786	-	A	LTE_NR_DC_CA_enh-Core

[13] R2-2208028	Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306)	Ericsson	CR	Rel-16	38.306	16.9.0	0787	-	F	LTE_NR_DC_CA_enh-Core

CRs [12][13] proposed to clarify the meaning of the term. "A-CSI trigger" and to remove prerequisite capability description because the prerequisite had been already mandated to report.

Q6: Do companies agree with the intention of the CRs [12][13]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes |  |

### PDCCH blind detection

[14] R2-2206911	Reply LS on PDCCH Blind Detection in CA (R1-2205320; contact: Huawei)	RAN1	LS in	Rel-17	NR_L1enh_URLLC-Core	To:RAN2

[15] R2-2208501	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-16	38.306	16.9.0	0789	-	F	NR_L1enh_URLLC-Core

[16] R2-2208502	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-17	38.306	17.1.0	0790	-	A	NR_L1enh_URLLC-Core

[17] R2-2208503	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-16	38.331	16.9.0	3429	-	F	NR_L1enh_URLLC-Core

[18] R2-2208504	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-17	38.331	17.1.0	3430	-	A	NR_L1enh_URLLC-Core

The TP in CRs [15][16][17][18] to capture agreements from RAN1 LS R2-2200079 and R2-2206911 [14]. Late non-critical extensions are added to avoid NBC problem.

Q6: Do companies agree with the intention of the CRs [15][16][17][18]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes |  |

### PUSCH repetition

[19] R2-2208505	Clarification on pusch-RepetitionTypeA-r16 capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.9.0	0791	-	F	NR_L1enh_URLLC-Core

[20] R2-2208506	Clarification on pusch-RepetitionTypeA-r16 capability	Huawei, HiSilicon	CR	Rel-17	38.306	17.1.0	0792	-	A	NR_L1enh_URLLC-Core

CRs [19][20] propose to capture prerequisites for FG11-6 mentioned in RAN1 feature list, and also to clarify corresponding parameters for unlicensed band case.

Q6: Do companies agree with the intention of the CRs [19][20]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes, but | The first change in pusch-RepetitionTypeA-r16 does not seem necessary. It is merely explaining how the feature works. Usually, we only provide a pointer to the feature itself, and do not explain the feature itself in detail. |

## 3.2 Second round: Further discussion

(TBD)

# 4 Conclusions

(TBD)

# 5 References

- R2-2207049	On early implementation and capability signaling of Power Class 1.5	MediaTek Inc.	discussion	Rel-16	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207094	Make PC1.5 an early implementation candidate	MediaTek Inc.	CR	Rel-16	38.331	16.9.0	3454	-	F	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207095	Make PC1.5 an early implementation candidate	MediaTek Inc.	CR	Rel-17	38.331	17.1.0	3455	-	A	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207085	PC1.5 and legacy power class capability reporting clarification	MediaTek Inc.	CR	Rel-16	38.306	16.9.0	0795	-	F	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207086	PC1.5 and legacy power class capability reporting clarification	MediaTek Inc.	CR	Rel-17	38.306	17.1.0	0796	-	A	LTE_NR_B41_Bn41_PC29dBm, HPUE_PC1_5_n77_n78, NR_UE_PC1_5_n79

- R2-2207113	Clarification on codebookParametersPerBC parameter for extension of CSI-RS capabilities reporting	MediaTek Inc.	CR	Rel-16	38.331	16.9.0	3452	-	F	NR_newRAT-Core, TEI16

- R2-2207114	Clarification on codebookParametersPerBC parameter for extension of CSI-RS capabilities reporting	MediaTek Inc.	CR	Rel-17	38.331	17.1.0	3453	-	A	NR_newRAT-Core, TEI16

- R2-2207331	Correction on beamManagementSSB-CSI-RS	Qualcomm Incorporated	CR	Rel-16	38.306	16.9.0	0765	-	F	TEI16

- R2-2207332	Correction on beamManagementSSB-CSI-RS	Qualcomm Incorporated	CR	Rel-17	38.306	17.1.0	0766	-	A	TEI16

- R2-2207640	CR to TS 38.306 on UE capability of MMSE-IRC receiver	China Telecom	CR	Rel-15	38.306	15.17.0	0775	-	F	NR_newRAT-Core

- R2-2207641	CR to TS 38.306 on UE capability of MMSE-IRC receiver	China Telecom	CR	Rel-16	38.306	16.9.0	0776	-	A	NR_newRAT-Core

- R2-2208027	Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306)	Ericsson	CR	Rel-17	38.306	17.1.0	0786	-	A	LTE_NR_DC_CA_enh-Core

- R2-2208028	Correction on crossCarrierA-CSI-trigDiffSCS-r16 (38.306)	Ericsson	CR	Rel-16	38.306	16.9.0	0787	-	F	LTE_NR_DC_CA_enh-Core

- R2-2206911	Reply LS on PDCCH Blind Detection in CA (R1-2205320; contact: Huawei)	RAN1	LS in	Rel-17	NR_L1enh_URLLC-Core	To:RAN2

- R2-2208501	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-16	38.306	16.9.0	0789	-	F	NR_L1enh_URLLC-Core

- [16] R2-2208502	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-17	38.306	17.1.0	0790	-	A	NR_L1enh_URLLC-Core

- [17] R2-2208503	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-16	38.331	16.9.0	3429	-	F	NR_L1enh_URLLC-Core

- [18] R2-2208504	Correction on PDCCH Blind Detection capability in CA	Huawei, HiSilicon	CR	Rel-17	38.331	17.1.0	3430	-	A	NR_L1enh_URLLC-Core

- R2-2208505	Clarification on pusch-RepetitionTypeA-r16 capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.9.0	0791	-	F	NR_L1enh_URLLC-Core

- [20] R2-2208506	Clarification on pusch-RepetitionTypeA-r16 capability	Huawei, HiSilicon	CR	Rel-17	38.306	17.1.0	0792	-	A	NR_L1enh_URLLC-Core