3GPP TSG-RAN WG2 #119-e	Tdoc R2-220XXXX

Electronic meeting, 9th - 20th May 2022

Agenda Item:	6.12.2.2

Source:	Ericsson (Rapporteur)

Title:	Report from [AT119-e][116][RedCap] Idle mode CR (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This is the report from the offline discussion below:

[AT119-e][116][RedCap] Idle mode CR (Ericsson)

Scope: Draft 38.304 CR, taking into account the relevant agreement from offline 115

Intended outcome: Agreeable 38.304 CR

Deadline (for companies' feedback): Thursday 2022-08-25 1000 UTC

Deadline (for 38.304 CR in R2-2208773): Friday 2022-08-25 1000 UTC

Companies should consider the following Tdocs and the discussions therein in mind when providing feedback to the offline discussion:

R2-2207007	Correction to description of first-PDCCH-MonitoringOccasionOfPO	Samsung Electronics Co., Ltd	draftCR	Rel-17	38.304	17.1.0	NR_redcap-Core

R2-2207207	38.304 Correction on the e-DRX for Redcap	Xiaomi Communications	draftCR	Rel-17	38.304	17.1.0	NR_redcap-Core

R2-2207622	Corrections on the intra-FreqReselection and eDRX supporting for RedCap	Huawei, HiSilicon	CR	Rel-17	38.304	17.1.0	0265	-	F	NR_redcap-Core

R2-2207750	Discussion on cellBar for RedCap	vivo, Guangdong Genius	discussion	Rel-17	NR_redcap-Core

R2-2208112	Miscellaneous correction on eDRX	ZTE Corporation, Sanechips	CR	Rel-17	38.304	17.1.0	0271	-	F	NR_redcap-Core

R2-2208221	Correction on eDRX-Allowed indication	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.304	17.1.0	0274	-	F	NR_redcap-Core

In this document, we discuss the remaining idle mode corrections based on the Tdocs provided above with the intention to formulate a list of proposals that are agreeable and a list of proposals that require further discussion during the next online session.

# Contact Information

Please fill in the following table for contact information:

| Company | Contact person - email@address.com |
| --- | --- |
| Ericsson | Emre A. Yavuz – emre.yavuz@ericsson.com |
| Futurewei | Yunsong Yang – yyang1@futurewei.com |
| Qualcomm | Linhai He – linhaihe@qti.qualcomm.com |
| Samsung | Seungbeom – s90.jeong@samsung.com |
| MediaTek | Pradeep Jose – pradeep dot jose at mediatek dot com |

# 2	Discussion on idle mode corrections

Q 2.1 Do you agree with the intention of changes in R2-2207007? Please elaborate your reply, especially if you do not and comment below if you have any suggestions for the wording if you do.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Futurewei | Yes | We are OK with the intention. |
| Qualcomm | Yes | It is probably a ‘D’ CR than a ‘F’ CR |
| Samsung | Yes (Proponent) |  |
| MediaTek | Needs clarification from proponent | The change indicates that ‘parameter first-PDCCH-MonitoringOccasionOfPO is signalled in SIB1 for paging in initialDownlinkBWP’, and for other BWPs, it’s part of the BWP configuration. This can be misread as SIB1 cannot be used to signal first-PDCCH-MonitoringOccasionOfPO in initialDownlinkBWP-RedCap. Could the proponent please clarify if this is the intention (i.e. to exclude paging configuration in RedCap-specific initial DL BWP)? |

Summary – Q 2.1

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.2 Do you agree with the intention of changes in R2-2207207? Please elaborate your reply, especially if you do not and comment below if you have any suggestions for the wording if you do.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Futurewei | No | The “if” here describes among which UE specific DRX value(s) should the shortest be determined. One should follow 38.331 regarding whether any of them is mandatory or optional for RRC_INACTIVE. Besides, the “if” applies to “RRC and/or upper layers”. So, technically the current text is still OK. We don’t see a problem here. |
| Qualcomm | No | The current text is correct, because the “if” applies to “RRC and/or upper layers”. No change is needed |
| Samsung |  | In our understanding, UE specific DRX value configured by RRC (i.e., RAN paging cycle) is mandatory to “UE in RRC_INACTIVE”. So, 1st change which applies to both RRC_IDLE and RRC_INACTIVE UEs seems not correct. Fine to either 2nd change or current text |
| MediaTek | No | There’s no possibility for misinterpretation. IE ran-PagingCycle is mandatory in the SuspendConfig, so a UE in Inactive mode will always have a value configured by RAN. Therefore, it is clear that the ‘if’ only applies to ‘upper layers’. Implementers are expected to read all specifications that affect a feature. |

Summary – Q 2.2

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.3 Do you agree with the intention of changes in R2-2207622? Please elaborate your reply, especially if you do not and comment below if you have any suggestions for the wording if you do.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Futurewei | See comments | For the first change, in 38.331, 5.2.2.5, we have: 2> else if the UE is unable to acquire the SIB1: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> if the UE is a RedCap UE: 4> peform barring as if intraFreqReselectionRedCap is set to allowed; Therefore, the RedCap UE will pass the following three ifs in 38.304 and end up executing the two mays: When cell status "barred" is indicated or to be treated as if the cell status is "barred", … - If the UE is not a RedCap UE, or if the UE is a RedCap UE and intraFreqReselectionRedCap in SIB1 is available: - If the field intraFreqReselection in MIB message is set to "allowed": - the UE may select another cell on the same frequency if re-selection criteria are fulfilled; - If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1: - the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds; We sympathize with Huawei in that one has to look at both 38.331 and 38.304 to figure the UE’s behavior. We don’t mind adding some text in 38.304, such as for the same “if” proposed by Huawei, the UE considers the cell as “barred” and intraFreqReselectionRedCap as if set to allowed. Then, the current text takes care of the rest. And we think it is reasonable to use “may”, instead of “shall”, for “exclude … for up to 300 seconds” because the UE may try to acquire the next SIB1 and actually succeed. No strong view for the second change. Can go with the majority. On the third change, we may need to wait for (or jointly consider with) P1 and P2 of e-mail discussion [115], i.e., changing the description of eDRX-AllowedIdle and eDRX-AllowedInactive. |
| Qualcomm | See comment | We have the same comment has FutureWei |
| Samsung |  | On 1st change, agree with Futurewei but have concern if we clarify 38.331 procedure in 38.304 again, it may lead to big correction. On 2nd change, agree with proposed change On 3rd change, we propose: The UE may operate in eDRX only if the UE is configured by RRC and/or upper layers and eDRX-AllowedInactive and/or eDRX-AllowedIdle is signalled in SIB1 |
| MediaTek | - | Similar views as FutureWei, i.e. 1st change: reasonable to use may 2nd change: No strong view 3rd change: Update based on discussion [115] |

Summary – Q 2.3

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4 Do you agree with the intention of changes in R2-2207750? Please elaborate your reply, especially if you do not and comment below if you have any suggestions for the wording if you do.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Futurewei | See comments | Agree with the intention. There may be more redundancy that can be removed, as follows: First, within Part 2, case 2-1 and case 3-1 are already covered by the first two ifs within case 2-3 and case 3-3 in Part 1, based on the same 38.331 text and reasoning that we have provided in Q2.3. (We consider “as if intraFreqReselectionRedCap is set to allowed;” in 38.331 means “as if intraFreqReselectionRedCap is available and set to allowed”.) Secondly, within Part 2, <When cell status "barred" is indicated for RedCap UEs with 1Rx/2Rx> is already covered by Part 1, because <When cell status "barred" is indicated> in Part 1 includes both cases of indicated by MIB barring indicator and indicated by 1Rx/2RX barring indicator. And since the 1Rx/2RX barring indicator is present, the IFRI-RedCap must be present as well. So, what remains in Part 2 is only case 2-2 and case 3-2, as follows: When the UE is a RedCap UE and the cell is to be treated as if the cell status is "barred" due to not supporting RedCap UEs, - The UE shall select another cell according to the following rule: - the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds. - the UE may select another cell on the same frequency if re-selection criteria are fulfilled. … With that, we think it is better to remove Part 2 (i.e., the second “When …” paragraph) completely and add case 2-2 and case 3-2 into Part 1 as follows: …. When cell status "barred" is indicated or to be treated as if the cell status is "barred", - The UE is not permitted to select/reselect this cell, not even for emergency calls. - The UE shall select another cell according to the following rule: - If the UE is a RedCap UE and the cell is to be treated as if the cell status is "barred" due to not supporting RedCap UEs: - the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds. - the UE may select another cell on the same frequency if re-selection criteria are fulfilled. - elseif the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB: - the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - the UE may select another cell on the same frequency if the selection criteria are fulfilled. - else: … |
| Qualcomm | Yes |  |
| Samsung | Yes | We support proposals in the contribution and also agree with Futurewei’s update. Given they have a lot of changes, so may need another offline for CR review, if RAN2 agrees to pursue these. |
| MediaTek | Yes | We can discuss the actual update in detail once the CR is available |

Summary – Q 2.4

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.5 Do you agree with the intention of changes in R2-2208112? Please elaborate your reply, especially if you do not and comment below if you have any suggestions for the wording if you do.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Futurewei | Postpone? | We may need to wait for (or jointly consider with) P1 and P2 of e-mail discussion [115], i.e., changing the description of eDRX-AllowedIdle and eDRX-AllowedInactive. |
| Qualcomm | See comment | 1st change is already addressed by discussion in #115 2nd change is not needed We are fine with the 3rd change |
| Samsung |  | Agree with Futurewei |
| MediaTek | See comments | 1st change: Update based on [115] 2nd and 3rd change: Ok to include |

Summary – Q 2.5

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.6 Do you agree with the intention of changes in R2-2208221? Please elaborate your reply, especially if you do not and comment below if you have any suggestions for the wording if you do.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Futurewei | Postpone? | We may need to wait for (or jointly consider with) P1 and P2 of e-mail discussion [115], i.e., changing the description of eDRX-AllowedIdle and eDRX-AllowedInactive. |
| Qualcomm | - | This issue is already addressed in #115 |
| Samsung |  | Agree with Futurewei |
| MediaTek | - | Address this based on [115] |

Summary – Q 2.6

???

Based on the observations above, the rapporteur proposes the following:

???

# 3	Conclusion

Based on the discussion above rapporteur suggests the following proposals:

Proposal 1	???

Proposal 2	???

Proposal 3	???

Proposal 4	???

Proposal 5	???

Proposal 6	???

# References

R2-2207007	Correction to description of first-PDCCH-MonitoringOccasionOfPO	Samsung Electronics Co., Ltd	draftCR	Rel-17	38.304	17.1.0	NR_redcap-Core

R2-2207207	38.304 Correction on the e-DRX for Redcap	Xiaomi Communications	draftCR	Rel-17	38.304	17.1.0	NR_redcap-Core

R2-2207622	Corrections on the intra-FreqReselection and eDRX supporting for RedCap	Huawei, HiSilicon	CR	Rel-17	38.304	17.1.0	0265	-	F	NR_redcap-Core

R2-2207750	Discussion on cellBar for RedCap	vivo, Guangdong Genius	discussion	Rel-17	NR_redcap-Core

R2-2208112	Miscellaneous correction on eDRX	ZTE Corporation, Sanechips	CR	Rel-17	38.304	17.1.0	0271	-	F	NR_redcap-Core

R2-2208221	Correction on eDRX-Allowed indication	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.304	17.1.0	0274	-	F	NR_redcap-Core