3GPP TSG-RAN WG2#119-e	R2-220xxxx

e-Meeting, 17th - 29th August, 2022

Title:	Reply LS on RAN feedback for low latency

Response to:	R2-2204438/ S2-2201767 LS on RAN feedback for low latency

Release:	Rel-18

Work Item:	FS_5TRS_URLLC

Source:	RAN2

To:	SA2

Cc:	RAN1, RAN3

Contact Person:

Name: Tao Cai

E-mail Address: tao.cai@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: NA

- 1. Overall Description:

RAN2 would like to thank SA2 for sharing problems and asking questions on low latency issues.

For the questions asked by SA2, RAN2 provides the answers as follows:

- What are the possible values for the periodicity of the TDD cycle that RAN can support? This question is related to Problem 1.

Answer:

RAN can support the following TDD cycles: {0.5ms, 0.625ms, 1ms, 1.25ms, 2ms, 2.5ms, 5ms, 10ms, 20ms};

Regarding Problem 1: Due to various reasons, for example, packets arriving to the gNB will experience buffering (i.e. resulting in increased delay) if their time of arrival is not aligned with the transmission opportunities of the TDD subframe, there is a need to adjust burst arrival time in TDD scenario.

- SA2 could not conclude whether a similar issue existing in FDD scenario (i.e. Problem 2) as Problem 1. Please RAN2 confirm whether it exists or not.

Answer:

There is no need to adjust burst arrival time in FDD scenario as there is no UL-DL symbols pattern issue.

- Does RAN see any additional aspects that SA2 should consider for the study?

Answer:

RAN2 sees no additional aspects that SA2 should consider for the study.

- 2. Actions:

To: SA2

ACTION: RAN2 respectfully asks SA2 to take the above information into account in the relevant work.

- 3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #119bis-e     			10 – 14 October 2022				Online

TSG-RAN WG2 Meeting #120     			14 – 18 November 2022			Europe