3GPP TSG-RAN WG2 Meeting #119-bis Electronic	R2-2209683

# Elbonia, 10 – 19 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0381 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR-DL-AoD-SignalMeasurementInformation corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Release 17 introduced RSRPP, which is not reflected as a measurement option in the definition of nr-DL-PRS-RxBeamIndex. - Release 17 introduced RSRPP, which necessitated the addition of nr-DL-PRS-FirstPathRSRP-Result-Diff-r17 in NR-DL-AoD-AdditionalMeasurementElement-r17. The fields nr-Dl-PRS-RSRP-ResultDiff-r17 and nr-Dl-PRS-FirstPathRSRP-ResultDiff-r17 are optional, conditional on the presence of the other. Because the release 16 specification does not define RSRPP, the conditional presence explanations are vague with regard to NR-DL-AoD-AdditionalMeasurementElement-r16, where nr-DL-PRS-RSRP-ResultDiff-r16 is mandatory. - nr-DL-PRS-RxBeamIndex indication is used for DL-PRS measurements only when additional DL-PRS measurements are also included and all these DL-PRS measurements are associated with a single TRP (up to 8 measurements in Rel-16 or 24 measurements in Rel-17). - According to 38.214 v17.3.0, section 5.1.6.5 “The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on DL PRS resources associated with the same dl-PRS-ID”. <…>. The UE may be configured to measure and optionally report, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on DL PRS resources associated with the same dl-PRS-ID”. This means both the nr-DL-AoD-AdditionalMeasurements-r16 and nr-DL-AoD-AdditionalMeasurementsExt-r17 fields cannot be included by the UE at the same time because the total reported measurements would exceed 24 in Rel-17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 6.5.11.4, NR-DL-AoD-SignalMeasurementInformation field descriptions, add DL-PRS RSRPP measurements to the types of measurements that, exceeding 2, require the nr-DL-PRS-RxBeamIndex field to be present. - In 6.5.11.4, NR-DL-AoD-SignalMeasurementInformation, conditional presence explanations, the fields nr-DL-PRS-FirstPathRSRP-ResultDiff and nr-DL-PRS-RSRP-Result diff should be appended with -r17 to increase clarity of the conditions. - In 6.5.11.4, NR-DL-AoD-SignalMeasurementInformation, clarify the field description for nr-DL-PRS-RxBeamIndex that it is used for DL-PRS measurements only when additional DL-PRS measurements are also included and all DL-PRS measurements are associated with a single TRP. - In 6.5.11.4, NR-DL-AoD-SignalMeasurementInformation, add field descriptions for nr-DL-AoD-AdditionalMeasurements-r16 and clarify that only one of and nr-DL-AoD-AdditionalMeasurements-r16 or nr-DL-AoD-AdditionalMeasurementsExt-r17 field can be included by the UE in the measurement report. - Misc. editorial corrections Impact analysis Impacted functionality: NR DL-AOD Measurement Reporting Inter-operability: - If the network is implemented according to the CR and the UE is not, an error might occur when the UE neglects to send the nr-DL-PRS-RxBeamIndex-r17 when sending two or more RSRPP measurements. - If the UE is implemented according to the CR and the network is not, an error might occur if the UE sends an unsolicited nr-DL-PRS-RxBeamIndex-r17 when sending two or more RSRPP measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Specification is ambiguous about the usage of nr-DL-PRS-RxBeamIndex and nr-DL-AoD-AdditionalMeasurements-r16 fields. - Without clarification to the conditional presence definitions, the definitons are in conflict with the -r16 message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.5.11.4	NR DL-AoD Location Information Elements

#### –	NR-DL-AoD-SignalMeasurementInformation

The IE NR-DL-AoD-SignalMeasurementInformation is used by the target device to provide NR DL-AoD measurements to the location server.

-- ASN1START

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15								OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15						OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 			OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 			OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126),

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

nr-DL-AoD-AdditionalMeasurements-r16

NR-DL-AoD-AdditionalMeasurements-r16	OPTIONAL,

...,

[[

nr-DL-PRS-FirstPathRSRP-Result-r17

INTEGER (0..126)						OPTIONAL,

nr-los-nlos-Indicator-r17		CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																		OPTIONAL,

nr-DL-AoD-AdditionalMeasurementsExt-r17

NR-DL-AoD-AdditionalMeasurementsExt-r17	OPTIONAL

]]

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF

NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasAoD-r17)) OF

NR-DL-AoD-AdditionalMeasurementElement-r17

NR-DL-AoD-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 			OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 			OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..30),

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

...

}

NR-DL-AoD-AdditionalMeasurementElement-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r17				NR-DL-PRS-ResourceID-r16		OPTIONAL,

nr-DL-PRS-ResourceSetID-r17				NR-DL-PRS-ResourceSetID-r16		OPTIONAL,

nr-TimeStamp-r17						NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r17			INTEGER (0..30)					OPTIONAL, -- Cond rsrp

nr-DL-PRS-RxBeamIndex-r17				INTEGER (1..8)					OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL, -- Cond rsrpp

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| rsrp | The field is mandatory present if the field nr-DL-PRS-FirstPathRSRP-ResultDiff-r17 is absent; otherwise it is optionally present, need ON. |
| rsrpp | The field is mandatory present if the field nr-DL-PRS-RSRP-ResultDiff-r17 is absent; otherwise it is optionally present, need ON. |

NR-DL-AoD-SignalMeasurementInformation field descriptions

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resource ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-TimeStamp This field specifies the time instance at which the measurement is performed.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL-PRS RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements associated with a single TRP in nr-DL-AoD-MeasList-r16 when additional DL-PRS measurements are also included in either nr-DL-AoD-AdditionalMeasurements-r16 or nr-DL-AoD-AdditionalMeasurementsExt-r17. If the value of the receive beam index for two or more DL-PRS measurements is the same, it indicates that the target device receive beam for the two or more DL-PRS measurements associated with a TRP were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements and/or DL-PRS RSRPP measurements from the same DL-PRS Resource Set associated with a TRP have been made with the same RX beam by the target device; otherwise it is not present.

nr-DL-AoD-AdditionalMeasurements This field specifies a list of additional DL-PRS RSRP measurements of different DL-PRS resources for the same TRP. If this field is present, the field nr-DL-AoD-AdditionalMeasurementsExt should not be present.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL-PRS reference signal received path power (DL-PRS RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the RSRP or RSRPP of first path measurement for the TRP or resource. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-DL-AoD-AdditionalMeasurementsExt This field specifies a list of additional DL-PRS RSRP and/or DL-PRS RSRPP measurements of different DL-PRS resources for the same TRP. If this field is present, the field nr-DL-AoD-AdditionalMeasurements should not be present.

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL-PRS reference signal received path power (DL-PRS RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the RSRP or RSRPP of first path measurement for the resource. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.