3GPP TSG-RAN WG2 #119bis-e	R2-221xxxx

Online, 10th – 19th Oct, 2022

Agenda Item:	8.1.2

Source:	ZTE Corporation

Title:	Report of [AT119bis-e][NCR] NCR open issues (ZTE)

Document for:	Discussion, Decision

Introduction

This document is the report of the following offline discussion:

- Side control information signaling options (i.e. RRC vs. OAM)

- RRC states of NCR-MT

- Support of SRBs/DRBs

- NCR-Fwd ON/OFF

- SI impacts

- RRM functions

- [AT119bis-e][701][NCR] NCR open issues (ZTE)

Scope: see above.

Intended outcome: Report.

Deadline: TBD

Contact Information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Georg Hampel | ghampel@qti.qualcomm.com |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| CATT | HaoXu | xuhao@catt.cn |
| Huawei, HiSilicon | Xubin | xubin10@huawei.com |

Discussion

This document is to collect company views on the NCR open issues

Side control information signalling options

According to the TR, there are 3 options for the NCR-MT to obtain the necessary configuration for receiving the L1/L2 signaling of the side control information.

-	Option 1: The necessary configuration is from RRC.

-	Option 2: The necessary configuration is from OAM or hard-coded.

-	Option 3: The necessary configuration is partially configured by RRC and partially configured by OAM or hard-coded.

Q1: Which option do companies prefer to configure NCR-MT for receiving L1/L2 signalling of side control information?

| Company | Option 1 (RRC); Option 2 (OAM); Option 3(RRC+OAM) | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | L1/2 signaling (DCI, MAC CE) is always RRC-configured. OAM configuration is out of scope. OAM should not be used for L1/L2 signaling configuration. |
| ZTE | Option 1 |  |
| CATT | Exclude Option2 | Option 1 can be used as baseline, and option3 can be FFS, which we think it is better to wait for more information from other groups. |
| Huawei, HiSilicon | Option 1 | According to the TR 38867, the “necessary configuration for receiving the L1/L2 signaling of the side control information” includes the following two aspects: - The configurations of PHY channels to carry the L1/L2 signaling - The configurations of L1/L2 signaling In our understanding, such configurations need to be changed dynamically according to the network condition, and thus should be configured by RRC naturally. |

RRC states of NCR-MT

Companies are invited to show your views on which RRC state(s) can be supported by NCR-MT.

- RRC_CONNECTED;

- RRC_IDLE;

- RRC_INACTIVE

Q2: Which RRC state(s) can be supported by NCR-MT?

| Company | applicable RRC states (answer Yes/No/FFS) |  |  | Comments |
| --- | --- | --- | --- | --- |
|  | RRC_ CONNECTED | RRC_ IDLE | RRC_ INACTIVE |  |
| Qualcomm | Yes | Yes | Yes | Same as IAB-MT. |
| ZTE | Yes | Yes | FFS | We understand RRC_INACTIVE state is not so important for NCR-MT because NCR-MT will not change its RRC state frequently and SDT seems not needed. Currently, RRC_INACTIVE is defined as mandatory feature with capability signalling， if most companies want to support RRC_INACTIVE state, we suggest to change it into optional feature for NCR-MT. |
| CATT | Yes | Yes | No | In our understanding, the main principle of introducing RRC_INACTIVE state is that the UE is able to return to the connected state quickly. Considering the C-link doesn’t need this requirement, we prefer not to support RRC inactive state in the current release. |
| Huawei, HiSilicon | Yes | Yes, but | FFS | Clarification is needed when NCR-MT should be in IDLE, e.g. whether it should be released to IDLE by the gNB deliberately and if so, in which case. For RRC_INACTIVE, more justification is needed. |

Support of SRBs/DRBs

Whether NCR-MT supports SRBs (i.e. SRB0/1/2) and DRB?

To facilitate the discussion, rapporteur has provided some information from other WG:

#RAN3 online agreement made on Thursday 10-13: The NCR-OAM connectivity requirement should be supported, further details can be discussed. Nok: This OAM requirement has no impact in RAN3

So RAN3 just agreed to support NCR-OAM connectivity requirement, as proposed by some companies, one possible way for transmitting OAM traffic from NCR-MT to gNB (or vice versa) is to establish a PDU-session over a DRB, similar to IAB.

Q3: Whether SRBs and DRB are supported by NCR-MT?

| Company | Support of SRB/DRB (answer Yes/No/FFS) |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | SRB0 | SRB1 | SRB2 | DRB |  |
| Qualcomm | Yes | Yes | Yes | Yes | SRB0 and SRB1 are mandatory for RRC setup SRB2 is mandatory for NAS. DRB should be optional. It may be used to enable OAM connectivity via PDU session. Note: The OAM connectivity REQUIREMENT implies that 3GPP has to support a mechanism for OAM connectivity. It does not imply that implementations have to use this mechanism. |
| ZTE | Yes | Yes | Yes | Yes | For DRB, we understand it is still a mandatory feature for NCR-MT, otherwise, we will introduce a new connection type which was not supported by legacy UEs (i.e. SRB0/1/2 but without DRB), and this may cause more specification impacts. |
| CATT | Yes | Yes | Yes | Yes | Thanks rapporteur providing the information about NCR-OAM connectivity requirement, hence we think it is reasonable to support DRB as well right now. |
| Huawei, HiSilicon | Yes | Yes | Yes | FFS | The NCR-OAM connectivity doesn’t necessarily reuqire the support of DRB. RAN2 may need to discuss more about what should be transmitted via the NCR-OAM connectivity. |

NCR-Fwd ON/OFF

Several companies propose to discuss the linkage between NCR-Fwd ON/OFF and the RRC state of NCR-MT, rapporteur has summarized them into following options:

- Option 1: When NCR-Fwd is ON, NCR-MT can be in any RRC states (e.g. RRC_CONNECTED or RRC_IDLE/INACTIVE);

- Option 2: When NCR-Fwd is ON, NCR-MT must be in RRC_CONNECTED state; when NCR-MT is in RRC_IDLE/INACTIVE states, NCR-Fwd must be “OFF”;

- Option 3: When NCR-Fwd is ON, NCR-MT must be in RRC_CONNECTED state; when NCR-MT is in RRC_IDLE state, NCR-Fwd must be “OFF”; when NCR-MT is in RRC_INACTIVE state, NCR-Fwd can be “ON” or “OFF”;

- Option 4: Up to RAN1, considering RAN1 is discussing the fallback mechanism for NCR.

Q4: Which option is preferred for the linkage between NCR-Fwd ON/OFF and the RRC state of NCR-MT?

| Company | Preferred Option | Comments |
| --- | --- | --- |
| Qualcomm | Option 2 | ON/OFF is part of side control and therefore signaled on slot-level time scale. The NCR-MT must be operational when receiving side control signaling. As the baseline, the NCR-MT is operational when RRC-CONNECTED. The NCR-MT is certainly not operational and cannot receive side control in RRC-IDLE. The question arises, if the NCR-MT could be operational in RRC-INACTIVE: - What are the benefits? - How would it receive MAC-CE’s in this state? - How would it perform beam control, power control, etc on the BH link? To keep things simple during the first NCR WI, we should assume that the NCR-MT can only receive side control including ON/OFF info when RRC CONNECTED. |
| ZTE | Option 4 | RAN1 is discussing the fallback mechanism for NCR, e.g. when the NCR-MT is in RRC_IDLE/INACTIVE state, the NCR-Fwd can still “ON”, but it operates like a traditional RF-repeater (no side control information). See RAN1 agreements made last meeting: The NCR-Fwd is always expected to be “OFF” unless otherwise explicitly or implicitly indicated by gNB. - Note-1: This applies to the case regardless of the RRC state of NCR-MT. - Note-2: Indication (e.g., received when NCR-MT in RRC-connected) or DRX state of NCR-MT to control the ON-OFF behaviour of NCR-Fwd when the NCR-MT is in RRC-idle/inactive is not precluded. The above is not meant to imply any signalling design for NCR-Fwd ON-OFF.. From RAN2 perspective, we suggest to wait for RAN1 and then decide if there is any RAN2 impact. |
| CATT | Option 4 | Same view as ZTE. |
| Huawei, HiSilicon | Option 4 | To avoid duplicated discussion and contradictory solutions in RAN1 and RAN2, Better to wait for RAN1 first. |

SI impacts

For legacy SIB configuration, companies are invited to check the following proposals:

- Proposal 1: NCR-MT should ignore cellBarred indication;

- Proposal 2: NCR-MT should ignore Unified Access Control (UAC) configuration;

- Proposal 3: NCR-MT should ignore cellReservedForOperatorUse, cellReservedForFutureUse，cellReservedForOtherUse and intraFreqReselection indications.

Q5: Which proposal(s) do you support?

| Company | Supported proposals (P1, P2, P3) | Comments |
| --- | --- | --- |
| Qualcomm | P1, P2, P3 | Same as IAB |
| ZTE | P1, P2, P3 | Same as IAB |
| CATT | P1~P3 | Same as IAB |
| Huawei, HiSilicon | P1, P2, P3 | Same as IAB |

RRM functions

For the following RRC functions, which one(s) should be or can be supported by NCR-MT?

- C1: RRM measurements in RRC_IDLE/INACTIVE;

- C2: RRM measurements in RRC_CONNECTED;

- C3: Cell (re)selection;

- C4: Handover;

- C5: RLM;

- C6: BFD, BFR

Note: based on the progress in RAN4, most companies suggest to wait for RAN2 input.

Q5: Which RRM functions should be or can be supported by NCR-MT?

| Company | Supported RRM functions (answer Yes/No/FFS) |  |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | C3 | C4 | C5 | C6 |  |
| Qualcomm | Optional | Optional | Mandatoary | Optional | Mandatory | Mandatory | C3 is necessary for NCR-MT to connect to the network. C5 is necessary for NCR-MT to determine RLF, i.e., if it is operational or not. C6 is necessary for the NCR-MT to keep the BH link stable C1/C2/C4: This is not necessary for NCR operation. It can be optional and left up to implementation. |
| ZTE | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory | Optional | C3 must be supported, so NCR-MT can select new cell when change its location; C1 is the basis for supporting C3; C4 is needed when operator/network wants to dynamically switch the NCR-MT to serve another overlapping cell or carrier; C2 is the basis for supporting C4; C5 must be supported, so NCR-MT can detect the radio link problem and reconnect if necessary; C6 can be optional for NCR-MT if RLF is supported. |
| CATT | No | No | No | No | No | No | We think that NCR is mainly deployed by operator. And according to the WID, NCR is only single hop stationary network-controlled repeaters. Hence, cell (re)selection, handover RRM measurement may be unnecessary. Considering the cost of supporting more useless function, we prefer not need to support RRM measurement, cell (re-)selection and handover. For the other functions, we fail to see the necessity of mandatory requirement in the current release. |
| Huawei, HiSilicon | FFS | FFS | Yes for cell selection | No | FFS | FFS | - The cell selection in C3 should be supported for the NCR-MT to access a cell. - The cell re-selection in C3 should not be supported as the NCR doesn’t move. - C4 should not be supported as the NCR doesn’t move. - C1 should not be supported. According to 38.133, there is no requirement of RRM measurement on NCR-MT to perform cell selection. - C2 should not be supported as the NCR doesn’t move. - C5/C6 should be FFS in RAN2 and wait for RAN1 as at least BFD/RLM is up to RAN1. If BFD/RLM is supported by RAN1, RAN2 can further discuss BFR. |

Conclusion

Based on companies’ input, proposals are listed as follows.

TBD