3GPP TSG-RAN WG2 Meeting #120	   R2-2211249

Toulouse, France, November 14th – 18th, 2022

Agenda item:		8.18.2

Source:	vivo

Title:	Supporting Mobile Terminated Small Data Transmission in RRC_INACTIVE

Document for:		Discussion and Decision

1 Introduction

In RAN#94 Plenary, the new WID on Mobile Terminated-Small Data Transmission (MT-SDT) for NR had been approved. The main working objectives for RAN2 in [1] are listed below,

The objectives of the WI are to specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3] - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response; - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE. Note: Data transmission in DL within paging message is not in scope of this WI.

In this contribution, we would like to provide our considerations on the MT-SDT procedure in terms of:

- MT-SDT triggering;

- Resource selection;

- DL data reception;

- Fallback.

2 Discussion

For the legacy RRC INACTIVE UE, when there is an arrival MT data for this UE (i.e. the MT data arrives at the last serving anchor), RAN paging amongst one or more than one cells within an RNA area will be initiated. Upon receiving a matched paging message, the UE will initiate RRC resumption procedure and transit to RRC CONNECTED for MT data reception, as shown in the following figure,

Figure 1: Paging triggered RRC resumption procedure for DL data reception

In our understanding, by leveraging on the MT-SDT procedure, less interaction between paging response and DL data reception should be achieved with a guarantee of transmission security, flexibility, efficiency, and reliability. In this sense, we assume RAN2 should at least consider the following four issues,

- What are the conditions of MT-SDT triggering;

- How to select a resource amongst multiple resources for UL response of MT-SDT;

- Which message is used to carry the first DL data;

- Whether to allow a fallback mechanism from MT-SDT to MO-SDT.

Proposal 1: RAN2 should discuss MT-SDT procedure considering at least the following issues,

- What are the conditions of MT-SDT triggering;

- How to select a resource amongst multiple resources for UL response of MT-SDT;

- Which message is used to carry the first DL data;

- Whether to allow a fallback mechanism from MT-SDT to MO-SDT.

  - 2.1	MT-SDT Triggering

In the legacy NR framework, paging message is used to inform the arrival of MT data. For MT-SDT, we think this logic is still applicable. Specifically, the existing paging framework can be taken as a baseline for MT-SDT triggering (potentially with a new triggering indicator). If a matched UE ID and a corresponding triggering indicator are received by the UE, then it can further evaluate the other conditions to see whether to trigger MT-SDT procedure.

Proposal 2: Existing Paging message is used to trigger MT-SDT procedure.

Proposal 3: No new paging search space nor new P-RNTI are defined for MT-SDT procedure.

After the reception of a matched paging message triggering MT-SDT procedure, additional conditions need to be checked for the applicable of MT-SDT procedure. For example, there should be at least one SDT-RB is configured and enabled for MT-SDT procedure, and the downlink pathloss reference should be higher than a configured threshold considering TBS of DL data might be much larger than that of Msg4 message.

Proposal 4: UE can initiate MT-SDT procedure if the following conditions are fulfilled,

- There is at least one RB configured as SDT-RB;

- Downlink pathloss reference is higher than a configured threshold.

  - 2.2	Resource selection

Upon the initialization of MT-SDT procedure, the INACTIVE needs to select a UL resource to transmit a response to the paging message. Based on the WID, it is stated that RA-SDT and CG-SDT are supported for the transmission of this kind of response. Thus, it seems a spontaneous logic that only the initial BWP can be used for MT-SDT procedure with both DL and UL transmission.

Proposal 5: Only the initial BWP (including the separate initial BWP for RedCap) is used for MT-SDT procedure.

Considering that there is usually no pending UL data for transmission upon the initialization of MT-SDT procedure, we think the UL response can only include the legacy UL-CCCH message. Since the data size of the UL response is not larger than the Msg3 size for the initial access procedure. We think the legacy RACH resource (e.g. 4-step RA and 2-step RA) can be used for the MT-SDT procedure. More specifically, after identifying the UE via the UL-CCCH message, the NW can deliver the DL data by Msg4/MsgB or during the subsequent transmission phase. The whole procedure works well.

Proposal 6: Legacy RA resources can be used for MT-SDT procedure.

Additionally, to further improve transmission efficiency and reliability of the UL response, specific CG resources can be configured for MT-SDT procedure. When both MT-SDT specific CG resources and CG-SDT are configured for a UE, then it will always choose the MT-SDT specific CG resource during the MT-SDT procedure, if applicable. Otherwise, it uses legacy RA resources.

Proposal 7: Separate CG resources specific to MT-SDT procedure can be configured.

Proposal 8: If separate CG resources are applicable, then those CG resources are used. Otherwise,  legacy RA resources are chosen.

  - 2.3	DL data reception

Technically, if the RA procedure is used for MT-SDT procedure, the first DL data delivery can be done via,

- scheduling by paging message,

- Msg2,

- Msg4/MsgB,

- the other DL scheduling after Msg4/MsgB.

Obviously, including a UE ID and the corresponding scheduling information for DL data delivery into the paging message will largely increase the paging overhead and may incur additional complexity on the CU operation. Thus, it should be excluded from the MT-SDT scope.

If multiplexing the DL data into the Msg2 message, more attention is required to investigate the security issue, the new MAC PDU structure design. For simplicity, we suggest ruling out this solution.

For the other two solutions, it is already supported in RA-SDT. They can be reused in MT-SDT procedure.

Having said the evaluation above, similar to the RA-SDT, we think the first DL data delivery shall not be performed before the transmission of Msg4/MsgB.

Proposal 9: For RA-based MT-SDT procedure,  the first DL data delivery shall not be performed before the transmission of Msg4/MsgB.

2.4 Fallback

After the initialization of MT-SDT procedure but before the transmission of the UL response message, it is likely that new UL data mapping to SDT-RB arrives and all the conditions for CG-SDT/RA-SDT procedure can be fulfilled. In this case, by UE implementation, we think the UE should use Rel-17 SDT instead, and anyway the DL data can be delivered after the UL transmission without any cost. So we propose the fallback mechanism as shown in Figure 2.

Figure 2: Fallback from MT-SDT to MO-SDT

Proposal 10: Fallback from MT-SDT to MO-SDT is allowed.

3 Conclusion

In this contribution, we have provided our initial considerations on the small data transmission procedure in terms of MT-SDT triggering, resource selection, DL data reception, and fallback. All our proposals are summarized as follows,

General:

Proposal 1: RAN2 should discuss MT-SDT procedure considering at least the following issues,

- What are the conditions of MT-SDT triggering;

- How to select a resource amongst multiple resources for UL response of MT-SDT;

- Which message is used to carry the first DL data;

- Whether to allow a fallback mechanism from MT-SDT to MO-SDT.

MT-SDT triggering:

Proposal 2: Existing Paging message is used to trigger MT-SDT procedure.

Proposal 3: No new paging search space nor new P-RNTI are defined for MT-SDT procedure.

Proposal 4: UE can initiate MT-SDT procedure if the following conditions are fulfilled,

- There is at least one RB configured as SDT-RB;

- Downlink pathloss reference is higher than a configured threshold.

Resource selection:

Proposal 5: Only the initial BWP (including the separate initial BWP for RedCap) is used for MT-SDT procedure.

Proposal 6: Legacy RA resources can be used for MT-SDT procedure.

Proposal 7: Separate CG resources specific to MT-SDT procedure can be configured.

Proposal 8: If separate CG resources are applicable, then those CG resources are used. Otherwise,  legacy RA resources are chosen.

DL data reception:

Proposal 9: For RA-based MT-SDT procedure,  the first DL data delivery shall not be performed before the transmission of Msg4/MsgB.

Fallback:

Proposal 10: Fallback from MT-SDT to MO-SDT is allowed.

4 References

[1] RP-213583, New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR, RAN#94e