3GPP TSG-RAN WG2 Meeting #120		R2- 2211321

Toulouse, France, 14th Nov. – 18th Nov., 2022

Agenda Item:	8.15.2

Source:	vivo

Title:	Further discussion on SL CAPC

Document for:	Discussion and Decision

Introduction

According to RAN2#119bis meeting, the following agreements were made[1]:

Agreement on CAPC: 1: Working assumption: PQI is used to determine the CAPC mapping as in NR-U. FFS whether the same principle is also applied to the UE side. 2: For SL-DRB the CAPC value is (pre)configurable per-DRB as in NR-U. 3: For all SL-SRBs, CAPC value is fixed to the highest priority (i.e., lowest CAPC value). 4: If PQI-based CAPC mapping is agreed, for all SL MAC CEs, CAPC value is fixed to the highest priority (i.e., lowest CAPC value). 5: If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed. 6: As in NR-U, if SL CAPC is determined based on PQI, as a baseline, for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. FFS if any specific work needed for RRC_INACTIVE/RRC_IDLE/OOC UEs. 7: If PQI-based CAPC mapping is agreed, as in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI: - If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used; FFS whether this rule can be extended to the case when SL MAC CE(s) multiplexed with STCH. - If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; - FFS how to select SL CAPC when SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

In this contribution, we discuss about the PQI to CPAC mapping, how to determine CAPC for non-standardized PQI, and how to determine the CAPC when SBCCH SDU is included in the SL TB.

Discussion

WA on using PQI to determine CAPC mapping

This section treats the below WA achieved in the last meeting:

Agreement on CAPC: 1: Working assumption: PQI is used to determine the CAPC mapping as in NR-U. FFS whether the same principle is also applied to the UE side.

In the last meeting, the counterpart solution is to map CAPC values with the L1 priorities (i.e. the priority included in SCI). In NR-U, the mapping between CAPC and 5QI actually functions as the guideline to regulate/guide gNB configuration of CAPC value for each LCH to serve associated services, taking into account also other radio access technologies co-existing in the unlicensed band (e.g. WiFi). As the 5QI can well reflect the performance requirements and characteristics of the services to be served by NR-U on the unlicensed band, such a guideline  is useful to ensure that the corresponding service(s) associated with each 5QI can be fairly treated and at the same time limit the impact to the precedent radio access technologies already operating on the unlicensed band within an acceptable level.

However, the above purpose might not be able to be realized, if one would intend to map CAPC to the L1 priority. In NR SL, the L1 priority is in nature the SL LCH priority (i.e. highest SL LCH priority included in the MAC PDU), and is no more the PPPP value as in LTE SL. Whilst the service to PPPP mapping (with also PDB able to be derived) was defined in some regional standards by SDOs other than 3GPP, making PPPP able to be regarded as a metric reflecting QoS requirements of the services on LTE PC5, the SL LCH priority in NR SL is completely (pre)configured by the gNB/NW and it is thus completely up to NW implementation on how to set its value. Therefore, if one intends to rely on CAPC to L1 priority mapping, the setting of the CAPC value for the services(s) mapped to each SL LCH is also fully up to NW implementation w/o any restriction/guideline, since both the service to SL LCH mapping and SL LCH priority setting are completely up to NW configuration. As an example, in order to pursue relatively high CAPC priorities, the gNB via implementation can map all the PC5 QoS flows of a UE into the SL LCH(s) with rather high SL LCH priority(ies) configured, which further correspond to high CAPC priorities due to the CAPC to L1 priority mapping, even though some of the PC5 QoS flows are not with high QoS requirements and do not need the channel access parameters with that high CAPC priorities. This is of course unfriendly to the transmission of other radio access technologies, and seems to imply that the configuration of gNB is arbitrary w/o any limitation for 3GPP designed SL-U.

In a word, we need a CAPC mapping rule to regulate the gNB-/pre-configuration on the CAPC values applicable to each service to be transmitted, but CAPC to L1 priority mapping cannot achieve this purpose as shown above, since the L1 priority itself is in nature the SL LCH priority whose setting is still up to gNB/NW implementation.

- Confirm the WA that PQI is used to determine the CAPC mapping as in NR-U (i.e. CAPC to PQI mapping table is introduced).

FFS on how to derive CAPC to PQI mapping table

This section treats the below FFS point left from the last meeting:

5: If PQI-based CAPC mapping is agreed, at least PDB can be used as the criterion to determine the CAPC mapping. FFS if any other additional criterions needed.

It was agreed in the last meeting to take PDB into account when producing the specific CAPC to PQI mapping table. Specifically, some companies proposed to partition the PQIs into different levels by their associated PDB values, and for each level a specific CAPC value is associated.

We think this is a feasible way for producing this mapping table. If the majority of companies would like to go with this direction, RAN2 can tentatively conclude the specific mapping table based on this criterion. However, as what we proposed in our earlier contributions, the CAPC value is associated with the L1 channel access parameters to be used in SL-U, and it should be ensured that the PQI(s) mapped to a given CAPC value can really be satisfied with the channel access parameters this CAPC value allows to use. Since RAN1 already concluded to apply the below channel access parameter table for SL-U [2], this RAN1 agreement should be taken into account together with the PDB metric when producing the CAPC to PQI mapping.

Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms..

Also, to avoid mismatch between the channel access parameters and the PQI(s) mapped to the same CAPC, it is proposed to send LS to RAN1 informing them of the RAN2 decision for RAN1 to check, once RAN2 concludes the specific CACP to PQI mapping table.

- Consider L1 channel access parameters for each CAPC concluded by RAN1 together with PDB, when specifying the CAPC to PQI mapping table.

- Send LS to RAN1, informing them of the specific CAPC to PQI mapping table, once concluded by RAN2.

CAPC for non-standardized PQI

This section treats the following FFS point for the CAPC determination for non-standardized PQI as follows:

6: As in NR-U, if SL CAPC is determined based on PQI, as a baseline, for non-standardized PQI, to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. FFS if any specific work needed for RRC_INACTIVE/RRC_IDLE/OOC UEs.

During last meeting, it was agreed that for non-standardized PQI, the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI is used, but we have not concluded whether such principle is applicable for both gNB and UE.

When a mode 1 or mode 2 SL UE is in RRC_CONNECTED state, it can report QoS profiles to the gNB. With this, the gNB can configure suitable CAPC for non-standardized PQI, and the method of NR-U can be reused, i.e. when configuring CAPC of non-standardized PQI, use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. The UE can simply follow the NW configuration.

- For a CONNECTED UE, it is up to gNB implementation to configure the CAPC of non-standardized PQI using the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI.

For the SL DRB configurations provided by SIB/pre-configuration, it is impractical to assume that all the possible sidelink non-standardized QoS parameters must be covered by the SL-DRB configurations provided by the SIB or pre-configuration. If an IDLE/INACTIVE/OOC UE is to perform a transmission for the PC5 QoS flow corresponding to a non-standardized PQI that cannot be mapped to any SL DRB configuration, the UE cannot determine the CAPC value for this QoS flow. As a SL specific case, how this case should be treated may need further RAN2 discussion.

For an IDLE/INACTIVE/OOC UE, upon transmitting QoS flow with non-standardized PQI, it can first check the SIB/pre-configuration. If the UE can find the non-standardized PQI mapped to a SLRB configuration from SIB/pre-configuration which contains a non-standardized PQI whose QoS characteristics are all the same with such non-standardized PQI, the UE can use the CAPC configured in this SLRB configuration for the non-standardized PQI. After all, it is up to gNB implementation to configure CPAC for some SLRBs which mapped to QoS flow of non-standardized QoS parameters in SIB/pre-configuration.

- For non-standardized PQI, it is up to gNB implementation whether to configure CPAC in associated SLRB configuration which contains the mapping of DRB to QoS flow of non-standardized PQI.

- For an IDLE/INACTIVE/OOC UE, if the QoS flow of non-standardized PQI can be mapped to a non-default SLRB, the UE determines the CAPC of this non-standardized PQI using the CAPC of this SLRB.

However, if the IDLE/INACTIVE/OOC UE cannot find the non-standardized PQI mapped to a SLRB configuration from SIB/pre-configuration which contains a non-standardized PQI whose QoS characteristics are all the same with such non-standardized PQI, the UE need to determine the CAPC for the non-standardized PQI.

During last meeting, some companies proposed to fully leave it to UE implementation to use the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. However, this method may lead to the following concerns:

- This is out of network control.

- UE-gNB inconsistence: The UE and the gNB may have different implementation of determining CAPC for a same non-standardized PQI. For instance, it is possible that the gNB configures CAPC value p1 to it for an IC UE, while an OOC UE determines the CAPC to be p2. From one perspective, if p1<p2, when the IC UE initiate a COT and share to the OOC UE, since the COT is initiated using CAPC = p1, the OOC UE may determine that it cannot used this shared COT according to regulation. This may cause traffic degradation. From the other perspective, if the IC UE and the OOC UE use different CAPC for the same non-standardized PQI, it may lead to traffic delay when the traffic density in the environment is high, e.g. lots of Wi-Fi devices around.

- UE-UE inconsistence: The UE and the peer UE may also have the inconsistence concern.

- It is out of network control, if one intends to fully leave it to UE implementation to decide the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. Meanwhile, there is UE-gNB/UE-UE implementation inconsistence concern.

To address the concerns, the following options can be considered for determining CPAC for current non-standardized PQI:

- option A: use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics, e.g. by comparing PDB, PriorityLevel or other QoS characteristics

- option B: use default CAPC value, e.g. the CAPC of default SLRB if it is confirmed in proposal 6 that gNB can configure CAPC for default SLRB, or a default CAPC which is pre-defined or configured by SIB/pre-configuration for this case.

Option A is to align UEs on how to determine the CAPC for non-standardized PQI instead of leaving it to UE implementation. Since CAPC-PQI mapping table may be introduced and the mapping relation may be based on PDB, the UE can first find the standardized PQI from Tables in the Annex or non-standardized PQI configured in SIB/pre-configuration whose PDB is the most closed. If there is more than one PQIs having the same PDB, the UE can further find among them whose other QoS characteristics (e.g. PriorityLevel) are the most closed to that of the non-standardized PQI.

Option B is controlled by the network. It is proposed basing on the fact that the gNB can configure UE to transmit QoS flow data with non-standardized PQI using the default SLRB. Therefore, if the gNB can configure a CAPC value for default SLRB, the UE can use the CAPC of the default SLRB for the non-standardized PQI. Another solution for option B is pre-defining or the gNB configuring a default CAPC value for this case.

- RAN2 to discuss whether gNB can configure a CPAC value for default SLRB.

- For an IDLE/INACTIVE/OOC UE, if the QoS flow of non-standardized PQI cannot be mapped to a non-default SLRB , the UE determines the CAPC of current non-standardized PQI by down-selecting from one of the following options:

- option A: use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics, e.g. by comparing PDB, PriorityLevel or other QoS characteristics;

- option B: use default CAPC value, e.g. the CAPC of default SLRB if it is confirmed in proposal 6 that gNB configures CAPC for default SLRB, or a default CAPC which is pre-defined or configured by SIB/pre-configuration for this case;

Per-TB CAPC determination

This section treats the below FFS points regarding how to determine the CAPC value to be used for a specific TB transmission:

7: If PQI-based CAPC mapping is agreed, as in NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI: - If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used; FFS whether this rule can be extended to the case when SL MAC CE(s) multiplexed with STCH. - If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used; - FFS how to select SL CAPC when SL CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

Frankly speaking, we do not see how/why SL-U deviates much from NR-U in this multiplexing aspect and why  an optimization is needed specific to the SL transmission. Therefore, we tend to directly remove the FFS left towards this multiplexing aspect between MAC CE and traffic data. Also, as some companies proposed in the last meeting, SBCCH is something that is specific to SL and needs to be inserted in the above CAPC determination rule. From our perspective, transmission of SBCCH should be taken as the SL transmission with the highest priority, a bit similar to how we treat CCCH transmission in NR-U. That means, SBCCH can enjoy the similar treatment as SCCH.

As a summary, the following CAPC determination for per-TB transmission when CAPC is not indicated in the DCI is proposed as follows:

- RAN2 agree the following rule to determine the CAPC of an SL TB, when the CAPC value is not indicated in the DCI for mode-1 and when the UE is operating in mode-2:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used;

- If SBCCH SDU is included in the SL TB, the highest priority SL CAPC is used;

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- The lowest priority CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

Conclusion

Based on the discussion, we have the following observation and proposals:

- For non-standardized PQI, it is up to gNB implementation whether to configure CPAC in associated SLRB configuration which contains the mapping of DRB to QoS flow of non-standardized PQI.

- It is out of network control, if one intends to fully leave it to UE implementation to decide the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI. Meanwhile, there is UE-gNB/UE-UE implementation inconsistence concern.

- Confirm the WA that PQI is used to determine the CAPC mapping as in NR-U (i.e. CAPC to PQI mapping table is introduced).

- Consider L1 channel access parameters for each CAPC concluded by RAN1 together with PDB, when specifying the CAPC to PQI mapping table.

- Send LS to RAN1, informing them of the specific CAPC to PQI mapping table, once concluded by RAN2.

- For a CONNECTED UE, it is up to gNB implementation to configure the CAPC of non-standardized PQI using the CAPC of the standardized PQI which best matches the QoS characteristics of the non-standardized PQI.

- For an IDLE/INACTIVE/OOC UE, if the QoS flow of non-standardized PQI can be mapped to a non-default SLRB, the UE determines the CAPC of this non-standardized PQI using the CAPC of this SLRB.

- RAN2 to discuss whether gNB can configure a CPAC value for default SLRB.

- For an IDLE/INACTIVE/OOC UE, if the QoS flow of non-standardized PQI cannot be mapped to a non-default SLRB, the UE determines the CAPC of current non-standardized PQI by down-selecting from one of the following options:

- option A: use the CAPC of the standardized PQI or the CAPC of non-standardized PQI configured in SIB/pre-configuration which best matches the QoS characteristics of the current non-standardized PQI based on one or more QoS characteristics, e.g. by comparing PDB, PriorityLevel or other QoS characteristics;

- option B: use default CAPC value, e.g. the CAPC of default SLRB if it is confirmed in proposal 6 that gNB configures CAPC for default SLRB, or a default CAPC which is pre-defined or configured by SIB/pre-configuration for this case;

- RAN2 agree the following rule to determine the CAPC of an SL TB, when the CAPC value is not indicated in the DCI for mode-1 and when the UE is operating in mode-2:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used;

- If SBCCH SDU is included in the SL TB, the highest priority SL CAPC is used;

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- The lowest priority CAPC of the SL logical channel(s) with MAC SDU multiplexed in the SL TB is used otherwise.

References

- RAN2-119bis-e chairman notes;

- Chair's notes RAN1_110bis-e v17;

Annex

- 3GPP TS 23.287 V17.4.0 (2022-09)

- 5.4.4	Standardized PQI to QoS characteristics mapping

The one-to-one mapping of standardized PQI values to PC5 QoS characteristics is specified in table 5.4.4-1.

Table 5.4.4-1: Standardized PQI to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21 | GBR | 3 | 20 ms | 10-4 | N/A | 2000 ms | Platooning between UEs – Higher degree of automation; Platooning between UE and RSU – Higher degree of automation |
| 22 | (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Sensor sharing – higher degree of automation |
| 23 |  | 3 | 100 ms | 10-4 | N/A | 2000 ms | Information sharing for automated driving – between UEs or UE and RSU - higher degree of automation |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 58 |  | 4 | 100 ms | 10-2 | N/A | N/A | Sensor information sharing – lower degree of automation |
| 59 |  | 6 | 500 ms | 10-1 | N/A | N/A | Platooning – reporting to an RSU |
| 90 | Delay Critical GBR | 3 | 10 ms | 10-4 | 2000 bytes | 2000 ms | Cooperative collision avoidance; Sensor sharing – Higher degree of automation; Video sharing – higher degree of automation |
| 91 | (NOTE 1) | 2 | 3 ms | 10-5 | 2000 bytes | 2000 ms | Emergency trajectory alignment; Sensor sharing – Higher degree of automation |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

NOTE 1:	For Standardized PQI to QoS characteristics mapping, the table will be extended/updated to support service requirements for other identified V2X services.

NOTE 2:	The PQIs may be used for other services than V2X.

NOTE 3:	A PQI may be used together with an application indicated priority, which overrides the Default Priority Level of the PQI.

- 3GPP TS 23.304 V17.4.0 (2022-09)

- 5.6.1	QoS handling for 5G ProSe Direct Communication

Table 5.6.1-1: Standardized PQI values that are additionally defined to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | GBR (NOTE 1) | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 25 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 26 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 60 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 61 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9 as specified in TS 23.501 [4]) |
| 92 | Delay Critical GBR (NOTE 1) | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset (See TS 22.261 [6]) |
| 93 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging (See TS 22.261 [6]) |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |