3GPP TSG-RAN WG2 Meeting #120                                               	                R2-2211475

Toulouse, France,  November 14 – 18, 2022

Agenda item:	    8.1.3

Source: 		Qualcomm Incorporated

Title: 	Management of network-controlled repeater

Document for:	Discussion

# 1	Introduction

The WID on Rel-18 network-controlled repeaters holds the following objectives [1]:

Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2]: - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability.

SA3 sent a reply LS on NCR solutions and stated the following [2]:

To SA3 Q1a: Is there any security issue for solution 2 which does not provide Uu security, non-protected NCR indication info and the OAM container in Step 5? Answer to RAN3: Yes. For solution 2, SA3 believes that this information can be tampered due to the lack of Uu security. It exposes the OAM indirectly to attacks over the air interface. To SA3 Q1b: Does SA3 believe that the NCR needs to be securely validated? Any security issue for configuring locally stored information in the gNB in Solution 1? Answer to RAN3: For the 1st question in Q1b, SA3 is not clear about what does "validation" mean. - For the 2nd question in Q1b, SA3 cannot provide answers before the security validation related steps in solution1 are clarified. In addition, the feasibility of such additional steps and what kind of information is stored in RAN are also unclear. Further clarification is expected.

RAN3#117bis-e agreed the following [3]:

The NCR authorization indicator is provided from AMF to gNB explicitly over the NG interface. - It can be applied to any solution selected.

This contribution discusses management of the NCR.

# 2	Discussion

SA3 confirmed that Solution 2 has security issues [2]. Solution 2 also does not meet the WID requirement for inter-vendor operability. Solution 2 should therefore be deprioritized.

Proposal 1: Solution 2 is deprioritized since it does not provide inter-vendor inter-operability and it does not meet SA3’s security requirements.

RAN3#117bis-e agreed that the AMF provides NCR authorization to the gNB over NGAP [2]:

The NCR authorization indicator is provided from AMF to gNB explicitly over the NG interface.

RAN3#117bis-e further confirmed that this CN-based authorization applies to any solution selected. This implies down-selection between Solutions 1, 3 and 4 is in RAN3’s scope.

Proposal 2: Solutions 1, 3 and 4 rely on CN-based authorization, which is in RAN3 scope.

If RAN3 selects Solution 1, RAN2 can discuss the RAN validation function.

Proposal 3: Discussions on the RAN validation function are pending the selection of Solution 1 by RAN3.

# Conclusion

This contribution discussed management of the NCR. The following proposals have been made:

Proposal 1: Solution 2 is deprioritized since it does not provide inter-vendor inter-operability and it does not meet SA3’s security requirements.

Proposal 2: Solutions 1, 3 and 4 rely on CN-based authorization, which is in RAN3 scope.

Proposal 3: Discussions on the RAN validation function are pending the selection of Solution 1 by RAN3.

# References

- RP-222673: New WID on NR network-controlled repeaters

- S3-223080: Reply LS on NCR Solutions, 3GPP TSG-SA3 Meeting #108e-AdHoc, e-meeting, October 10 – 14, 2022

- RAN3-117bis-e – Chairman Notes