3GPP TSG-RAN WG2 Meeting #120	R2-2211488

# Toulouse, France, 14th Nov. – 18th Nov. 2022

Agenda item:	8.4.3

Source:	vivo

Title:	Discussion on NR-DC with selective activation cell of groups

Document for:		Discussion & Decision

# 1. Introduction

In RAN2 #119bis-e meeting, the scenarios, as well as some general principles for NR-DC with selective activation cell of groups were discussed and the following agreements have been reached[1]:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs): - a. Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell. - b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells. - c. Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell. Confirm that “CPA” selective activation of cell groups will be supported for this WI objective Confirm that we aim to support delta configuration, i.e. that there need to be a known reference. RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In this contribution, we will further discuss the left scenarios, security aspects and the configuration details e.g. about delta configuration to support this subsequent cell group change and give our suggestions on that.

# 2. Discussion

## 2.1 Scenarios

According to the agreement so far for subsequent CPAC, there are aspects that need to be further discussed as follows:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs): - a. Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell. - b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells. - c. Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell. Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

There are three issues needing to be discussed further:

- Issue 1: Whether the agreed ‘Baseline procedure’ applies to both MN-initiated CPC and SN-initiated CPC?

- Issue 2: Whether the scenario for subsequent secondary cell group change should be extended to cover more reasonable scenarios, e.g. ‘SCG release’?

- Issue 3: As ‘CPA selective activation of cell groups will be supported’, what is the CPA configuration used for, e.g. subsequent CPC or CPA?

We would discuss these three questions in the following sections.

### Issue 1: MN-initiated CPC and SN-initiated CPC

For MN-initiated CPC procedure, as the MN is aware of the CPC configuration and no change of MN, there seems no difficulty to support this scenario for subsequent CPC procedure.

For SN-initiated intra-SN CPC procedure, it would be simple to be supported as no security key refresh is needed because the cell change would be within on CU, and also the execution conditions configuration is within a same SN to make things easier.

For SN-initiated inter-SN CPC procedure, it is doubtful whether CPC configuration received previously by UE can be valid after the PSCell changes. In order to make the configuration valid, the coordination between source PSCell, target PSCell, and candidate PSCells may be needed. However, we think it is beneficial to support SN-initiated conditional CPC procedure. In RAN2 #116e meeting, the following agreement was reached:

No consensus to support A3/A5 for PSCell in MN-initiated CPC.

With configuring A3/A5 events in SN-initiated conditional CPC procedure as the execution condition, the CPC procedure may have better performance with considering both candidate PSCell and serving PSCell, therefore, we prefer to also support the SN-initiated CPC scenario for subsequent CPC procedure.

Proposal 1: Configurations of both MN-initiated and SN-initiated CPC can be kept after PSCell change for subsequent CPC evaluation/execution.

If the SN-initiated CPC procedure is agreed to be supported, then maybe we could go to some details about how the UE can keep the configuration for subsequent CPC evaluation/execution.

Let’s take the SN-initiated conditional PSCell Change procedure as an example, in the CPC preparation phase, the step 1 (source SN sending SN Change Required to MN) is as follows[2]:

## 10.5 Secondary Node Change (MN/SN initiated) ### 10.5.2 MR-DC with 5GC SN initiated conditional SN Change <omitted…> 1. The source SN initiates the conditional SN change procedure by sending the SN Change Required message, which contains a CPC initiation indication. The message also contains candidate node ID(s) and may include the SCG configuration (to support delta configuration), and contains the measurements results which may include cells that are not CPC candidates. The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells that can be prepared by the candidate SN(s), and may also include the SCG measurement configurations for CPC (e.g. measurement ID(s) to be used for CPC).

Observation 1: For SN-initiated inter-SN CPC procedure, the source SN decides and sends candidate node ID(s) and the associated execution conditions to MN. And SCG measurement configuration may also be included for MN to interpret the execution conditions.

Also, in TS 38.331, it can be seen that what is provided by MN as follows[3]:

CG-Config-v1700-IEs ::= SEQUENCE { candidateCellInfoListCPC-r17 CandidateCellInfoListCPC-r17 OPTIONAL, twoPHRModeSCG-r17 ENUMERATED {enabled} OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } CandidateCellInfoListCPC-r17 ::= SEQUENCE (SIZE (1..ffsUpperLimit)) OF CandidateCellInfo-r17 -- FFS CandidateCellInfo-r17 ::= SEQUENCE { ssbFrequency-r17 ARFCN-ValueNR, candidateList-r17 SEQUENCE (SIZE (1..ffsUpperLimit)) OF CandidateCell-r17 -- FFS } CandidateCell-r17 ::= SEQUENCE { physCellId-r17 PhysCellId, condExecutionCondSCG-r17 OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL -- FFS whether the Optional flag is to be removed from condExecutionConditionSN-r17 }

In order to support the SN-initiated subsequent CPC procedure, we understand from the NW point of view, after the PSCell change, the target SN should be provided by MN with at least the following information that is also included in CandidateCellInfoListCPC-r17 from source SN to MN:

- SSB frequency and physical cell ID for candidate PSCell(s).

- Associated execution conditions.

With the above information, the target SN may decide whether to update the subsequent CPC configuration to UE e.g. reconfigure the execution conditions or release the candidate PSCell(s).

Proposal 2: After PSCell change, the target SN is informed by MN with the following information to decide whether an update for configuration of SN-initiated CPC is needed:

- Candidate cell ID, i.e. SSB frequency and physical cell ID for candidate PSCell(s).

- Associated execution conditions.

### Issue 2: SCG release in selective cell group change procedure

In Rel-17, CPC configuration is also released upon the release of SCG. Meanwhile, the CPA configuration has been released once the UE is changed from SA to DC, i.e. after PSCell addition procedure or CPA procedure. Therefore, there is no CPA configuration when the release of SCG happens.

However, in Rel-18, considering the concept of subsequent CPA/CPC, we think it is actually possible for the UE to keep the CPAC configuration after SN release for subsequent CPA evaluation/execution, and for subsequent CPC evaluation/execution when the UE is added with SN again. In this way, the UE is possible to go back to DC mode without reconfiguration/re-initiation of CPA, and can continue the CPC evaluation/execution while staying in DC mode.

Proposal 3: After releasing SCG, CPAC configuration is kept by UE for subsequent CPAC evaluation/execution.

### Issue 3: CPA selective activation of cell groups

In the following agreement, it is not clear what the kept CPA configuration is used for.

Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

There can be two different understandings:

- After successful PSCell addition procedure (normal or conditional), UE is allowed to keep CPA configurations for further CPC evaluation/execution.

- After successful PSCell addition procedure (normal or conditional), UE is allowed to keep CPA configurations for later subsequent CPA evaluation/execution when SN release happens.

We understand for the above option-a, it is straightforward to be supported so that the CPA configuration can be further utilized by UE for CPC, especially considering A4 events can be configured for both MN-initiated CPA or MN-initiated CPC. For option-b, there seem also benefits so that the UE is possible to go back to DC mode without reconfiguration/re-initiation of CPA. Therefore, we propose:

Proposal 4: RAN2 to confirm that after successful PSCell addition procedure (normal or conditional), UE is allowed to keep CPA configurations for the following two operations:

- Subsequent CPC evaluation/execution

- Subsequent CPA evaluation/execution after SN release happens

Meanwhile, if it is confirmed that CPA configurations can be kept for CPC, a similar issue can be discussed, which is whether CPC configuration can be used for CPA as well when SN release happens. As we mentioned before, this is possible because A4 events can be configured for both MN-initiated CPA or MN-initiated CPC. Therefore,

Proposal 5: RAN2 to discuss that after successful PSCell change procedure (normal or conditional), whether UE is allowed to keep CPC configurations for the following operation:

- Subsequent CPA evaluation/execution after SN release happens, if applicable (e.g. execution condition is based on A4 event)

## 2.2 Differentiation for CPA and CPC configuration

In RAN2 #119bis-e meeting, the following proposal was discussed but without consensus:

Proposal 2: for a UE without SCG, a group of candidate cells can be configured with condition for CPA and different condition for subsequent CPC.

It is not clear whether a same configuration can be used for both CPA or CPC, or different configurations should be used, e.g. with different execution conditions.

In Rel-17, the CPA and CPC configuration are using the same configuration structure wherein a config-ID, execution condition(s), and SCG configuration. Only SN initiated inter-SN CPC would be separately indicated by using a field condExecutionCondSCG-r17.

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 CondReconfigToAddMod-r16 ::= SEQUENCE { condReconfigId-r16 CondReconfigId-r16, condExecutionCond-r16 SEQUENCE (SIZE (1..2)) OF MeasId OPTIONAL, -- Need M condRRCReconfig-r16 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Cond condReconfigAdd ..., [[ condExecutionCondSCG-r17 OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL -- Need M ]] } CondReconfigExecCondSCG-r17 ::= SEQUENCE (SIZE (1..2)) OF MeasId

The logic here is clear, for a CPAC configuration provided to a UE, it is for CPA when the UE is in Standalone and for CPC while in Dual-connectivity mode.

Observation 2: In Rel-17, CPAC configuration is used for CPA or CPC based on whether UE is in DC mode or not when the CPAC configuration is received.

However, in Rel-18, for some scenarios, it should be further discussed how the CPAC configuration works, e.g., as we agreed “CPA” selective activation of cell groups will be supported, it should be questioned in this scenario, whether a same set of CPAC configurations can serve the UE or separate CPA and CPC configuration would be needed, as the UE would perform subsequent CPC after successful CPA?

Observation 3: It is not clear whether a same set of CPAC configuration(s) is used for UE or separate CPA and CPC configuration are needed, e.g. in subsequent CPC procedure after successful CPA procedure performed by the UE, without any reconfiguration.

In our understanding, there may be two possible solutions:

Solution 1: No differentiation of CPAC configuration, which means a same set of CPAC configurations can be used for CPA and/or CPC.

Solution 2: CPA and CPC configuration should be differentiated, so that the UE knows which configuration is for which subsequent procedure (CPA or CPC)

For solution 1, as we discussed before, A4 event can be configured for both CPA and MN initiated CPC. In case the MN thinks the same A4 threshold can be used for CPA and CPC, then the corresponding conditional reconfiguration can be applied for both CPA and CPC, i.e. Solution1. Otherwise, the corresponding conditional reconfiguration can be applied separately for CPA or CPC, i.e. Solution2.

Observation 4: If a same set of CPAC configuration(s) is used for CPA and CPC, only A4 events are possible for execution condition and the threshold value can only be the same.

In order to achieve more flexibility, we understand the differentiation is preferred in CPAC configuration to indicate it is for subsequent CPA or CPC.

Proposal 6: At least for MN-initiated CPAC associated with A4 event, MN can indicate UE whether the CPAC configuration can be applied to only CPA, only CPC or both CPA and CPC.

## 2.3 Keeping and releasing for subsequent CPAC configuration

In RAN2 #119bis-e agreement, there is an FFS as follows:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

The FFS is related to one of the most important question to answer, which is:

Question 1: How to enable non-release of CPAC configuration on top of Rel-17 framework?

For question 1, as the release of CPAC configuration would be performed by Rel-17 UEs after successful (conditional) PSCell addition/change, the Rel-18 UEs should only keep the configurations in case of some extra indications, e.g. indication configured by network. In this way, the backward compatibility can be guaranteed.

On the other hand, even if the CPAC configuration can be kept by UE for further evaluation/execution, we should also specify the time point when it should be released, after all this kind of configuration may not be useful all the time. Considering the two aspects mentioned above, there are two possible schemes for the CPAC configuration maintaining and release:

Scheme 1: Indication to allow subsequent CPAC configuration is sent to UE in CPAC configuration procedure, and UE autonomously keep the configurations for subsequent CPAC procedure until NW explicitly releases them or certain condition is satisfied (e.g. when the associated PSCell has already been accessed).

Scheme 2: Indication to allow subsequent CPAC configuration is sent to UE after each successful (conditional) PSCell addition/change procedure, and then UE is allowed to keep the CPAC configurations for subsequent CPAC procedure.

Scheme 2 actually makes the subsequent CPAC procedure more controllable by network but requires more signalling between gNB and UE, although the signalling may be as simple as a one-bit indication or something. While scheme 1 makes it more flexible for the UE for subsequent CPAC procedure but specification works may be more than scheme 2. RAN2 can discuss which one is a better trade-off. Meanwhile, scheme 2 may lead to inter-operation issue e.g. when there is Rel-18 UE under Rel-17 gNB, the CPAC configuration is supposed to be released after successful CPAC procedure so the indication must be send at the very beginning of CPAC initiation procedure.

Therefore, our preference is that we go for scheme 1.

Furthermore, an FFS was left in RAN2#119e meeting:

- FFS how many subsequent conditional changes are targeted (and what is the impact of such assumption).

We understand the motivation to define the upper bound for times of subsequent CPC procedure also serve the purpose of determining when the UE should release the subsequent CPAC configuration. In this sensing, both timer-based or counter-based solutions can be considered, for the UE to decide when to release the CPAC configuration, e.g. after a period of time or after reaching the maximum number allowed for subsequent CPAC. Combining the motivation and the scheme 1 and 2 as we analysed above, we propose:

Proposal 7: Indication to allow subsequent CPAC configuration is sent to UE in CPAC initiation procedure. Then the CPAC configuration is kept by UE until being modified or released explicitly by network. FFS on supporting other condition to release CPAC configuration, e.g. Counter/timer-based condition.

## 2.4 Delta configuration

According to the agreement in RAN2#119bis-e meeting as below, the delta configuration is supported for subsequent CPAC, and we would like to discuss how to support it.

Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

Delta configuration is not a new issue and could be supported for UEs already. However, the current delta configuration takes the source PSCell as reference, which couldn’t work well for subsequent CPAC since the source PSCell will be changed, and the saved delta configuration will be invalid after that. Hence, we should discuss how to determine the reference configuration for delta configuration in subsequent CPAC.

Observation 5: Taking the configuration of source PSCell as reference for delta configuration cannot work in subsequent CPAC scenario after PSCell changes.

To solve this problem, UE can simply use a reference configuration. With some ‘absolute reference configuration’, all the following delta configuration can be achieved with the same understanding for target SNs and UE. However, how a UE determines an RRC configuration (e.g. the candidate cell#1) is a reference configuration should be studied, basically there are two options:

- Option 1: Static reference configuration. E.g. the very first source PSCell configuration, or a pre-defined PSCell configuration;

- Option 2: Non-static reference configuration, e.g. based on gNB indication.

We think to achieve the flexibility and to make the update of the reference configuration more convenient, the preferred way could be introducing an indication in the configuration to indicate whether the configuration is a reference. Then, this reference configuration can be applied until the release/update condition for reference configuration is satisfied. Therefore,

Proposal 8: UE determines the reference configuration for delta configuration based on the network indication.

## 2.5 Security

In RAN2 #119e meeting, the security issue of NR-DC with selective activation cell of groups was discussed and there was an FFS left as follows:

FFS whether there is a security issue: e.g. to determine vertical or horizontal key derivation, e.g. security parameters re-used as part of subsequent CG switch (for the case when UE goes back to a previous cell, maybe in another SN), and FFS on the procedure/method with which the UE derives the SN security, e.g. based on a prior MN config (without RRC CPC config at the time of SN switch).

Later in RAN2 #119bis-e meeting, the security issue is further discussed with the following agreement:

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

With this aim, we further discuss what should be the solutions to achieve the goal.

In NR-DC scenario, the key used for a particular DRB is derived based on two parameters sk-counter and keyToUse. In detail, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KgNB) for a particular DRB, and the secondary key is derived from the master key and sk-Counter[2].

When UE receives the sk-counter, the key derivation/update would be performed by the UE, as follows[3]:

#### 5.3.5.7 AS Security key update The UE shall: <omitted…> 1> else if this procedure was initiated due to reception of the sk-Counter (UE is in NE-DC, or NR-DC, or is configured with SN terminated bearer(s)): 2> derive or update the secondary key (S-KgNB or S-KeNB) based on the KgNB key and using the received sk-Counter value, as specified in TS 33.501 [11]; 2> derive the KRRCenc key and the KUPenc key as specified in TS 33.501 [11] using the ciphering algorithms indicated in the RadioBearerConfig associated with the secondary key (S-KgNB or S-KeNB) as indicated by keyToUse; 2> derive the KRRCint key and the KUPint key as specified in TS 33.501 [11] using the integrity protection algorithms indicated in the RadioBearerConfig associated with the secondary key (S-KgNB or S-KeNB) as indicated by keyToUse; NOTE 2: If the UE has no radio bearer configured with keyToUse set to secondary and receives the sk-Counter without any RadioBearerConfig with keyToUse set to secondary, the UE does not consider it as an invalid reconfiguration.

Observation 6: In DC, for radio bearers which are configured with keyToUse set to secondary, secondary key would be derived/updated when UE is provided with sk-counter.

Moreover, from the network point of view, whether to update the secondary key (KSN) and provide sk-counter to UE can be based on different reasons, e.g. whenever MN would like to update the KSN, or when SN requests the MN to update the KSN if uplink and/or downlink PDCP COUNTs are about to wrap around for any of the SCG DRBs or SCG SRB[4]. From UE’s point of view, the UE does not need to maintain the counter value of sk-counter after it has computed the KSN since the MN provides the UE with the current sk-counter value when the UE needs to compute a new KSN.

Observation 7: Whether to update the secondary key is up to network decision and the UE only needs to be provided with a fresh sk-Counter each time when the UE needs to compute a new secondary key.

According to the current mechanism, when we discuss the subsequent CPAC procedure, how this secondary key derivation/update procedure can work should be discussed. In normal PSCell change procedure or R16 CPC in NR-DC, for a PSCell change that does not require a KgNB chang, S-KgNB key refresh is not always required e.g. if the PDCP termination point of the SN is not changed[2]. Therefore, the network can decide whether to provide sk-counter to UE in configuration associated to a PSCell based on e.g. whether the PDCP termination point of the SN has changed compared to previous PSCell. However, in subsequent CPAC procedure, as the network has no idea how the subsequent CPAC procedure would be performed especially about the order in which UE accesses to different PSCells, sk-counter may need to be always provided in the conditional configuration for a PSCell no matter for intra-SN or inter-SN scenario, because the UE may access to an intra-SN PSCell-A at the first time, then change to an inter-SN PSCell-B, and then change to an inter-SN PSCell-C which is actually under the same SN of PSCell-A. Therefore, we observe and propose:

Observation 8: In normal PSCell change procedure, the network can decide whether to provide sk-counter to UE based on e.g. whether the PDCP termination point of the SN is changed.

Observation 9: In subsequent CPAC procedure, the network has no idea in advance about the order in which UE accesses to different PSCells.

Proposal 9: To support subsequent CPC procedure, sk-counter should be always provided in the conditional configuration for a PSCell, since the network cannot determine in advance whether a candidate cell will be used as the target cell for intra-SN CPC or inter-SN CPC.

The next issue is about the sk-counter increment. For the counter maintenance, the sk-counter should be monotonically incremented by MN for each additional calculated KSN as follows[4]:

#### 6.10.3.1 SN Counter maintenance <omitted…> The MN maintains the value of the counter SN Counter for a duration of the current 5G AS security context between UE and MN. The UE does not need to maintain the SN Counter after it has computed the KSN since the MN provides the UE with the current SN Counter value when the UE needs to compute a new KSN. The SN Counter is a fresh input to KSN derivation. That is, the UE assumes that the MN provides a fresh SN Counter each time and does not need to verify the freshness of the SN Counter. NOTE: An attacker cannot, over the air modify the SN Counter and force re-use of the same SN Counter. The reason for this is that the SN Counter is delivered over the RRC connection between the MN and the UE, and this connection is both integrity protected and protected from replay. The MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. The MN shall set the SN Counter to ‘1’ after the first calculated KSN, and monotonically increment it for each additional calculated KSN. The SN Counter value '0' is used to calculate the first KSN. <omitted…>

Observation 10: In legacy secondary key derivation/update, MN shall monotonically increment SN Counter for each additional calculated KSN.

In subsequent CPAC procedure, as we discussed in Observation 9, the UE may access to different PSCells in random order and the sk-counter used by UE may not be in an ascending order. However, in our understanding the MN still monotonically increment SN Counter for each additional calculated KSN in conditional configuration preparation phase and on the UE side, as long as the sk-counter(s) for different PSCells are one-to-one mapping to the SN counter on MN side and are not repetitive, there should not be any security problem. This can be further confirmed by SA3, if needed.

Observation 11: In subsequent CPAC procedure, UE may use multiple sk-counter(s) which is not in an ascending order.

Then according to the chairman minutes, there is a scenario which needs to be considered that:

- for the case when UE goes back to a previous cell, maybe in another SN

Actually, whether we should consider the scenario for a UE to move back to the previous cell in subsequent CPAC procedure, is still under discussion and there is no agreement on this. The proposals from companies are like, e.g. to suggest UE can perform subsequent CPC to the same PSCell for more than one time.

Observation 12: Whether a UE can perform subsequent CPC to the same PSCell for more than one time is still hang in doubt.

We understand this scenario is ok to be supported, only if the security problem has been resolved. In our understanding, considering the previous cell could be in another SN compared to UE’s current serving PSCell, the UE should use different sk-counter value to calculate secondary key, thus keeping the computed key fresh. Also, if we look at the legacy SN release/re-add procedure, on network side, SN counter is also increased by MN to keep the computed KSN fresh, as in TS 33.501[4][4].

Observation 13: If the MN decides to release the SN and later re-add the same SN, the sk-Counter value shall keep increasing, thus ensure the UE to use different Ksn every time it accesses to the same serving cell.

Therefore, to support this case in Observation 12, the straightforward method, on top of our Proposal 9, is to allow multiple sk-counter configuration for a single PSCell. Therefore, we propose:

Proposal 10: Multiple sk-counter can be included in conditional configuration associated to one PSCell, to allow the UE to perform subsequent CPC to the same PSCell for more than one time.

For Observation 11 and Proposal 10, we think they should be confirmed with SA3 whether there are any security problem.

Proposal 11: RAN2 to send an LS to SA3 to confirm whether there is security issue on:

- Including multiple sk-counter in conditional configuration associated to one PSCell

- For one or more PSCells, UE using multiple sk-counters to derive/update secondary key in an order which is not monotonically incremented, with considering the sk-counters are not repetitive.

At last, combining Proposal 10 to 12, then the whole procedure for key derivation/update in subsequent CPAC procedure can be as follows:

- Step 1: Network provide multiple CPAC configuration to UE, in which each PSCell is associated to one or more sk-counter. All the sk-counters are not repetitive.

- Step 2: After CPAC execution and UE access to a new PSCell, UE uses a new sk-counter to derive the secondary key.

- Step 3: If the UE access to a previous PSCell, the UE uses a new sk-counter to derive the secondary key which is different from the ones has already been used when UE access to that PSCell previously. If no unused sk-counter is available, the access to the concerned PSCell is not allowed.

Proposal 12: RAN2 to consider the following baseline procedure for security aspect in selective activation of cell groups:

- Step 1: Network provide multiple CPAC configuration to UE, in which each PSCell is associated to one or more sk-counter. All the sk-counters are not repetitive.

- Step 2: After CPAC execution and UE access to a new PSCell, UE uses a new sk-counter associated to the PSCell to derive the secondary key.

- Step 3: If the UE access to a previous PSCell, the UE uses a new sk-counter associated to the PSCell to derive the secondary key.The sk-counter is different from the ones has already been used when UE access to that PSCell previously. If no unused sk-counter is available, the access to the concerned PSCell is not allowed.

# 3. Conclusion

In this contribution, we discussed about the left scenarios, security aspects and the configuration details e.g. about delta configuration to support this subsequent cell group change in subsequent CPAC procedure.

We have the following observations and proposals:

Observation 1: For SN-initiated inter-SN CPC procedure, the source SN decides and sends candidate node ID(s) and the associated execution conditions to MN. And SCG measurement configuration may also be included for MN to interpret the execution conditions.

Observation 2: In Rel-17, CPAC configuration is used for CPA or CPC based on whether UE is in DC mode or not when the CPAC configuration is received.

Observation 3: It is not clear whether a same set of CPAC configuration(s) is used for UE or separate CPA and CPC configuration are needed, e.g. in subsequent CPC procedure after successful CPA procedure performed by the UE, without any reconfiguration.

Observation 4: If a same set of CPAC configuration(s) is used for CPA and CPC, only A4 events are possible for execution condition and the threshold value can only be the same.

Observation 5: Taking the configuration of source PSCell as reference for delta configuration cannot work in subsequent CPAC scenario after PSCell changes.

Observation 6: In DC, for radio bearers which are configured with keyToUse set to secondary, secondary key would be derived/updated when UE is provided with sk-counter.

Observation 7: Whether to update the secondary key is up to network decision and the UE only needs to be provided with a fresh sk-Counter each time when the UE needs to compute a new secondary key.

Observation 8: In normal PSCell change procedure, the network can decide whether to provide sk-counter to UE based on e.g. whether the PDCP termination point of the SN is changed.

Observation 9: In subsequent CPAC procedure, the network has no idea in advance about the order in which UE accesses to different PSCells.

Observation 10: In legacy secondary key derivation/update, MN shall monotonically increment SN Counter for each additional calculated KSN.

Observation 11: In subsequent CPAC procedure, UE may use multiple sk-counter(s) which is not in an ascending order.

Observation 12: Whether a UE can perform subsequent CPC to the same PSCell for more than one time is still hang in doubt.

Observation 13: If the MN decides to release the SN and later re-add the same SN, the sk-Counter value shall keep increasing, thus ensure the UE to use different Ksn every time it accesses to the same serving cell.

Proposal 1: Configurations of both MN-initiated and SN-initiated CPC can be kept after PSCell change for subsequent CPC evaluation/execution.

Proposal 2: After PSCell change, the target SN is informed by MN with the following information to decide whether an update for configuration of SN-initiated CPC is needed:

- Candidate cell ID, i.e. SSB frequency and physical cell ID for candidate PSCell(s).

- Associated execution conditions.

Proposal 3: After releasing SCG, CPAC configuration is kept by UE for subsequent CPAC evaluation/execution.

Proposal 4: RAN2 to confirm that after successful PSCell addition procedure (normal or conditional), UE is allowed to keep CPA configurations for the following two operations:

- Subsequent CPC evaluation/execution

- Subsequent CPA evaluation/execution after SN release happens

Proposal 5: RAN2 to discuss that after successful PSCell change procedure (normal or conditional), whether UE is allowed to keep CPC configurations for the following operation:

- Subsequent CPA evaluation/execution after SN release happens, if applicable (e.g. execution condition is based on A4 event)

Proposal 6: At least for MN-initiated CPAC associated with A4 event, MN can indicate UE whether the CPAC configuration can be applied to only CPA, only CPC or both CPA and CPC.

Proposal 7: Indication to allow subsequent CPAC configuration is sent to UE in CPAC initiation procedure. Then the CPAC configuration is kept by UE until being modified or released explicitly by network. FFS on supporting other condition to release CPAC configuration, e.g. Counter/timer-based condition.

Proposal 8: UE determines the reference configuration for delta configuration based on the network indication.

Proposal 9: To support subsequent CPC procedure, sk-counter should be always provided in the conditional configuration for a PSCell, since the network cannot determine in advance whether a candidate cell will be used as the target cell for intra-SN CPC or inter-SN CPC.

Proposal 10: Multiple sk-counter can be included in conditional configuration associated to one PSCell, to allow the UE to perform subsequent CPC to the same PSCell for more than one time.

Proposal 11: RAN2 to send an LS to SA3 to confirm whether there is security issue on:

- Including multiple sk-counter in conditional configuration associated to one PSCell

- For one or more PSCells, UE using multiple sk-counters to derive/update secondary key in an order which is not monotonically incremented, with considering the sk-counters are not repetitive.

Proposal 12: RAN2 to consider the following baseline procedure for security aspect in selective activation of cell groups:

- Step 1: Network provide multiple CPAC configuration to UE, in which each PSCell is associated to one or more sk-counter. All the sk-counters are not repetitive.

- Step 2: After CPAC execution and UE access to a new PSCell, UE uses a new sk-counter associated to the PSCell to derive the secondary key.

- Step 3: If the UE access to a previous PSCell, the UE uses a new sk-counter associated to the PSCell to derive the secondary key.The sk-counter is different from the ones has already been used when UE access to that PSCell previously. If no unused sk-counter is available, the access to the concerned PSCell is not allowed.

# 4. References

RAN2 #119bis-e chairman notes;

3GPP TS 37.340 V17.1.0 (2022-06);

3GPP TS 38.331 V17.1.0 (2022-06)

3GPP TS 33.501 V17.6.0 (2022-06)