3GPP TSG-RAN WG1 Meeting #120 	R2-2211602

Toulouse, France, November 14-18, 2022

Agenda item:	    8.3.5

Source: 		Qualcomm Incorporated

Title: 	NES Connected Mode Mobility

Document for:	Discussion

# 1	Introduction

The SID on Rel-18 network energy saving holds the following objectives [1]:

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed. Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

In the last meeting, connected mode mobility as a target for NES technique was briefly discussed with the following agreements reached:

=> Scenario 1: UEs are HO’ed due to switch of SOURCE cell to NES mode is considered for further study. FFS whether any enhancements is needed. => FFS Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected based on its mode of operation => As a first priority, discussion on RAN2 group handover are confined to the CHO framework

In this contribution, we give our view on NES for connected mode mobility

# 2 	Source Cell Handover

## 2.1 Motivation

The motivation for source cell HO is to facilitate offloading UEs from a source cell, when the source cell has a low load that can be efficiently served by neighbouring cells. This allows cells with low load to sleep and save energy without radio quality degradation.

Observation 1: Source cell HO allow cells with low load to sleep after offloading camped UEs to neighbouring cells.

## 2.2 Procedure

As per the meeting agreement:

As a first priority, discussion on RAN2 group handover are confined to the CHO framework

It is proposed that the UEs in the cell are configured with NES CHO triggers which can be activated by a source cell that wants to offload UEs for NES purposes. Note that this does not mean that all camped UEs would handover into the same target cell, but rather, all signalled UEs will perform a preconfigured CHO command in order to handover to the target cell indicated by an earlier CHO configuration.

Proposal 1: Source cell HO is configured by RRC as CHO commands to be triggered upon indication from the source cell.

For specific CHO triggers, this can be further assessed by RAN2 to find a low-complexity low-overhead solution that maximizes NES gains.

Observation 2: Attempting to trigger CHO for every camped UE in a unicast manner at source cell causes high signalling overhead, high energy consumption of repeated unicast signalling, and high latency to complete HO at every UE (due to limited PDCCH capacity) which shortens the sleeping time.

Proposal 2: RAN2 to introduce new L1/L2 group signalling to trigger CHO at all camped UEs before source cell transfers into sleep state.

Note that although this scenario targets low loads, this does not necessarily mean that the number of UEs in the cell are low as there could be a large number of UEs transmitting/receiving low traffic loads.

To enable this new trigger, CHO configurations need some modification to at least mandate that the UE performs a pre-configured CHO upon receiving the new trigger. Thus, it is useful for RAN2 to discuss how CHO configurations can be enhanced to accommodate the new required behavior.

Proposal 3: RAN2 to discuss RRC CHO configurations enhancements needed to realize source cell HO.

# 3	Target Cell Handover

## 3.1 Motivation

For scenario 2, the goal is to make HO decisions not only based on radio link quality between individual UE and target cell, but rather based on the overall Network energy savings, this can be illustrated through the following examples:

- UE detects source link degradation, initiates handover to an NES target cell in sleep mode (whereby the sleep mode here does not comprise of switching off reference signalling transmission).

- UE detects source link degradation, initiates handover to an NES target cell in the process (or about to start the process) of offloading camped UEs to neighbouring cells and go to sleep mode (whereby the sleep mode can comprise switching off reference signals). In this case one of two things may happen:

  - Source cell may abort the process of going into sleep mode, reducing the overall NES gains possible.

  - Source cell may still trigger NES-motivated HO which means that this UE would ping-pong between cells attempting to go into sleep mode. We emphasize that this is not a corner case but rather the expected/likely behaviour when scenario 1 is applied without scenario 2 for one or more UEs.

- Low traffic NES UE may handover to a target cell that does not apply cell DTX contributing to an increase to overall network energy consumption.

- Another use case is that the UE is configured with a DRX configuration and needs to switch the source SCell into a target SCell where the UE C-DRX cycle is misaligned with cell DTX cycle. This means that the SCell cannot properly sleep during the cell DTX inactive period.

We also not that this uses the same rationale as the cell (re)-selection for idle UEs, where it was agreed that there is a need to discourage/deprioritize UEs from accessing cells that are attempting to go into some NES mode.

Thus, we conclude with the following two observations:

Observation 3: NES techniques would be undermined without NES-aware target cell CHO.

Observation 4: Source cell CHO without target cell CHO (Scenario 1 without Scenario 2) can cause UEs to ping-pong between cells attempting to perform source cell CHO to save energy.

Proposal 4: RAN2 to study target-cell aware CHO as an NES technique.

## 3.2 Procedure

The idea behind target cell CHO is to identify new HO events that are implicitly or explicitly influenced by the cell NES-state. This means that CHO events can be enhanced explicitly include an NES state that is communicated to the UE or define new CHO that implicitly disincentivize the UE from HOing into a target cell in NES state without forbidding the UE if no non-NES-state target cells have good radio quality to the UE.

Observation 5: NES-aware target cell CHO can balance overall NES considerations with individual UE QoS and continuity of service.

Observation 6: NES-aware target cell CHO can be performed with or without UE awareness of NES cell state at the UE, e.g., the UE can be transparent to NES-state of the cell but configured with two possible target cells with different offsets whereby the non-NES cell is favoured if radio link qualities to the UE are comparable.

Proposal 5: RAN2 to discuss the following two options for NES aware target cell CHO

- Option 1: Define new NES-aware CHO events.

- Option 2: Enhance the existing CHO events to include implicit or explicit NES state of target cell.

# BWP adaptation

It is proposed in [3] that connected UEs are configured with an NES-specific BWP, and group-common signaling is used to swich UEs to the latter when the cell enters NES mode. This proposal can be readily achieved based on legacy BWP operation and with no explicit signaling for BWP switching as follows:

- The network provides BWP configuration to served UEs in a unicast manner. The network aligns the default BWP for the UEs by implementation. This default BWP part is referred to as “NES-specific BWP”

- When the serving cell moves to NES, all UEs switch to the NES-specific BWP based on expiry of the bwp-Inactivity timer due to lack of activity

- The network may re-initiate activity on any other BWP at any time via DCI

Proposal 6: Deprioritize discussions on BWP adaptation + group signalling

# Conclusion

Observation 1: Source cell HO allow cells with low load to sleep after offloading camped UEs to neighbouring cells.

Proposal 1: Source cell HO is configured by RRC as CHO commands to be triggered upon indication from the source cell.

Observation 2: Attempting to trigger CHO for every camped UE in a unicast manner at source cell causes high signalling overhead, high energy consumption of repeated unicast signalling, and high latency to complete HO at every UE (due to limited PDCCH capacity) which shortens the sleeping time.

Proposal 2: RAN2 to introduce new L1/L2 group signalling to trigger CHO at all camped UEs before source cell transfers into sleep state.

Proposal 3: RAN2 to discuss RRC CHO configurations enhancements needed to realize source cell HO.

Observation 3: NES techniques would be undermined without NES-aware target cell CHO.

Observation 4: Source cell CHO without target cell CHO (Scenario 1 without Scenario 2) can cause UEs to ping-pong between cells attempting to perform source cell CHO to save energy.

Proposal 4: RAN2 to study target-cell aware CHO as an NES technique.

Observation 5: NES-aware target cell CHO can balance overall NES considerations with individual UE QoS and continuity of service.

Observation 6: NES-aware target cell CHO can be performed with or without UE awareness of NES cell state at the UE, e.g., the UE can be transparent to NES-state of the cell but configured with two possible target cells with different offsets whereby the non-NES cell is favoured if radio link qualities to the UE are comparable.

Proposal 5: RAN2 to discuss the following two options for NES aware target cell CHO

- Option 1: Define new NES-aware CHO events.

- Option 2: Enhance the existing CHO events to include implicit or explicit NES state of target cell.

Proposal 6: Deprioritize discussions on BWP adaptation + group signalling

# References

- RP-213544: New SI: Study on network energy savings for NR; TSG RAN Meeting #94, Electronic Meeting, Dec 6 - 17, 2021

- RAN2 119 bise – Chair Notes