3GPP TSG RAN WG2 #120	R2-2212637

Toulouse, FR, 11th – 14th November, 2022

Agenda item:	8.5.4.2

Source:	CMCC

Title:	Enhancement on CG for XR-specific capacity improvement

Document for:	Decision, Discussion

1	Introduction

In RAN2 119bis-e, some consensuses on scheduling enhancement for XR have been reached, some of which are shown as follows,

1: Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting.

2: RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

In this contribution, some potential enhancements on CG (configured grant) will be discussed.

2	Discussion

2.1 Overview

In [1], the data rate of DL VR traffic is expected to be 30mbps or 40mbps, which is similar with that of DL cloud gaming (CG) and same with that of DL AR, and the data rate of UL AR is expected to be 10mbps or 20mbps. The frame rate for all services is supposed to be at least 60 frame per second, which means the minimal interval of scheduling should not exceed 16.7ms at most. For certain services like CG and AR, there also exist PDB requirements.

To exploit the characteristics of XR data is necessary for study the enhancements on XR-specific capacity improvements. Generally, the UL XR data includes the input data from user and controller or game pad, as well as feedback on reception of DL data and some other signaling, those data are relatively small in quantity when compared to the high-resolution video frames in DL and arrives at relatively stable interval since the head tracking or pose tracking are implemented by hardware with fixed reporting rate.

However, when it comes to UL AR traffic, the UE will send data on reality (vision or LBS) to gNB and application server for virtual texture or information overlay. Data in UL AR traffic are usually composed by static or dynamic picture, video, audio, and geographic or position information, which also have their own reporting interval or reporting rate. AR service is more sensitive to UL bandwidth than other XR services.

2.2 Enhancement on CG

As previously analyzed, the UL traffic of XR service has independent but stable packet arrival interval and packet size for each type of service, e.g., user input, feedback from controller or game pad, head tracking or pose control information, and feedback on DL reception. This kind of feature makes CG a potential candidate for XR-specific capacity improvements.

Since that CG will pre-assigned physical radio resource to UE before the real transmission begins, UE doesn’t have to send SR in PUCCH for requesting UL Grant and it will reduce the over all delay in UL. At the same time, since the physical radio resource assigned to UE are pre-determined in CG, the delay caused by the absence of available resource can also be eliminated.

The other potential benefit of CG is power saving. Monitoring PDCCH has been regarded as a major reason for the UE power consumption. From our perspective, CG can make it possible to configure less and short period for UE to monitoring PDCCH, which, if possible, can be discussed during next stage.

Observation 1: CG benefits user experience in XR service due to the certainty of pre-configured radio resource and less delay compared with DG.

However, there also exist problems in CG which makes legacy CG can not fulfill the requirements of XR service. From our perspective, the most important problems in current stage are the mismatch between (video frame) packet arrival interval and CG period, and the inconsistency between the actual data volume of XR traffic and pre-configured transmission block size (TBS) in CG.

Proposal 1: RAN2 is kindly asked to confirm that legacy CG needs to be enhanced for the consistency between CG period and frame arrival interval, the adoption of actual data volume of XR traffic, and jitter handling.

An easy solution is that gNB can configure multiple legacy CG configurations with independent period and TBS to eliminate the mismatch and inconsistency caused by packet arrival interval and actual data volume of XR traffic. The performance of this solution depends mainly on the number of legacy CG configurations and whether the parameters of legacy CG configurations reflect the characteristic of UL XR traffic.

A common concern is that since every CG configuration will be configured with at most 16 HARQ processes and the max number of PUSCH HARQ processes is 16 or 32, multiple legacy CG configurations will occupy so many HARQ processes that would even have impacts on UE for other LCH transmission. Due to the stop-and-wait protocol performed in HARQ, less available HARQ processes will make HARQ entity waste more time waiting for feedback and impossible to perform uninterrupted transmission.

The other concern is the signaling overhead in activation or deactivation for the desired legacy CG configuration. Activating multiple CG configuration will not only occupy HARQ processes but also make the pre-configured radio resource in CG unavailable for other users. Therefore, for high radio resource utilization ratio, it is necessary to explicitly indicate the desired legacy CG configuration. But it is afraid that the overhead in activation and deactivation is not appreciated and the complexity in implementation should also be taken into consideration since activation and deactivation may happen for every CG transmission occasion in the worst case.

Observation 2: Multiple legacy CG configurations may occupy too many HARQ processes and cause extra signaling overhead for activation and deactivation.

Proposal 2: RAN2 is kindly asked to agree that new CG enhancements should cause limited overhead and not waste too much radio resource if possible.

2.3 Assitance information on CG

It is in RAN2’s discussion that how to enhance BSR for capacity improvements. Since BSR is a MAC layer signaling and it can provide information directly to the MAC scheduler, BSR can be enhanced to provide assistance information for CG optimization and/or modification, such as the jitter of packet arrival and PDU size.

Proposal 3: RAN2 is kindly asked to discuss that whether BSR can be enhanced to provide assistance information for CG.

3	Summary

This contribution discusses potential enhancements on CG for XR, the following are the observations and proposals that may be taken into consideration.

Enhancement aspect:

Observation 1: CG benefits user experience in XR service due to the certainty of pre-configured radio resource and less delay compared with DG.

Proposal 1: RAN2 is kindly asked to confirm that legacy CG needs to be enhanced for the consistency between CG period and frame arrival interval, the adoption of actual data volume of XR traffic, and jitter handling.

Observation 2: Multiple legacy CG configurations may occupy too many HARQ processes and cause extra signaling overhead for activation and deactivation.

Proposal 2: RAN2 is kindly asked to agree that new CG enhancements should cause limited overhead and not waste too much radio resource if possible.

Assistance information aspect:

Proposal 3: RAN2 is kindly asked to discuss that whether BSR can be enhanced to provide assistance information for CG.

References

- TR 38.838, V17.0.0, Technical Specification Group Radio Access Network; Study on XR (Extended Reality) Evaluations for NR (Release 17), 3GPP