# 3GPP TSG RAN WG2 Meeting #120      	                           R2-2212909
Toulouse, November 14th – 18th, 2022

Agenda item:	8.6.3

Source:	Turkcell

Title:	Discussion on Mobility Enhancements of IoT NTN

WID/SID:	IoT_NTN_Enh – Release 18

Document for:	Discussion and Decision

# 1 Introduction

WI of IoT NTN Enhancement was revised in RAN #96 [1]. The related WID objectives involving RAN2 are summarized below.

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

Objectives:

The work item aims to specify further enhancements for E-UTRA (LTE-RAN) based NTN (non-terrestrial networks) according to the following assumptions:

-	GEO and NGSO (LEO and MEO).

-	Earth fixed Tracking area. Earth fixed & Earth moving cells for NGSO

-	FDD mode

-	UEs with GNSS capabilities

The detailed objectives are to specify enhanced NB-IoT NTN and eMTC NTN radio interfaces and E-UTRAN/NG-RAN as follows:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1]

•	NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

4.1.2 Mobility enhancements

The following mobility enhancements objectives are listed.

-	Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel 17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

-	Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2]

-	Define UE RRM core requirements for the above mobility enhancement features [RAN4].

4.1.3 Further enhancement to discontinuous coverage

-	Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2.  [RAN2, RAN3].

Agreements:

- For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events

Agreements:

- For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events

In RAN2 #119bis-e [2], the agreements related to AI 8.6.3 mobility enhancements aspects are below:

We share our views on conditional handover in this contribution.

# 2 Discussion

The serving cell is fixed for the static sensors/devices. For these devices, the neighbor cell measurement is unnecessary for Terrestrial Network (TN). But in Non-Terrestrial Networks (NTNs), these sensors need to measure their neighbors due to Non-Geostationary Satellite Orbit (NGSO).

The connected mode enhancements to CHO and measurement triggering in 3GPP Rel-17 NTN NR can be used for eMTC with minimum necessary changes, as the WID suggests. We support the minimum changes in UE behavior and 3GPP technical specifications. Location-based CHO was discussed in the previous meeting, RAN2 #119bis-e, for IoT NTN.

Observation 1: Both location-based and time-based CHO are essential for mobility in NTN. The signal difference between the cell edge and the middle is similar in NTNs. Due to power consumption concerns in IoT NTN, time-based CHO is more suitable for earth-fixed cells, and location-based CHO is more suitable for earth-moving cells.

Proposal 1: Both location-based and time-based trigger conditions should be simultaneously configured. Due to power limitations in IoT, it can be used only one option at the same time.

The handover mechanism requires a large amount of signaling interaction among user equipment, source gNodeB, and target gNodeB. The CHO approach can trigger signaling overhead and signaling storm during the execution part.

Observation 2: Signaling load can be a problem for mobile network operators.

Proposal 2: Both location-based and time-based trigger conditions should be decided based on the signaling load generation.

# 3 Conclusion

In this contribution, we share our views on mobility enhancements for IoT NTN. Our observations and proposals are shared below:

Observation 1: Both location-based and time-based CHO are essential for mobility in NTN. The signal difference between the cell edge and the middle is similar in NTNs. Due to power consumption concerns in IoT NTN, time-based CHO is more suitable for earth-fixed cells, and location-based CHO is more suitable for earth-moving cells.

Observation 2: Signaling load can be a problem for mobile network operators.

Based on observations, our proposals are:

Proposal 1: Both location-based and time-based trigger conditions should be simultaneously configured. Due to power limitations in IoT, it can be used only one option at the same time.

Proposal 2: Both location-based and time-based trigger conditions should be decided based on the signaling load generation.

# 4 References

[1] RP-221806 Revised WID on IoT NTN enhancements.

[2] RAN2 #119bis-e, Chair’s Notes, RAN2-119bis-e - NR-NTN-IoT-NTN_EOM.