3GPP TSG-RAN WG2 Meeting #121	R2-2300257

27 Feburary – 3 March, 2023, Athens, GR

Agenda item:	8.16.2

Source:	Dell Technologies

Title:	On AI/ML model transfer over radio interface

Document for:	Discussion & Decision

# Introduction

The study item on ‘’AI/ML for NR air radio interface’’ has been agreed during RAN#94-e for Release-18 [1]. Three basic use cases are currently under RAN1 study for AI/ML support as follows: (1) enhanced channel state information acquisition, (2) beam management, and (3) positioning, respectively. In the SID description, the RAN2 study scope as follows:

- Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case

During the last RAN2 meeting (RAN2#120), email discussions are established for discussing the pros and cons of AI/ML model delivery/transfer over CP and UP planes, respectively.

For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions

Therefore, this contribution presents our views on the potential AI/ML model delivery/transfer issues, when exchanged either over CP or UP.

# Discussion

AI/ML model transfer implies the handling of AI/Ml model delivery over the radio interface. Based on the outcomes of RAN1#110-bis, two categories of AI/ML model transfer are defined as follows:

- Scheme 1: AI/ML model transfer transparent to 3GPP (i.e., level y)

- Scheme 2: AI/ML model transfer on 3GPP signaling (i.e., level z)

Scheme 2 denotes that AI/ML model payload is stored on servers, as OTTs or OEMs, which is outside 3GPP scope. Scheme 2 means that AI/ML model payload is stored at an arbitrary 3GPP entity (e.g., gNBs and/or various core network functions) and are delivered to devices based on 3GPP control and/or data signaling procedures, respectively.

Observation 1: Level Z of AI/ML model transfer based on 3GPP signalling is relevant to RAN2 scope, where AI/ML model payload is delivered based on 3GPP signalling. This calls for 3GPP CP or UP signalling and protocol enhancements to adapt to emerging AI/ML model delivery requirements.

# AI/ML model transfer over CP

Transferring the AI/ML model payload over the control plane (CP) allows the AI/ML model payload to be part of each UE context information, which offers multiple AI/ML model delivery benefits to each UE needs and AI/ML capabilities. Specifically, with this delivery option, the AI/ML model is transferred as part of the radio resource control (RRC) reconfiguration message. It has the following pros and cons:

Pros:

- It offers lesser AI/ML transfer latency compared to delivering AI/ML models over UP

- AI/ML models are considered as part of UE context, which can be adaptively tuned to each UE AI/ML capability.

- It requires much less RAN2 spec impact, which is only limited to RRC.

Cons (limitations):

- It only supports small sized AI/ML models; otherwise, the number of RRC message segmentations is significantly expanded.

- It may potentially block other non-AI/ML mode RRC information if a larger AI/ML model is exchanged as part of existing RRCreconfiguration message, especially in handover and poor RF conditions scenarios.

- It does not support mobility forwarding. Thus, in case of handover, the AI/ML model RRC segments will be lost, and the whole RRCreconfig message should be transmitted again.

- RRC spec changes are strictly slow and fundamental. However, emerging AI/ML studies and future specifications are expected to be rapidly paced, thus, this may require frequent RRC spec changes which is not desirable.

Observation 2: Transferring the AI/ML models over the CP has the following pros:

  - It offers lesser AI/ML transfer latency compared to delivering AI/ML models over UP

  - AI/ML models are considered as part of UE context, which can be adaptively tuned to each UE AI/ML capability.

  - It requires much less RAN2 spec impact, which is only limited to RRC.

Observation 3: Transferring the AI/ML models over the CP has the following cons:

  - It only supports small sized AI/ML models; otherwise, the number of RRC message segmentations is significantly expanded.

  - It may potentially block other non-AI/ML mode RRC information if a larger AI/ML model is exchanged as part of existing RRCreconfiguration message, specially in handover and poor RF conditions scenarios.

  - It does not support mobility forwarding. Thus, in case of handover, the AI/ML model RRC segments will be lost, and the whole RRCreconfig message should be transmitted again.

Therefore, to avoid the mentioned RRC issues and to support efficient AI/ML model as part of the RRC signalling, we propose introducing a new RRC message signalling, which is specifically defined for carrying the AI/ML model payload. The new RRC message is treated in a different way than basic RRC signalling as:

- The new AI/ML model RRC signalling is separate from existing non-AI/ML RRC signalling such that it does not block critical RRC information.

- The new AI/ML model RRC signalling can adopt different RRC segmentation size thresholds and numbers compared to existing RRC signalling. This allows the RRC segment size to be match the AI/ML model sizes which can significantly vary depending on the AI/ML driven use case and model complexity.

- The new AI/ML model RRC signalling should support mobility forwarding, unlike existing RRC basic signalling. Thus, in case of handover, the RRC segments, carrying the AI/ML model payload, are exchanged among source and target cells, and hence, the remaining segments are only transmitted. This is beneficial specially for large AI/ML models by avoiding transmitting the whole RRC segment set carrying large AI/ML models.

Proposal 1: A new RRC message signalling is defined, independently from existing RRCreconfig message, for carrying AI/ML model information as UE context information.

Proposal 2: The new RRC signalling message, for carrying AI/ML model information, to support mobility forwarding, and different RRC segmentation compared to existing RRC messages, which matches the RRC model sizes.

# AI/ML model transfer over UP

Transferring the AI/ML information on the user plane (UP) offers more flexibility for supporting various AI/ML model formats and sizes, by transmitting those over a DRB instead of an SRB. However, this leads to two cases:

- Non-collocated gNB-UPF: if the AI/ML model data is available at the gNB and is to be transmitted on a UPF-terminated DRB, the AI/Ml model payload must be exchanged back and forth between gNB and UPF before being transferred over the radio interface, which imposes additional interface and processing loading. Thus, a new gNB-terminated DRB is needed for delivering AI/ML model content. We believe this is outside of RAN2 scope and should be left to SA2.

- Collocated gNB-UPF: Here, no need for a new gNB-terminated DRB definition. However, this limits the flexibility of transferring AI/ML models over radio only to gNB-UPF collocated deployments.

Observation 4: Transferring the AI/ML models over the CP has the following pros

- Little to no RAN2 spec impact

- Suitable for any model sizes and formats.

Observation 5: Transferring the AI/ML models over the CP has the following cons

- Delivering the AI/ML model on a DRB is less reliable and robust than SRB.

- Without a new gNB-terminated DRB definition, this option is only suitable for deployments of collocated UPF-gNB.

Proposal 3: RAN2 not to conclude on the new gNB-terminated DRB for AI/ML model transfer over UP and leave it up to SA2.

Proposal 4: For a flexible support of AI/ML model transfer, RAN2 to support both AI/ML model transfer methods over UP and CP, respectively.

# Conclusions

This contribution has presented Dell’s views on AI/ML model transfer over UP and CP, respectively. The following observations and proposals are therefore made:

Observation 1: Level z of AI/ML model transfer based on 3GPP signalling is relevant to RAN2 scope, where AI/ML model payload is delivered based on 3GPP signalling. This calls for 3GPP CP or UP signalling and protocol enhancements to adapt to emerging AI/ML model delivery requirements.

Observation 2: Transferring the AI/ML models over the CP has the following pros:

  - It offers lesser AI/ML transfer latency compared to delivering AI/ML models over UP

  - AI/ML models are considered as part of UE context, which can be adaptively tuned to each UE AI/ML capability.

  - It requires much less RAN2 spec impact, which is only limited to RRC.

Observation 3: Transferring the AI/ML models over the CP has the following cons:

  - It only supports small sized AI/ML models; otherwise, the number of RRC message segmentations is significantly expanded.

  - It may potentially block other non-AI/ML mode RRC information if a larger AI/ML model is exchanged as part of existing RRCreconfiguration message, specially in handover and poor RF conditions scenarios.

  - It does not support mobility forwarding. Thus, in case of handover, the AI/ML model RRC segments will be lost, and the whole RRCreconfig message should be transmitted again.

Proposal 1: A new RRC message signalling is defined, independently from existing RRCreconfig message, for carrying AI/ML model information as UE context information.

Proposal 2: The new RRC signalling message, for carrying AI/ML model information, to support mobility forwarding, and different RRC segmentation compared to existing RRC messages, which matches the RRC model sizes.

Observation 4: Transferring the AI/ML models over the CP has the following pros

- Little to no RAN2 spec impact

- Suitable for any model sizes and formats.

Observation 5: Transferring the AI/ML models over the CP has the following cons

- Delivering the AI/ML model on a DRB is less reliable and robust than SRB.

- Without a new gNB-terminated DRB definition, this option is only suitable for deployments of collocated UPF-gNB.

Proposal 3: RAN2 not to conclude on the new gNB-terminated DRB for AI/ML model transfer over UP and leave it up to SA2.

Proposal 4: For a flexible support of AI/ML model transfer, RAN2 to support both AI/ML model transfer methods over UP and CP, respectively.

# References

[1] RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface