3GPP TSG RAN WG2 Meeting #121               	R2-2301104

Athens, Greece, February 27th – March 3rd 2023

Agenda Item: 	6.1.3.1

Source:		Sony

Title:			Inclusion of SIB17 in Dedicated SIB request

Document for:	Discussion & Decision

# Introduction

In the Rel-17, the following agreements were made at RAN2#115 meeting, related to providing TRS/CSI-RS information in SIB17, but the On-demand SI request for SIB17 is missing in Dedicated SIB Request message. (RP-212307_Status report for WI_UE Power Saving Enhancements for NR) [1].

- RAN2 agreements

The TRS/CSI-RS configuration is provided in a new SIB. On demand SI should be possible for the SIB with TRS/CSI-RS information.

In this contribution propose to include SIB-17 in Dedicated SIB Request. See proposed CR in appendix.

# Motivation for SIB17 inclusion

In the Rel-17, the following agreements were made at RAN2#115 meeting, related to providing TRS/CSI-RS information in SIB17. (RP-212307_Status report for WI_UE Power Saving Enhancements for NR).

- RAN2 agreements

The TRS/CSI-RS configuration is provided in a new SIB. On demand SI should be possible for the SIB with TRS/CSI-RS information.

A CR 2924 to 38.331 in R2-2204265) [2] was agreed at RAN2#117 “Introduction of ePowSav in TS 38.331”, but the On-demand SI request for SIB17 is missing in Dedicated SIB Request message.

In order for the UE to also be able to acquire SIB17, this should be added as an option in the System information acquisition chapter 5.2.2, which is in line with agreements made for Rel-17 UE Power save as mentioned above.

Proposal 1: Approve the inclusion of SIB17 in Dedicated SIB Request message as per attached CR proposal.

# Summary

In this contribution, we have proposed the inclusion of SIB17 in Dedicated SIB Request

Proposal 1: Approve the inclusion of SIB17 in Dedicated SIB Request message as per attached CR proposal

# References

- (RP-212307_Status report for WI_UE Power Saving Enhancements for NR

- CR 2924 to 38.331 in R2-2204265

# Appendix

3GPP TSG-RAN WG2 Meeting #121	R2-23xxxxx

Athens, Greece, February 27th – March 3rd 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | Xxxx | rev | x | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SIB-17 On-demand request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding SIB 17 to dedicated SIB request |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the Rel-17, the following agreements were made at RAN2#115 meeting, related to providing TRS/CSI-RS information in SIB17, but the On-demand SI request for SIB17 is missing in Dedicated SIB Request message. (RP-212307_Status report for WI_UE Power Saving Enhancements for NR). - RAN2 agreements. - The TRS/CSI-RS configuration is provided in a new SIB. - On demand SI should be possible for the SIB with TRS/CSI-RS information.. Impact Analysis: Impacted functionality: On-demand System information related to SIB17 and TRS/CSI-RS configuration request in RRC Connected mode. Interoperability issue: - . |
| Consequences if not approved: |  | Not possible to acquire SIB 17 via Dedicated SIB request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 6.2.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR… |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.2.2	System information acquisition

#### 5.2.2.1	General UE requirements

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE receives NSAG information for cell reselection from upper layer), SIB17 (if the UE is capable of using TRS resources in Idle mode) SIB19 (if UE is accessing NR via NTN access).

The UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20 and SIB21, regardless of the RRC state the UE is in.

The UE shall ensure having a valid version of the posSIB requested by upper layers.

2nd Changes

### 6.2.2	Message definitions

<<skipped>>

#### DedicatedSIBRequest

The DedicatedSIBRequest message is used to request SIB(s) required by the UE in RRC_CONNECTED as specified in clause 5.2.2.3.5.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

DedicatedSIBRequest message

-- ASN1START

-- TAG-DEDICATEDSIBREQUEST-START

DedicatedSIBRequest-r16 ::=      SEQUENCE {

criticalExtensions               CHOICE {

dedicatedSIBRequest-r16          DedicatedSIBRequest-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

DedicatedSIBRequest-r16-IEs ::=  SEQUENCE {

onDemandSIB-RequestList-r16       SEQUENCE {

requestedSIB-List-r16            SEQUENCE (SIZE (1..maxOnDemandSIB-r16)) OF SIB-ReqInfo-r16                OPTIONAL,

requestedPosSIB-List-r16         SEQUENCE (SIZE (1..maxOnDemandPosSIB-r16)) OF PosSIB-ReqInfo-r16          OPTIONAL

} OPTIONAL,

lateNonCriticalExtension         onDemandSIB-RequestList-r17            OPTIONAL,

……..lateNonCriticalExtension		OCTET STRING

nonCriticalExtension             SEQUENCE {}              OPTIONAL

}

onDemandSIB-RequestList-r17       SEQUENCE {

requestedSIB-List-r17            SEQUENCE (SIZE (1..maxOnDemandSIB-r17)) OF SIB-ReqInfo-r17                OPTIONAL

}

SIB-ReqInfo-r16 ::=                   ENUMERATED { sib12, sib13, sib14, sib20-v1700, sib21-v1700, spare3, spare2, spare1 }

SIB-ReqInfo-r17 ::=                   ENUMERATED { sib17-v17, spare3, spare2, spare1 }

PosSIB-ReqInfo-r16 ::=       SEQUENCE {

gnss-id-r16                  GNSS-ID-r16                  OPTIONAL,

sbas-id-r16                  SBAS-ID-r16                  OPTIONAL,

posSibType-r16               ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6,

posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4,

posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10,

posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15,

posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20,

posSibType2-21, posSibType2-22, posSibType2-23, posSibType3-1, posSibType4-1,

posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3,..., posSibType1-9-v1710,

posSibType1-10-v1710, posSibType2-24-v1710, posSibType2-25-v1710,

posSibType6-4-v1710, posSibType6-5-v1710, posSibType6-6-v1710 }

}

-- TAG-DEDICATEDSIBREQUEST-STOP

-- ASN1STOP

DedicatedSIBRequest field descriptions

requestedSIB-List Contains a list of SIB(s) the UE requests while in RRC_CONNECTED.

requestedPosSIB-List Contains a list of posSIB(s) the UE requests while in RRC_CONNECTED.

PosSIB-ReqInfo field descriptions

gnss-id The presence of this field indicates that the request positioning SIB type is for a specific GNSS. Indicates a specific GNSS (see also TS 37.355 [49])

sbas-id The presence of this field indicates that the request positioning SIB type is for a specific SBAS. Indicates a specific SBAS (see also TS 37.355 [49]).

End of Changes