3GPP TSG-RAN WG2 Meeting #121	R2-2301358

Athens, Greece, 27 February - 3 March 2023

Agenda Item:	8.4.2.1

Source:	Huawei, HiSilicon

Title:	LTM procedure descriptions and stage 2 aspects

Document for: Discussion

# Introduction

In the last RAN2#120 meeting, RAN2 made several agreements on LTM procedure and most of them have already been captured in the 38.300 running CR endorsed in the last meeting. The overall procedure of LTM is divided into 4 phases, including LTM preparation phase, early synchronization phase, LTM execution phase and LTM completion phase. To reduce the interruption time of LTM procedure, enhancements could be made in every phase of LTM procedure. This contribution discusses the remaining issues in the above phases which are related to the overall procedure of LTM or related to other work groups.

# Discussion

## LTM preparation phase

In the LTM preparation phase, the network should provide candidate cells and the corresponding configurations of the candidate cells to the UE, to allow fast application of configurations upon LTM execution. It is implied in the latest 38.300 running CR[1] that the gNB decides to use LTM and initiates LTM candidate preparation after receiving L3 measurement results (i.e. MeasurementReport). Whether L1 measurement is involved in the determination of candidate cells is still not quite clear to RAN1 and RAN3 while this greatly impacts their subsequent discussions. For example, if L1 measurement is considered, RAN1 and RAN3 may need to design how to provide L1 measurement configuration of neighbor cells before the LTM decision and how to select candidate cells using L1 measurement results.

Since the determination of the candidate cells is a cell-level selection, only taking the L3 measurement reports into consideration is enough. When receiving the L3 measurements, the gNB-CU can decide the potential candidate cells directly. The gNB-CU sends the requests to the corresponding gNB-DU(s) to require the configurations of candidate cells, and then sends the pre-configuration to the UE by RRCReconfiguration message.

Proposal 1: Confirm that the pre-configuration of candidate configuration is initiated based on L3 measurements, rather than on L1 measurements (i.e. initiated only by gNB-CU, but not by gNB-DU).

## Early synchronization phase

RAN2 has agreed that both RACH-based and RACH-less procedure for LTM may be supported and RACH-less can be used if the UE doesn’t need to acquire TA during the cell switch. RAN1 has discussed several options for the acquisition of TA and one agreed solution is to trigger the UE to perform RACH towards a candidate PSCell using PDCCH-order procedure so that the network can determine the required TA adjustment and provide it to the UE at some point. Therefore, the UE can obtain the TA before receiving the LTM cell switch command and then skip the RACH procedure during the LTM execution to further reduce the interruption time.

The UL/DL synchronization is very tied to the RAN1 discussion and the details of the above-mentioned solution and any other solution are still under the RAN1’s discussion. RAN2 can wait for the progress of RAN1 before working on the details of early synchronization phase.

## LTM execution phase

In the last meeting, RAN2 have made the following agreement:

- RAN2 assumes that LTM (intra DU and inter DU) is network-controlled mobility where the control is from the source, i.e. measurements (L1 measurements) are configured in the UE from the source Cell, and the decision to switch cell is by the source cell, and enhancements considered for LTM before cell switch, e.g. pre-synchronization, TA handling, target beam mgmt (to the extent it is supported) may be by the source cell. RAN2 understands that this may require cooperation source DU CU target DU and/or OAM coord. RAN2 don’t see any blocking issue to share information between DUs but the support of this is in RAN3 domain. RAN2 see no necessity for a direct inter-DU-interface to support this.

As mentioned in the highlighted text, cooperation between the gNB-CU, the serving gNB-DU and the candidate gNB-DU (for inter-DU LTM case) is necessary during the LTM procedure. From RAN2 perspective, latency due to inter-node coordination just before LTM execution should be avoided, especially as the radio channel quality between the source cell and the UE might be poor at that time.

For example, for the inter-DU case, if the source gNB-DU would need to send a request for cell switch to the candidate gNB-DU, via the gNB-CU, and wait for the confirmation by the candidate gNB-DU, this would increase the latency (e.g. tens of milliseconds) and degrade the performance of the LTM procedure. Therefore, it is better to have any such coordination in the early LTM preparation phase only. In general, we should avoid “late” or “timely” CU-DU interactions upon triggering the LTM procedure.

Observation 1: Any checking/confirmation with target gNB-DU about the candidate cell can be done in the LTM preparation phase, rather than in the later cell switch decision phase.

Proposal 2a: In the cell switch decision phase, avoid CU-DU interactions to determine the content at the time of sending the cell switch command.

Proposal 2b: The LTM cell switch decision (i.e. when to switch and which cell to switch to) is made by the gNB-DU in the intra-DU case and by the source gNB-DU in the inter-DU case.

The need for coordination between source gNB -DU and candidate (target) gNB-DU depends on the contents of the LTM cell switch command. For example, for RACH-less LTM procedure, RAN1 has agreed to acquire TA based on the PDCCH ordered RACH before triggering LTM execution and one option is to include the TA value in the cell switch command MAC CE towards the UE. In this case, the source gNB-DU may need to get the TA value from the target gNB-DU before triggering the LTM. However, this is still under the discussion in RAN1. Moreover, other potential contents of cell switch command MAC CE may also have impact on the F1AP coordination, so we can make the final decision after we determine the full contents of cell switch command.

After the LTM decision, the serving gNB-DU sends the cell switch command MAC CE to the UE, including the candidate configuration index of the target cell. The source cell will receive the HARQ ACK so there is no need to introduce any new uplink signalling towards the source cell.

Proposal 3: The HARQ ACK is sufficient as the response to the LTM cell switch MAC CE to the source cell (i.e. no need of a "goodbye MAC CE" to the source cell).

## LTM completion phase

In the LTM completion phase, the UE indicates successful completion of the LTM cell switch towards the target cell. There is an editor’s note in the 38.300 running CR whether an uplink signal or message after the UE has switched to the target cell is used to indicate successful completion of LTM cell switch.

Just like legacy handover procedure, if the UE is configured to perform CBRA to the target cell, the target gNB-DU can identify the UE by the C-RNTI MAC CE in the Msg3 or MsgA. If CFRA is used, the gNB-DU can know the UE by the dedicated RACH resource in the Msg1 or MsgA. Then the target gNB-DU can send Msg4 or MsgB. There is no need to introduce new signaling to indicate successful completion of LTM cell switch to the target gNB-DU for the RACH-based LTM procedure.

Proposal 4a: In RACH based LTM, the target cell is aware that the UE has accessed the target cell based on the reception of Msg1 in CFRA and on the reception of Msg3 in CBRA, like for legacy HO.

When the UE is configured to perform RACH-less LTM procedure, the UE should send an uplink signaling to the target gNB-DU to indicate the access to the target cell. The gNB-DU can identify the UE by this uplink signaling, and then realize the successful completion of LTM procedure. The exact contents of the uplink signaling are related to other issues, for example, inter-DU or intra-DU LTM case and we can have further discussion.

Proposal 4b: In RACH-less LTM, the target cell is aware that the UE has accessed the target cell based on receiving the first UL MAC PDU on the dedicated UL grant provided by target cell.

In the Rel-15 handover procedure, the UE sends the RRCReconfigurationComplete message to complete the RRC handover procedure. The RRCReconfigurationComplete message can also carry information that help the gNB schedule the UE in an optimal way, such as UL Tx direct current. Such information can be useful for any kind of handover, including for LTM. Therefore, it should also be possible for the network to obtain such information at LTM execution. A simple solution would be to send an RRCReconfigurationComplete message according to legacy procedures.

Proposal 5: At LTM execution, the UE can send the RRCReconfigurationComplete message to the target cell with contents sent according to existing procedures (e.g. possibly to include the uplinkTXDirectCurrent, needForGaps, etc)

## Stage-2 TP suggestions

Based on the latest RAN2 progress on LTM, we suggest a number of modifications of the TS 38.300 TP for the stage2 procedure, as in the annex at the end of this document.

According to the figure, L1 measurement reports will only be provided after the UL/DL synchronization towards LTM target cells. However, as the UE is configured with L1 measurements, these reports can also come earlier.

Proposal 6a: Clarify that step 5 (L1 measurement reports) can be earlier than step 4 (DL/UL sync).

For L1 measurement, the TP indicates that the UE performs L1 measurements "on configured LTM candidate target cell(s)". However, this may not always make sense. For example, the network may configure as LTM candidate target cells cell 1 and cell 2, which are in opposite directions as compared with the current UE location, if the UE moves to cell 1, cell 2 will not be a neighbour cell so the configuration of LTM candidate cell 1 will probably not include L1 RS of candidate cell 2.

Therefore, we suggest adding "configured with L1 measurement RS" to the description.

Proposal 6b: Clarify in step 5 that "The UE performs L1 measurements on the LTM candidate target cell(s) configured with L1 measurement RS".

The TP captured that "Subsequent LTM between candidates (i.e., UE does not release other candidate cell configurations after LTM is triggered) can be performed without RRC reconfiguration" but that aspect is not visible in the sequence chart. In our understanding, this means that the steps 4 to 8 can occur multiple times, without reproducing the previous steps.

Proposal 6c: Capture as a further step or box after step 8, step 9 that "The sequence of steps 4 to 8 can be repeated as long as the LTM configuration is not released".

Proposal 6d: Consider the changes to TS 38.300 TP in the annex to update the LTM procedure.

## LTM failure

In the Rel-17 CHO, when initial CHO execution attempt fails or RLF occurs in the source cell, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after failure, then the UE attempts CHO execution once, otherwise re-establishment is performed. For the Rel-18 LTM procedure, as the UE always maintains the configuration of the candidate cells for LTM, when RLF in the source occurs or the initial LTM procedure fails, the UE performs cell selection and if the selected cell is a LTM candidate cell, the UE can perform the LTM procedure according to the stored candidate configuration of the selected cell. This may require a random access procedure but compared to performing the RRC re-establishment directly, the interruption time can be further decreased. Whether to introduce a configurable indication to enable/disable the function like attemptCondReconfig for CHO can be further discussed.

Proposal 7: At RLF or LTM execution failure, if the UE performs cell selection and the selected cell is an LTM candidate cell, the UE can perform LTM execution to the selected cell, (like CHO).

## Coexistence with L3 handover

As mentioned in the WID, the Rel-18 LTM procedure is applicable to the intra-DU case and to the intra-CU inter-DU case. The inter-CU handover, which involves the security update, should be performed based on the RRC procedure. When the gNB has provided the candidate configuration of LTM to the UE, the gNB should also be allowed to trigger L3 handover if the quality of a cell (possibly a cell under another CU) is good enough. Whether the target cell of L3 handover can be the candidate cells for the LTM procedure or not can be further studied.

If the UE receives the L3 handover command before the triggering of LTM procedure, the UE should perform handover to the target cell indicated in the L3 handover command. After completing the L3 handover, the UE may maintain or release the LTM candidate configurations, e.g. as instructed by the network in the L3 handover command. This issue can also be discussed later after determining the coexistence scenario between L3 handover and LTM procedure.

Proposal 8: While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network.

# Conclusion

- This contribution discusses stage 2 aspects of LTM and makes the following proposals:

LTM preparation

Proposal 1: Confirm that the pre-configuration of candidate configuration is initiated based on L3 measurements, rather than on L1 measurements (i.e. initiated only by gNB-CU, but not by gNB-DU).

LTM execution

Proposal 2a: In the cell switch decision phase, avoid CU-DU interactions to determine the content at the time of sending the cell switch command.

Proposal 2b: The LTM cell switch decision (i.e. when to switch and which cell to switch to) is made by the gNB-DU in the intra-DU case and by the source gNB-DU in the inter-DU case.

Proposal 3: The HARQ ACK is sufficient as the response to the LTM cell switch MAC CE to the source cell (i.e. no need of a "goodbye MAC CE" to the source cell).

LTM completion

Proposal 4a: In RACH based LTM, the target cell is aware that the UE has accessed the target cell based on the reception of Msg1 in CFRA and on the reception of Msg3 in CBRA, like for legacy HO.

Proposal 4b: In RACH-less LTM, the target cell is aware that the UE has accessed the target cell based on receiving the first UL MAC PDU on the dedicated UL grant provided by target cell.

Proposal 5: At LTM execution, the UE can send the RRCReconfigurationComplete message to the target cell with contents sent according to existing procedures (e.g. possibly to include the uplinkTXDirectCurrent, needForGaps, etc).

Stage 2 TP suggestions

Proposal 6a: Clarify that step 5 (L1 measurement reports) can be earlier than step 4 (DL/UL sync).

Proposal 6b: Clarify in step 5 that "The UE performs L1 measurements on the LTM candidate target cell(s) configured with L1 measurement RS".

Proposal 6c: Capture as a further step or box after step 8, step 9 that "The sequence of steps 4 to 8 can be repeated as long as the LTM configuration is not released".

Proposal 6d: Consider the changes to TS 38.300 TP in the annex to update the LTM procedure.

LTM failure

Proposal 7: At RLF or LTM execution failure, if the UE performs cell selection and the selected cell is an LTM candidate cell, the UE can perform LTM execution to the selected cell, (like CHO).

Coexistence with L3 handover

Proposal 8: While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network.

# References

- R2-2213332 38.300 running CR for introduction of NR further mobility enhancements

# Annex: Text Proposal to TS 38.300

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BFD	Beam Failure Detection

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CG	Configured Grant

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

FSA ID	Frequency Selection Area Identity

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network selection

GSO	Geosynchronous Orbit

H-SFN	Hyper System Frame Number

HAPS	High Altitude Platform Station

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

LTM	L1/L2-triggered Mobility

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCL	Neighbour Cell List

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NGSO	Non-Geosynchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDC	Propagation Delay Compensation

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PH	Paging Hyperframe

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PRS	Positioning Reference Signal

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PTM	Point to Multipoint

PTP	Point to Point

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QMC	QoE Measurement Collection

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RLM	Radio Link Monitoring

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SFI-RNTI	Slot Format Indication RNTI

SHR	Successful Handover Report

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRAP	Sidelink Relay Adaptation Protocol

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SSSG	Search Space Set Group

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TB	Transport Block

TCE	Trace Collection Entity

TNL	Transport Network Layer

TPC	Transmit Power Control

TRP	Transmit/Receive Point

TRS	Tracking Reference Signal

U2N	UE-to-Network

UCI	Uplink Control Information

UDC	Uplink Data Compression

UE-Slice-MBR	UE Slice Maximum Bit Rate

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

VR	Virtual Reality

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

#### 9.2.3.x	L1/L2-triggered mobility

##### 9.2.3.x.1	General

LTM is a procedure in which a gNB receives L1 measurement reports from UEs, and on their basis the gNB changes UEs’ serving cell(s) through MAC CE. The gNB prepares one or multiple candidate cells and provides the candidate cell configurations to the UE through RRC message. Then LTM cell switch is triggered, by selecting one of the candidate configurations as target configuration for LTM by the gNB. The candidate cell configurations can only be added, modified and released by network via RRC signaling.

Editors’ note: FFS later whether some optimization should be applied e.g. for release.

Editor’s note: Current options to configure a LTM candidate cell:

a.	One RRCReconfiguration message for candidate target cell

b.	One CellGroupConfig IE for each candidate target cell

The following principles apply to LTM:

-	Candidate cell configuration can be provided as delta configurations on top of a reference configuration. The reference configuration is managed separately, and a UE stores the reference configuration as a separate configuration.

-  User plane is continued whenever possible (e.g. intra-DU), without reset, with the target to avoid data loss and the additional delay of data recovery.

-	Security is not updated in LTM.

-	 Subsequent LTM between candidates (i.e., UE does not release other candidate cell configurations after LTM is triggered) can be performed without RRC reconfiguration.

LTM supports both intra-gNB-DU and intra-gNB-CU inter-gNB-DU mobility. LTM also supports inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. The following scenarios are supported:

-	PCell change in non-CA scenario,

-	PCell change without SCell change in CA scenario,

-	PCell change with SCell change(s) in CA scenario, including the following cases:

a) The target PCell/target SCell(s) is not a current serving cell (CA-to-CA scenario with PCell change)

b) The target PCell is a current SCell

c) The target SCell is the current PCell.

-	Dual connectivity scenario, at least for the PSCell change without MN involvement case, i.e. intra-SN.

Inter-cell beam management is also supported, but is not considered as a prerequisite for using LTM.

Editor’s note: The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different.

Editor’s note: FFS whether ASN.1 decoding and validity/compliance check of candidate cell configuration are performed upon reception of the candidate cells configuration, and if this needs to be specified.

##### 9.2.3.x.2	C-plane handling

Cell switch trigger information is conveyed in a MAC CE, which contains at least a candidate configuration index. Cell-specific, radio bearer, and measurement configurations can be part of an LTM candidate cell configuration.

Editor’s note: FFS if the MAC CE can indicate TCI state(s) (or other beam info) to be activate for the target Cell(s)

Editor’s note: FFS if it should be possible to perform SCell activation/deactivation (amongst SCells associated with the candidate configuration) simultaneously with the LTM triggering MAC CE.

UE may perform CBRA or CFRA at cell switch. UE may also skip random access procedure if UE doesn’t need to acquire TA for the target cell during cell switch. RACH resources for CFRA are provided in RRC configuration.

Editor’s note: FFS if the CFRA resources can be provide via MAC CE.

The overall procedure for LTM is shown in Figure x below. Subsequent LTM is done by repeating the early synchronization, LTM execution, and LTM completion steps without releasing other candidates after each LTM completion.

Figure x. Signaling procedure for LTM

The procedure for LTM is as follows.

1.	The UE sends a MeasurementReport message to the gNB. The gNB decides to use LTM and initiates LTM candidate preparation.

2.	The gNB transmits an RRCReconfiguration message to the UE including the configuration of one or multiple LTM candidate target cells.

3.	The UE stores the configuration of LTM candidate target cell(s) and transmits a RRCReconfigurationComplete message to the gNB.

4.   The UE may perform DL synchronization and TA acquisition with candidate target cell(s) before receiving the LTM cell switch command.

Editor’s note: DL synchronization for candidate cell(s) before cell switch command is supported, at least based on SSB. FFS necessary mechanism.

Editor’s note: TA acquisition of candidate cell(s) before LTM cell switch command is supported, at least based on PDCCH ordered RACH, where the PDCCH order is only triggered by source cell. FFS detailed mechanism.

5.	The UE performs L1 measurements on the LTM candidate target cell(s) configured with L1 measurement RS, and transmits lower-layer measurement reports to the gNB. The order the UE performs L1 measurements or transmits lower-layer measurement reports to the gNB and performs DL/UL synchronization of candidate target cells is not defined.

Editor’s note: FFS whether the lower-layer measurement reports are carried on L1 or MAC.

6.	The gNB decides to execute LTM cell switch to a target cell, and transmits a MAC CE triggering LTM cell switch by including the candidate configuration index of the target cell. The UE switches to the configuration of the LTM candidate target cell.

Editor’s note: FFS how beam indication is done.

7.	The UE performs random access procedure towards the target cell, if TA is not available.

8.   The UE indicates successful completion of the LTM cell switch towards target cell.

9.   The UE can subsequently perform the step 4-8 until stopped by the gNB.

Editor’s note: FFS whether an uplink signal or message after the UE has switched to the target cell is used to indicate successful completion of the LTM cell switch.

##### 9.2.3.x.3	U-plane handling

In LTM, whether the UE performs partial or full MAC reset, re-establish RLC, performs data recovery with PDCP during cell switch is explicitly controlled by the network.

Editor’s note: ON the determination of whether to reset L2: two options on the table:

1)	  The UE determines whether the switch is intra DU or inter DU and the follows different rule or configuration for these two cases which controls whether to reset or not reset. Determination could be based on configuration (e.g. of a DU ID, cell group id etc)

2)	  The UE receives command to reset or not reset by MAC CE.

Editor’s note: FFS what partial reset is.

Editor’s note: For UE processing, the following (not exhaustive) is assumed to be performed after receiving the cell switch command:

- MAC/RLC reset (when configured)

- RF retuning (e.g. needed for inter-frequency), baseband retuning

Editor’s note: FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility.

##### 9.2.3.x.4	Data Forwarding