# 3GPP TSG-RAN WG2 Meeting #121							R2-2301454

# Athens, Greece, 27th February – 3rd March 2023

Agenda Item:	6.1.3.1

Source:	Ericsson

Title:	Dedicated and broadcast signalling of re-selection priorities

Document for:	Discussion

# 1	Introduction

In this document we discuss slice-based cell re-selection when the re-selection information is provided in dedicated signalling.

# 2	Discussion

## 2.1 Usage of slice-based re-selection information in dedicated signalling when SIB16 is not broadcast

At RAN2#120, RAN2 made an agreement on dedicated vs broadcasted provision of configuration parameters to UE of slice-based prioritization.

R2-2212316	Discussion on postponed issue for slice specific cell reselection 	Samsung R&D Institute India	discussion

Observation 1: A UE provided with slice based cell reselection priorities in RRCRelease uses the slice availability lists in SIB16.

Observation 2: A gNB sending RRC Release may not be aware of the cells other than its neighbor cells.

Observation 3: Normally, a UE can move to cells which are not the neighbors of the cell that send RRC Release while dedicated slice priority is valid.

Proposal 1: When the NSAG-Frequency pair configured in dedicated slice information is not available in the SIB16, consider the below options

a.	UE doesn’t use the NSAG-Frequency pair for deriving slice based cell reselection priority in this cell.

b.	UE derives the slice based cell reselection priority assuming all the cells in the frequency support the NSAG.

Proposal 2: RAN2 is requested to select option a) and adapt the given TP, when the NSAG-Frequency pair configured in dedicated slice information is not available in the SIB16.

-	Apple thinks that if SIB doesn’t have the frequency in dedicated signalling, UE does not need to consider that frequency. Thinks option a is the correct option. Nokia agrees on the lack of frequency, but thinks option B is about NSAG usage and can also be correct. CATT also agrees with option a. QC agrees with Nokia.

Proposal 1: When the NSAG-Frequency pair configured in dedicated slice information is not available in the SIB16, consider the below options

a.	UE doesn’t use the NSAG-Frequency pair for deriving slice based cell reselection priority in this cell.

b.	UE derives the slice based cell reselection priority assuming all the cells in the frequency support the NSAG.

RAN2 common understanding is that option A in P1 is the correct interpretation.

This agreement means that a UE cannot re-select a cell based on slice based cell re-selection priorities provided in dedicated signalling, unless SIB16 is broadcasted in the current cell.

Agreement from RAN2#120 makes it impossible to use slice based cell re-selection with NSAG re-selection information provided in RRCRelease without SIB16 being broadcast.

The main argument for this agreement where the lack of Xn signalling to populate sliceAllowedCellListNR/ sliceExcludedCellListNR lists for a large number of cells (Xn interfaces are typically only configured between a cell and its neighour cells, and not to more distant cells). These lists are used by the UE to decide whether a target ncell at cell re-selection supports the prioritized NSAG/slice. But a typical case could very well be  that the NSAG/slice is “supported” (at least from cell re-selection point of view) by all the cells on a frequency, and there is no need for the lists. It seems very strange to not have the option that slice-based cell re-selection can be configured with dedicated signalling only (i.e. without SIB16). This should be up to the network operator to decide.

Considering the above, there is no reason to introduce a restriction/requirement in TS 38.304 that NSAG-Frequency pair configured in dedicated slice information is also available in the SIB16. The agreement from RAN2#120 should be reverted.

RAN2 agrees that when an NSAG-Frequency pair configured in dedicated slice information, and related sliceAllowedCellListNR/sliceExcludedCellListNR are not provided in SIB16, UE assumes all the cells in the frequency support the NSAG.

One could argue that this UE behaviour is already the intended UE behaviour as captured in existing TS 38.304 text. But if the specification change discussed in 2.2 is agreed, this intended UE behaviour becomes more clearly captured in the specification.

## 2.2 Use of cell lists from SIB16 when nsag-CellReselectionPriority is provided in RRCRelease.

The following text in TS 38.304 is relevant in this discussion:

#### 5.2.4.1	Reselection priorities handling

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If any fields with cellReselectionPriority or nsag-CellReselectionPriority are provided in dedicated signalling, the UE shall ignore any fields with cellReselectionPriority and nsag-CellReselectionPriority provided in system information.

:

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

From the texts above, we understand that:

- UE only considers an NR frequency for re-selection if listed in SIB4, and there is a priority provided for that frequency (in SIB4 (cellReselectionPriority), in SIB16 (nsag-CellReselectionPriority) or in RRCRelease message (cellReselectionPriority  or nsag-CellReselectionPriority)

- In SIB4, a frequency can be listed without a priority. This frequency is considered by UE only if priority relevant for the UE (relevant NSAG priority in SIB16, relevant NSAG priority in RRCRelease or cellReselectionPriority in RRCRelease

- SIB16 may provide NSAG cell re-selection priorities for some or all the frequencies listed in SIB4

- nsag-CellReselectionPriority provided in RRCRelease overwrites cellReselectionPriority in SIB4 (if provided) and nsag-CellReselectionPriority in SIB16 (if provided).

RAN2 is asked to confirm 1-4 above.

RAN2 agreed that the sliceAllowedCellListNR/sliceExcludedCellListNR lists are signalled to UE only in system information (SIB16) and not in dedicated signalling (RRCRelease). This is reflected in the ASN.1 in TS 38.331. But the related UE behaviour that UE shall use the lists provided in SIB16 when NSAG cell re-selection priorities is included in RRCRelease is missing.

It is currently missing in TS 38.304 is that UE shall use the slice-availability cell lists in SIB16, if provided, when NSAG cell re-selection priorities are included in RRCRelease.

In [2], we provide a text proposal that resolves this. This CR also covers (or gives more clear specification text to) the intended UE behaviour discussed in section 2.1

RAN2 to agree on the CR to TS 38.304 in [2].

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Agreement from RAN2#120 makes it impossible to use slice based cell re-selection with NSAG re-selection information provided in RRCRelease without SIB16 being broadcast.

Observation 2	It is currently missing in TS 38.304 is that UE shall use the slice-availability cell lists in SIB16, if provided, when NSAG cell re-selection priorities are included in RRCRelease.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 agrees that when an NSAG-Frequency pair configured in dedicated slice information, and related sliceAllowedCellListNR/sliceExcludedCellListNR are not provided in SIB16, UE assumes all the cells in the frequency support the NSAG.

Proposal 2	RAN2 is asked to confirm 1-4 above.

Proposal 3	RAN2 to agree on the CR to TS 38.304 in [2].

# References

[1]	R2-2212316 	Discussion on postponed issue for slice specific cell reselection
 				Samsung R&D Institute India	discussion

[2]	R2-2301453		Correction to slice-support cell lists (CR to TS 38.304), Source: Ericsson