3GPP TSG-RAN WG2 Meeting #121	R2-230xxxx

Athens, Greece, 27 February – 03 March 2023

Title:	[DRAFT] LS on Comparison of SL-RSRP and SD-RSRP measurements

Response to:	-

Release:	Release 18

Work Item:	NR_SL_relay_enh

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG1, TSG RAN WG 4

Cc:

Contact Person:

#### Name:	György Wolfner

E-mail Address:	gyorgy<dot>Wolfner<at>@nokia<dot>com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

During the work on session continuity enhancements within NR_SL_relay_enh work item a new measurement report trigger has been proposed yet with no consensus in RAN2:

Event Z2: The measured RSRP of a candidate L2 U2N Relay UE becomes an offset better than measured RSRP of the serving L2 U2N Relay UE

This new event requires either  the comparison of SD-RSRP measurements  from different UEs, or the comparison of SL-RSRP with SD-RSRP measurements. During the discussion some companies have concerns on direct comparison of SD-RSRP and SL-RSRP due to the power control mechanism applied on PC5 unicast links, while other companies think the network can handle such comparison by using an appropriate offset in the measurement event.

Before agreeing to the introduction of this measurement report trigger, RAN2 would like to ask feedback from RAN1 and RAN4 on the comparisons of RSRP measurements:

Q: 	Is the comparison of SL-RSRP and SD-RSRP measurement for the purposes of triggering a measurement report feasible?

2. Actions:

To RAN WG1 and RAN WG 4

ACTION: 	RAN2 would like to ask feedback from RAN1 and RAN4 on the comparisons of RSRP measurements, more specifically to the question above.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#121-bis-e	from 2023-04-17	to 2023-04-26		Electronic

RAN2#122	from 2023-05-22	to 2023-05-26		Incheon