3GPP TSG-RAN WG2 Meeting #121-bis electronic  	                            R2-2303080

April 17-26, 2023										                                (Revision of R2-2301088)

Agenda Item:	7.3.5

Source: 	Sony

Title:	Handover enhancement for NES

Document for:	Discussion

# Introduction

In RAN2#121 meeting, connected mode mobility for NES was discussed and RAN2 reached the agreements as follows.

- Study whether CHO enhancements are needed for the purpose of turning off the cell - Continue discussing CHO in the context of different NES techniques.

In this contribution, we will focus on the technical details on the CHO enhancement for energy saving.

# Discussion

2.1 NES-aware CHO

In the study phase, RAN2 agreed that from RAN2 perspective, CHO enhancements are feasible. And during the switching of NES modes, the UEs should:

- Evaluate the handover conditions depending on the NES mode of source and/or target cell.

- Know when to trigger the CHO evaluations.

During the CHO evaluation, the NES mode of the target cell could be considered in order to avoid UEs selecting cells operating in NES mode, which may result in frequent handovers.

In the legacy CHO, after UE is configured with CHO conditions, it is up to UE to evaluate that those execution conditions are fulfilled or not and if one of the conditions is fulfilled, a handover will be executed. But for NES CHO, there are reasons that the tigger to tell the UE to start the evaluation should originate from network, as listed as follows.

- Network knows the NES mode of itself as well as its neighbour cells is going to change or has changed, and network has the knowledge that which UEs may be impacted by such NES mode change (according to its configurations).

- The NES mode, especially when the UE is configured with explicit NES CHO conditions, may have changed. As a result, UE needs to be notified of such change and may start performing corresponding CHO evaluations.

- It is the network’s decision to change its NES mode e.g., a source cell wants to offload its UEs to its neighbour cells or a cell is switched on to accept new UEs.

So, it should be the network to notify the UE to start performing CHO execution conditions due to the NES mode change. This notification could be e.g. notify the UE of the latest NES mode of related cells or configure the UE to start performing configured CHO conditions.

There was an email discussion [POST121][311][NES] on DTX/DRX, and some companies mentioned the dynamic DTX/DRX in order to adapt to the dynamic traffic characteristics, and such dynamic NES status change will impose requirement on the notification signalling in order to support efficient NES based CHO..

We have the proposals as follows.

Proposal 1: Network should notify the UE to start performing NES CHO execution conditions when the NES mode of source or candidate cells is going to change or has changed.

# Conclusion

We propose RAN2 to discuss the proposals as follows.

Proposal 1: Network should notify the UE to start performing NES CHO execution conditions when the NES mode of source or candidate cells is going to change or has changed.