3GPP TSG-RAN WG2 Meeting #121e-bis              		         R2-2304175

Online, 17 -26 April 2023

Agenda item:	7.16.2.3

Source:	LG Electronics Inc.

Title:	AIML method Model Transfer/Delivery

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we discuss the model transfer/delivery methods.

# 2. Discussion

## 2.1 Model transfer/delivery method

AI/ML model transfer/delivery methods are being discussed in RAN2, where RAN2 need to discuss which network entity should be responsible for transfer/deliver of the AI/ML model and how transfer/delivery is done. Two outstanding alternatives are CP-based and UP-based method.

We think it is important to consider AI/ML model configuration functionality. AI/ML model may have many versions of a model each using different programming languages, libraries, or different versions of the same library. If an AI/ML model is delivered through a CP-based solution, the network entity responsible for AI/ML model transfer/delivery may need to understand the AI/ML model language as well as the contents of the configuration. For example, if RRC is in charge of configuring a proper AI/ML model, it is required that the gNB have a complete understanding of the AI/ML model to use. Also, it requires the massive update of existing gNBs to support ML functionalities.

Observation 1. If RRC is in charge of configuring an AI/ML model, it is required that the gNB have a complete understanding of the AI/ML model to use. This also requires the massive update of existing gNBs to support ML functionalities.

Since AI/ML technologies evolve rapidly, model scalability and new model availability should be considered seriously. As new features are added to the AI/ML model, new model-specific settings and its description should be added. If RRC starts to specify AI/ML model description, RAN2 spec impact would be significant. For any model to use, model and model-related parameters shall be specified. This would introduce a strong but unnecessary dependency between RRC and ML syntax. Furthermore, RRC can hardly catch up with the latest AI/ML model pool that is rapidly evolving and expanding, because RRC spec change is quite slow and strictly controlled.

Observation 2. If RRC is in charge of configuring an AI/ML model, whenever new features are added to the AI/ML model, RAN2 spec impact is significant. RRC cannot catch up with the latest AI/ML model pool applicable in the field/ML community.

Therefore, it is proposed not to consider using ASN.1 for model transfer/delivery in RRC

Proposal 1. Do not consider using ASN.1 for model transfer/delivery in RRC

In addition, any CP solution (whether the network entity is CN or gNB) can be problematic in setting if the model size is large.

First, segmented delivery of the RRC message is needed to carry a large-size of AI/ML model. RRC messages can be segmented every 9000 bytes, and as the size of the model increases, more RRC messages are generated. For example, in the case of a 10Mbyte model, 1000 segments of RRC messages are required. If many segments are needed, there is a problem of extending the RRC buffer size. Currently, up to 45kbytes are supported, and DL RRC messages can be segmented up to 5 accordingly.

Secondly, when PDCP re-establishment is performed during mobility, for SRBs, all stored PDCP SDUs and PDCP PDUs are discarded. That is, if model transfer/delivery fails before/during mobility, the UE should receive the model again from the beginning. Therefore, many radio resources may be wasted and signaling overhead may occur.

Lastly, to avoid head-of-line-blocking of time-critical SRBs, SRB4 or a new SRB of lower priority should be used to send the RRC message carrying the AI/ML model.

Observation 3. CP-based solutions are not suitable for delivering models with large sizes.

- If many segments are needed, there is a problem of extending the RRC buffer size. Currently, up to 45kbytes are supported, and DL RRC messages can be segmented up to 5 accordingly.

- When PDCP re-establishment is performed during mobility, for SRBs, all stored PDCP SDUs and PDCP PDUs are discarded. If model transfer/delivery fails before/during mobility, the UE should receive the model again from the beginning.

- To avoid head-of-line-blocking of time-critical SRBs, SRB4 or a new SRB of lower priority should be used to send the RRC message carrying the AI/ML model.

On the other hand, for models with large sizes, UP-based solutions is more appropriate. From architectural point of view, several UP based solutions being discussed are not mutually exclusive ones but just implementation variants of a common UP-based architecture that employs a network entity such as application function in charge of AI/ML model provisioning. From RAN2 perspective, a new application protocol stack that terminates UE and a network entity may need to be introduced to provide model provision services in standardized manner.

<Figure 1. UP solution between gNB and UE>

<Figure 2. UP solution between CN and UE>

<Figure 3. UP solution between server and UE>

The UP based solution does not require AI/ML model-related settings to be specified in the specification. This makes using the model more flexible and extensible. In addition, since it is less restricted by model size/format, the available models may be more diverse. Therefore, we propose to study the UP solution for model transfer/delivery. Through this, we can also narrow the scope of the discussion.

Observation 4. UP based solution does not require AI/ML model-related settings to be specified in the specification. This makes using the model more flexible and extensible. In addition, since it is less restricted by model size/format, the available models may be more diverse.

Proposal 3. To study UP solution for model transfer/delivery

## 2.2 Delta Model transfer/delivery

In the RAN2 #121 meeting, there was an argument regarding delta model transfer/delivery. According to the agreements from RAN1 #112 meeting, there are different methods of model transfer/delivery for UE-sided model and two-sided model, depending on the model storage location and training location.

RAN1 #112 Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side.

Among the different cases, case z2, z4, and z5 involve model transfer/delivery through the 3GPP air interface. Excluding proprietary models and cases where the network is unaware of the UE's model structure, z4 remains as a possible scenario for delta model transfer/delivery. This is because in the case of z4, it is possible to modify certain parameters and deliver them to the UE, making it a feasible option for delta model transfer/delivery.

Observation 5. Only z4 case, i.e., model transfer in open format of a known model structure at UE, can be applicable for delta configuration.

However, delta model transfer/delivery should be considered in the same way as initial model transfer/delivery because it also has the same observations (Observation1-Observation 4) as initial model transfer/delivery. Therefore, the delta configuration can be transferred via the UP solution and there is nothing to specify in RAN2.

Proposal 4. Do not study delta model transfer/delivery in RAN2

# 3. Conclusion

Observation 1. If RRC is in charge of configuring an AI/ML model, it is required that the gNB have a complete understanding of the AI/ML model to use. This also requires the massive update of existing gNBs to support ML functionalities.

Observation 2. If RRC is in charge of configuring an AI/ML model, whenever new features are added to the AI/ML model, RAN2 spec impact is significant. RRC cannot catch up with the latest AI/ML model pool applicable in the field/ML community.

Proposal 1. Do not consider using ASN.1 for model transfer/delivery in RRC

Observation 3. CP-based solutions are not suitable for delivering models with large sizes.

- If many segments are needed, there is a problem of extending the RRC buffer size. Currently, up to 45kbytes are supported, and DL RRC messages can be segmented up to 5 accordingly.

- When PDCP re-establishment is performed during mobility, for SRBs, all stored PDCP SDUs and PDCP PDUs are discarded. If model transfer/delivery fails before/during mobility, the UE should receive the model again from the beginning.

- To avoid head-of-line-blocking of time-critical SRBs, SRB4 or a new SRB of lower priority should be used to send the RRC message carrying the AI/ML model.

Observation 4. UP based solution does not require AI/ML model-related settings to be specified in the specification. This makes using the model more flexible and extensible. In addition, since it is less restricted by model size/format, the available models may be more diverse.

Proposal 3. To study UP solution for model transfer/delivery

Observation 5. Only z4 case, i.e., model transfer in open format of a known model structure at UE, can be applicable for delta configuration.

Proposal 4. Do not study delta model transfer/delivery in RAN2

# 4. References

[1] Draft_R2_121_meeting_report_v1.docx