3GPP TSG-RAN WG2 Meeting #121-bis electronic	R2-2xxxxxx

Online, April 17-26, 2023

Agenda item:	8.10

Source: 	Session Chair (Ericsson)

Title:	Report from eRedCap breakout session

# Organizational

## At meeting email discussions:

[AT121bis-e][750] Organizational – eRedCap (Ericsson)

Scope:

    - Share plans for the meeting and list of ongoing email discussions

    - Share meetings notes and agreements for review and endorsement

Intended outcome:

    - General information sharing about the sessions

Deadline:

    - Deadline: EOM

[AT121bis-e][751] eDRX for RRC_INACTIVE (OPPO)

Scope:

    - Summarize and identify agreeable proposals for agenda item 7.19.2

Intended outcome:

    - Report with agreeable proposals in R2-2304361

Deadline:

    - Deadline for comments: Wednesday 23:59 UTC

    - Rapporteur proposals: Thursday 10:00 UTC

    - Document deadline: 1h before session

[AT121bis-e][752] Further reduced UE complexity in FR1 (Huawei)

Scope:

    - Summarize and identify agreeable proposals for agenda item 7.19.3

Intended outcome:

    - Report with agreeable proposals in R2-2304362

Deadline:

    - Deadline for comments: Wednesday 23:59 UTC

    - Rapporteur proposals: Thursday 10:00 UTC

    - Document deadline: 1h before session

## Schedule:

Dates and deadlines (see also RP-230050)

March 31st 	Deadline for Long email discussions into R2 121.

April 3rd – 7th	Inactive period, no email discussions.

April 7th 1000 UTC	Tdoc Submission Deadline.

April 17th 0700 UTC	e-Meeting Start (by email), Week 1
Rapporteurs in non-favourable time zones may kick off AT meeting offline / email discussions before meeting start (at most 12h before). It is assumed that participants starts paying attention to offline / email discussions after e-meeting start.

April 21st 1000 UTC 	Weekend break, Suspend decision making in email discussions (= no deadlines etc). It should be possible for a delegate to take the weekend off, rejoin and not miss decisions.

April 24th 1000 UTC 	Resume after weekend. Resume decision making in email discussions, Week 2.

April 26th 1000 UTC	e-Meeting Stop, no more technical comments for AT-meeting email discussions. Decision confirmations announced within 24h. Session notes for email checking.

April 28th 1000 UTC	Deadline Short Email Discussions (limited possibility - for very short email discussions, if needed short email discussion can be started before e-meeting Stop). E.g. for LS outs, or other priority topics e.g. conclusion of R17 CRs.

May 1st – 5th		Inactive period, no email discussions.

May 12th 1000 UTC	Tdoc submission deadline RAN2 122 (next meeting).

Very limited possibility for long email discussions.

Web Conference Schedule

Note that this schedule is indicative and can change. After Week 1 the schedule for Week 2 will be updated.

WEEK 1:

| Time Zone UTC | Web Conference R2 - Main | Web Conference R2 - BO1 | Web Conference R2 - BO2 | Offline GTW Session (limited use, only specific issues if needed, need approval by session chair) |
| --- | --- | --- | --- | --- |
| Monday |  |  |  |  |
| 12:30-13:30 | NR18 Mobility Enh [2] (Johan) | NR18 XR [2] (Tero) | NR18 SL Relay [1.5] (Nathan) |  |
| 13:30-14:30 | NR18 Mobile IAB [0.5] (Johan) | NR18 UAV [1] (Diana) | NR18 Pos [2] (Nathan) |  |
| 14:30-15:30 | NR18 AIML [1] (Johan) | NR18 NCR [0.5] (Sasha) | Maintenance Early items (Nathan Kyeongin/Qianxi) |  |
| Tuesday |  |  |  |  |
| 12:30-13:30 | NR18 LP WUS [0.5] (Johan) | NR18 NTN enh [1] (Sergio) | NR18 SL evolution [1] (Kyeongin/Qianxi) |  |
| 13:30-14:30 | NR18 Other [2] (Johan) | NR18 NTN enh [1] (Sergio) | NR18 SL evolution [1] (Kyeongin/Qianxi) |  |
| 14:30-15:30 | NR18 Mobility Enh [2] (Johan) | Maintenance Early Items (Sergio, Tero) | NR18 SL evolution [1] (Kyeongin/Qianxi) |  |
| Wednesday |  |  |  |  |
| 12:30-13:30 | NR18 AIML [1] (Johan) | NR18 QoE [1] (Tero) | NR18 SL Relay [1.5] (Nathan) |  |
| 13:30-14:30 | – TBD (Johan) | NR18 Network Energy Saving [1] Early items (Diana) | NR18 SL Relay [1.5] (Nathan) |  |
|  | NR18 MBS UP/CP [0.75] (Dawid) |  |  |  |
| 14:30-15:30 | NR18 MBS [0.75] (Dawid) | NR18 URLLC [0.5] (Diana) | NR18 Pos [2] (Nathan) |  |
| Thursday |  |  |  |  |
| 03:30-04:30 | NR18 Other [2], NR18 TEI [1] (Johan) | NR18 XR [2] (Tero) | LTE18 IoT NTN [1] (Sergio) |  |
| 04:30-05:30 | NR18 Mobility Enh [2] (Johan) | NR18 XR [2] (Tero) | LTE18 IoT NTN [1] (Sergio) |  |
| Friday |  |  |  |  |
| 03:30-04:30 | NR18 MIMO evo [0.5] (Erlin) | eRedcap [1] (Mattias) 7.19.1 Organizational 7.19.2 Enhanced eDRX in RRC_INACTIVE Incl. AT-meeting email disc summary 7.19.3 Further reduced UE complexity in FR1 Incl. AT-meeting email disc summary | NR18 SONMDT [0.5] (HuNan) |  |
| 04:30-05:30 | NR18 fCovEnh [0.5] (Eswar) |  | NR18 Pos [2] (Nathan) |  |

WEEK 2:

| Time Zone UTC | Web Conference R2 - Main | Web Conference R2 - BO1 | Web Conference R2 - BO2 | Offline GTW Session (limited use, only specific issues if needed, need approval by session chair) |
| --- | --- | --- | --- | --- |
| Monday |  |  |  |  |
| 12:30-13:30 | NR18 Mobility Enh [2] (Johan) | NR18 XR [2] (Tero) | NR18 Pos [2] (Nathan) |  |
| 13:30-14:30 | Maintenance CB (Johan) | NR18 MUSIM [0.5] (Tero) | NR18 UAV [1] (Diana) |  |
| 14:30-15:30 | Maintenance CB (Johan) | CB (Sergio, Tero) | NR18 Network Energy Saving [1] (Diana) |  |
| Tuesday |  |  |  |  |
| 12:30-13:30 | Maintenance CB (Johan) | NR18 QoE [1] (Tero) | Maintenance CB (Diana) |  |
| 13:30-14:30 | NR18 CBs (Sasha) | NR18 CBs (Sergio) | NR18 CB (Diana) |  |
| 14:30-15:30 | NR18 CBs | NR17/18 CBs (Dawid) | CBs (Kyeongin/Qianxi) |  |
| Wednesday |  |  |  |  |
| 03:30-04:30 | NR18 CBs (All?) | NR18 CBs (Mattias?) | NR18 CBs (Nathan) |
| 04:30-05:30 | CB (All) | CB (Sergio?) | CB (Nathan?) |

## 7.19	Enhanced support of reduced capability NR devices

(NR_redcap_enh-Core; leading WG: RAN1; REL-18; WID: RP-223544)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 7.19.1	Organizational

Incoming LSs, etc.

R2-2302417	Reply LS on long eDRX support for RRC_INACTIVE (R3-230803; contact: Ericsson)	RAN3	LS in	Rel-18	FS_REDCAP_Ph2	To:SA2, RAN2

### 7.19.2	Enhanced eDRX in RRC_INACTIVE

PTW location and duration in overlapping/non-overlapping PHs. Which paging to monitor in the PTWs/calulation of T.

Fallback behaviour when UE moves to cell not supporting INACTIVE eDRX > 10.24s.

Support of INACTIVE eDRX (only for UEs supporting Rel-17 eDRX?).

R2-2302496	Fallback behaviour for eRedcap UE	NEC	discussion	NR_redcap_enh-Core

R2-2302497	Paging monitoring for Inactive UE in enhanced eDRX	NEC	discussion	NR_redcap_enh-Core

R2-2302531	Discussion on enhanced eDRX in RRC_INACTIVE	OPPO	discussion	Rel-18	NR_redcap_enh-Core

R2-2302565	Discussion on enhanced eDRX in RRC_INACTIVE	CATT	discussion	Rel-18	NR_redcap_enh-Core

R2-2302642	Discussion on enhanced eDRX in RRC_INACTIVE	China Telecommunications	discussion	Rel-18	NR_redcap_enh-Core

R2-2302703	Discussion on e-DRX for eRedcap Devices	Xiaomi Communications	discussion

R2-2302735	RAN2 impacts to support eDRX in RRC_INACTIVE above 10.24 sec	Intel Corporation	discussion	Rel-18	NR_redcap_enh-Core

R2-2302803	On eDRX for enhanced RedCap	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_redcap_enh-Core

R2-2302815	Discussion on UE fallback behaviour for INACTIVE eDRX	vivo, Guangdong Genius	discussion	Rel-18	NR_redcap_enh-Core

R2-2302816	Enhanced eDRX cycle in RRC_INACTIVE for eRedCap UEs	vivo, Guangdong Genius	discussion	Rel-18	NR_redcap_enh-Core

R2-2302824	Further discussion on longer eDRX in RRC_INACTIVE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_redcap_enh-Core

R2-2303304	Enhanced eDRX in RRC_INACTIVE	MediaTek Inc.	discussion	Rel-18	NR_redcap_enh-Core

R2-2303321	Discussion on available eDRX configurations	Samsung	discussion	Rel-18

R2-2303322	Discussion on enhanced eDRX in RRC_INACTIVE	Samsung	discussion	Rel-18

R2-2303396	RedCap PTW/PH operation for >10.24sec INACTIVE eDRX	Apple	discussion	Rel-18	NR_redcap_enh-Core

R2-2303397	RedCap UE behavior in cells not supporting R18 eDRX	Apple	discussion	Rel-18	NR_redcap_enh-Core

R2-2303468	Discussion on enhanced eDRX in RRC_INACTIVE	Huawei, HiSilicon	discussion	Rel-18	NR_redcap_enh-Core

R2-2303542	Discussion on eDRX in RRC_INACTIVE	CMCC	discussion	Rel-18	NR_redcap_enh-Core

R2-2303561	Discussion on enhanced eDRX in RRC inactive	Qualcomm Incorporated	discussion	NR_redcap_enh-Core

R2-2304063	Extending eDRX cycles in RRC_INACTIVE	Ericsson	discussion	Rel-18	NR_redcap_enh-Core

### 7.19.3	Further reduced UE complexity in FR1

Early indication.

Access restriction for eRedCap.

Capability related, e.g. how to define an eRedCap UE.

R2-2302528	Discussion on access restriction for eRedCap	Futurewei	discussion	Rel-18	NR_redcap_enh-Core

R2-2302532	Discussion on early indication for eRedCap UE	OPPO	discussion	Rel-18	NR_redcap_enh-Core

R2-2302544	Discussion on cellbarring for eRedCap UEs	OPPO	discussion	Rel-18	NR_redcap_enh-Core

R2-2302566	Discussion on further UE complexity reduction	CATT	discussion	Rel-18	NR_redcap_enh-Core

R2-2302640	Discussion on access restriction and capability related for eREDCAP 	China Telecommunications	discussion	Rel-18	NR_redcap_enh-Core

R2-2302641	Discussion on Early Indication for eREDCAP	China Telecommunications	discussion	Rel-18	NR_redcap_enh-Core

R2-2302704	Discussion on early indication for eRedcap devices	Xiaomi Communications	discussion

R2-2302705	Discussion on UE access restrictions and other impacts for eRedcap devices	Xiaomi Communications	discussion

R2-2302736	RAN2 impacts to support Rel-18 RedCap UEs	Intel Corporation	discussion	Rel-18	NR_redcap_enh-Core

R2-2302737	Capability impacts to support Rel-18 RedCap UEs	Intel Corporation	discussion	Rel-18	NR_redcap_enh-Core

R2-2302802	On access restrictions for enhanced RedCap	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_redcap_enh-Core

R2-2302817	Discussion on access restriction and capability for eRedCap	vivo, Guangdong Genius	discussion	Rel-18	NR_redcap_enh-Core

R2-2302825	Early indication and access restriction for eRedCap UE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_redcap_enh-Core

R2-2302826	Capability definition and report for eRedCap UE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_redcap_enh-Core

R2-2302949	Discussion on early indication and access restriction for eRedCap	NEC	discussion	Rel-18	NR_redcap_enh-Core

R2-2303069	Early identification and access restriction for eRedCap UEs	Huawei, HiSilicon	discussion	Rel-18	NR_redcap_enh-Core

R2-2303070	Discussion on how to define and capture the capability of eRedCap UEs	Huawei, HiSilicon	discussion	Rel-18	NR_redcap_enh-Core

R2-2303149	Discussion on access restriction for eRedCap	Sharp	discussion

R2-2303305	Early identification for eRedCap devices	MediaTek Inc.	discussion	Rel-18	NR_redcap_enh-Core

R2-2303306	Access restrictions for eRedCap devices	MediaTek Inc.	discussion	Rel-18	NR_redcap_enh-Core

R2-2303323	Discussion on early indication and access restriction	Samsung	discussion	Rel-18

R2-2303543	Discussion on further reduced UE complexity	CMCC	discussion	Rel-18	NR_redcap_enh-Core

R2-2303562	Discussion on further complexity reduction for eRedCap UE	Qualcomm Incorporated	discussion	NR_redcap_enh-Core

R2-2303563	Discussion on optional UE capability filter for eRedCap UE	Qualcomm Incorporated, Ericsson, Intel	discussion	NR_redcap_enh-Core	R2-2301294

R2-2303568	Discussion on further reduced UE complexity in FR1 for Rel-18 RedCap UE	Spreadtrum Communications	discussion	Rel-18

R2-2303657	Early indication and access restrictions for eRedCap UE	Sierra Wireless. S.A.	discussion

R2-2303689	On early indication for enhanced RedCap	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_redcap_enh-Core

R2-2304010	Further discussion on early indication for Rel-18 RedCap UE	LG Electronics Inc.	discussion	Rel-18	NR_redcap_enh-Core

R2-2304062	Early indication for eRedCap UEs	Ericsson	discussion	Rel-18	NR_redcap_enh-Core

R2-2304064	Discussion on cell barring for eRedCap UEs	Ericsson	discussion	Rel-18	NR_redcap_enh-Core

R2-2304069	Discussion on further UE complexity reduction for eRedCap	NTT DOCOMO INC.	discussion	Rel-18	NR_redcap_enh-Core

=> Revised in R2-2304190

R2-2304190	Discussion on further UE complexity reduction for eRedCap	NTT DOCOMO INC.	discussion	Rel-18	NR_redcap_enh-Core

R2-2304171	Considerations on Further reduced UE complexity for eRedcap	Sequans Communications	discussion	Rel-18	NR_redcap_enh-Core