3GPP TSG-RAN WG2 #121bis-e	Tdoc R2-23xxxxx

Electronic meeting, Apr 17th – 26th, 2023

Agenda Item:	5.1.3.1

Source:	Ericsson

Title:	[AT121bis-e][002][NR1516] RRC 1

Document for:	Discussion, Decission

# 1	Introduction

The following document summarizes the following email discussion:

[AT121bis-e][002][NR1516] RRC 1 (Ericsson)

Scope: Treat R2-2303635, R2-2303636, R2-2303282, R2-2303283, R2-2303284, R2-2303285, R2-2302881, R2-2302882, R2-2304093, R2-2304094, R2-2304095
Ph1: Determine agreeable parts. Ph2: For agreeable parts, if any, reflect these in agreeable CRs.

Intended outcome: Report, If applicable: In-Principle-Agreed CRs

Deadline: Schedule 1

Discussions with Deadline Schedule 1:

A first round with Deadline W1 Thursday April 21th 1200 UTC to settle scope what is agreeable etc

A Final round with Final deadline W2 Wednesday April 26th 1000 UTC (EOM) to settle details / agree CRs etc.

Companies are invited to fill in contact details.

| Company | Contact details |
| --- | --- |
| Ericsson | hakan.l.palm@ericsson.com |

# 2	Discussion

## 2.1	SIB and PosSIB mappings to SI message

high level decision done at previous meeting – Discussion on CRs was postponed

R2-2303635	SIB and PosSIB mappings to SI message	Ericsson, MediaTek Inc.	CR	Rel-16	38.331	16.12.0	3895	1	F	NR_newRAT-Core, NR_pos-Core	R2-2301452

R2-2303636	SIB and PosSIB mappings to SI message	Ericsson, MediaTek Inc.	CR	Rel-17	38.331	17.4.0	3894	1	F	NR_newRAT-Core, NR_pos-Core	R2-2301451

Q1. Do companies agree with the intention and need of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proponent) | Yes |  |

Q2. If “yes” on Q2.1, please provide detailed comments on the CR.

| Company | Comments |
| --- | --- |
| Ericsson (proponent) | Rel-17 CR should be Cat A (error at tdoc allocation, CR cover page is correct). |

## 2.2	drb-ContinueROHC

R2-2303282	Clarification on drb-ContinueROHC	ZTE Corporation, Sanechips	discussion	Rel-15	NR_newRAT-Core

In this contribution, the followong proposals and observations are made:
Observation 1: Based on current specification, when drb-ContinueROHC field is included, the UE shall continue ROHC during PDCP re-establishment, otherwise, the UE shall reset ROHC.
Observation 2: If drb-ContinueROHC was signalled before, but the network does not include the parent Need M IE pdcp-Config in follow up RRC message, the UE behaviors are different.
Observation 3: Based on the definition of Need N, the UE does not store the Need N field.
Observation 4: There are other examples in 38.331 that when parent Need M IE is not included, its child Need N field will be treated as “not present”.
Proposal 1: RAN2 confirms that during PDCP re-establishment, when pdcp-Config is not included and Need M works, the child Need N IE drb-ContinueROHC is treated as “not present” and the UE shall reset ROHC protocol (i.e. the UE does not store the drb-ContinueROHC field for future use).

R2-2303283	Clarification on handling of Need N fields	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.21.0	4002	-	F	NR_newRAT-Core

R2-2303284	Clarification on handling of Need N fields	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.12.0	4003	-	A	NR_newRAT-Core

R2-2303285	Clarification on handling of Need N fields	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.4.0	4004	-	A	NR_newRAT-Core

Q3. Do companies agree with P1 in R2-2303282?

Proposal 1: RAN2 confirms that during PDCP re-establishment, when pdcp-Config is not included and Need M works, the child Need N IE drb-ContinueROHC is treated as “not present” and the UE shall reset ROHC protocol (i.e. the UE does not store the drb-ContinueROHC field for future use).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | It is clear that Need N field is one-shot and not memorized by UE. |

Q4. Do companies agree with the intention and need of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Maybe | A CR is not essentially needed, since already clear (see above). If anyway RAN2 thinks this need clarification in Guidelines, see below for comments. The change should be captured in a 38331 Rapp CR of non-controversial changes. |

Q5. If “yes” on Q3, please provide detailed comments on the CRs.

| Company | Comments |
| --- | --- |
| Ericsson | Ok to add new Need N field in example, but simplify the text e.g. as - if field1 in RRCMessage-IEs is absent, UE does not modify or take any action on child fields configured within field1 (regardless of their need codes); |

## 2.3	RLC-Config

R2-2302881	Correction on Need code of IE RLC-Config	Intel Corporation	CR	Rel-16	38.331	16.12.0	3969	-	F	NR_IIOT-Core

R2-2302882	Correction on Need code of IE RLC-Config	Intel Corporation	CR	Rel-17	38.331	17.4.0	3970	-	F	NR_IIOT-Core, NR_NTN_solutions-Core

Q6. Do companies agree with the intention and need of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | We agree the Need N should have been Need R, and are fine to change to this. One could expect that networks always include t-StatusProhibit-v1610 when a value from this range is used (since not clear that UE keeps the value, if rlc-Config-v1610 is included). |

Q7. If “yes” on Q3, please provide detailed comments on the CR.

| Company | Comments |
| --- | --- |
| Ericsson | See above. If change from Need N to Need R is not acceptable in RAN2, we should describe the expected nw workaround as above (networks always include t-StatusProhibit-v1610 when a value from this range is used). Then, change to Need M or Need R does not matter. t-StatusProhibit-v1610 can be released thanks to the Need R on rlc-Config-v1610. |

## 2.4	Coreset0 for PSCell

R2-2304093	Clarification on presence of Coreset0 for PSCell	Ericsson	CR	Rel-15	38.331	15.21.0	4054	-	F	NR_newRAT-Core

R2-2304094	Clarification on presence of Coreset0 for PSCell	Ericsson	CR	Rel-16	38.331	16.12.0	4055	-	A	NR_newRAT-Core

R2-2304095	Clarification on presence of Coreset0 for PSCell	Ericsson	CR	Rel-17	38.331	17.4.0	4056	-	A	NR_newRAT-Core

Q8. Do companies agree with the intention and need of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proposent) | Yes |  |

Q9. If “yes” on Q3, please provide detailed comments on the CR.

| Company | Yes/No | Comments |
| --- | --- | --- |

Summary: TBD

.

TBD

# Conclusion

The following is proposed as outcome of this email discussion.

Proposal 1	TBD

# Appendix