3GPP TSG-RAN WG2 #121bis-e	Tdoc R2-23xxxxx

Electronic meeting, Apr 17th – 26th, 2023

Agenda Item:	5.1.3.1

Source:	Ericsson

Title:	[AT121bis-e][002][NR1516] RRC 1

Document for:	Discussion, Decission

1	Introduction

The following document summarizes the following email discussion:

[AT121bis-e][002][NR1516] RRC 1 (Ericsson)

Scope: Treat R2-2303635, R2-2303636, R2-2303282, R2-2303283, R2-2303284, R2-2303285, R2-2302881, R2-2302882, R2-2304093, R2-2304094, R2-2304095
Ph1: Determine agreeable parts. Ph2: For agreeable parts, if any, reflect these in agreeable CRs.

Intended outcome: Report, If applicable: In-Principle-Agreed CRs

Deadline: Schedule 1

Discussions with Deadline Schedule 1:

A first round with Deadline W1 Thursday April 21th 1200 UTC to settle scope what is agreeable etc

A Final round with Final deadline W2 Wednesday April 26th 1000 UTC (EOM) to settle details / agree CRs etc.

Companies are invited to fill in contact details.

| Company | Contact details |
| --- | --- |
| Ericsson | hakan.l.palm@ericsson.com |
| Qualcomm Inc | mambriss@qti.qualcomm.com |
| MediaTek | chun-fan.tsai@mediatek.com |
| Xiaomi | Wangshukun3@xiaomi.com |
| OPPO | shicong@oppo.com |

2	Discussion

2.1	SIB and PosSIB mappings to SI message

high level decision done at previous meeting – Discussion on CRs was postponed

R2-2303635	SIB and PosSIB mappings to SI message	Ericsson, MediaTek Inc.	CR	Rel-16	38.331	16.12.0	3895	1	F	NR_newRAT-Core, NR_pos-Core	R2-2301452

R2-2303636	SIB and PosSIB mappings to SI message	Ericsson, MediaTek Inc.	CR	Rel-17	38.331	17.4.0	3894	1	F	NR_newRAT-Core, NR_pos-Core	R2-2301451

Q1. Do companies agree with the intention and need of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proponent) | Yes |  |
| Qualcomm Inc | Yes | Changes are aligned with our understanding. |
| MediaTek | Yes | We co-sing the CR |
| Xiaomi | Yes |  |
| OPPO | Yes |  |

Q2. If “yes” on Q2.1, please provide detailed comments on the CR.

| Company | Comments |
| --- | --- |
| Ericsson (proponent) | Rel-17 CR should be Cat A (error at tdoc allocation, CR cover page is correct). |

2.2	drb-ContinueROHC

R2-2303282	Clarification on drb-ContinueROHC	ZTE Corporation, Sanechips	discussion	Rel-15	NR_newRAT-Core

In this contribution, the followong proposals and observations are made:
Observation 1: Based on current specification, when drb-ContinueROHC field is included, the UE shall continue ROHC during PDCP re-establishment, otherwise, the UE shall reset ROHC.
Observation 2: If drb-ContinueROHC was signalled before, but the network does not include the parent Need M IE pdcp-Config in follow up RRC message, the UE behaviors are different.
Observation 3: Based on the definition of Need N, the UE does not store the Need N field.
Observation 4: There are other examples in 38.331 that when parent Need M IE is not included, its child Need N field will be treated as “not present”.
Proposal 1: RAN2 confirms that during PDCP re-establishment, when pdcp-Config is not included and Need M works, the child Need N IE drb-ContinueROHC is treated as “not present” and the UE shall reset ROHC protocol (i.e. the UE does not store the drb-ContinueROHC field for future use).

R2-2303283	Clarification on handling of Need N fields	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.21.0	4002	-	F	NR_newRAT-Core

R2-2303284	Clarification on handling of Need N fields	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.12.0	4003	-	A	NR_newRAT-Core

R2-2303285	Clarification on handling of Need N fields	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.4.0	4004	-	A	NR_newRAT-Core

Q3. Do companies agree with P1 in R2-2303282?

Proposal 1: RAN2 confirms that during PDCP re-establishment, when pdcp-Config is not included and Need M works, the child Need N IE drb-ContinueROHC is treated as “not present” and the UE shall reset ROHC protocol (i.e. the UE does not store the drb-ContinueROHC field for future use).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | It is clear that Need N field is one-shot and not memorized by UE. |
| Qualcomm Inc | No | it’s a clear violation to the 38.331 spec section 6.1.2 For downlink RRC message and sidelink PC5 RRC messages, the need codes, conditions and ASN.1 defaults specified for a particular (child) field only apply in case the (parent) field including the particular field is present. Thus, if the parent is absent the UE shall not release the field unless the absence of the parent field implies that. Since the Parent IE (pdcp-Config) is not present to consider the Need Code of the Child IE, nor the absence of the parent IE does imply the release (as it’s a Need M), therefore considering the Child IE and its Need Code by releasing it, is considered against the behaviour described above in the spec. |
| MediaTek | Yes | By definition, Need N for one-shot behavior. Also in this particular case, there is procedure text saying that the UE only indicating “drb-ContinueROHC is configured” to PDCP if drb-ContinueROHC is included. PDCP entity shall NOT continue ROHC if this field is not present. 1> for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer: 2> if the reestablishPDCP is set: 3> [Skip unrelated part] 3> if drb-ContinueROHC is included in pdcp-Config: 4> indicate to lower layer that drb-ContinueROHC is configured; |
| Xiaomi | Yes | For need N code, it is one-short and is not stored. If network wang to use this filed, the network should indicate again. It makes sense that the delta configuration filed only apply to the stored fields. |
| OPPO | Yes |  |

Q4. Do companies agree with the intention and need of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Maybe | A CR is not essentially needed, since already clear (see above). If anyway RAN2 thinks this need clarification in Guidelines, see below for comments. The change should be captured in a 38331 Rapp CR of non-controversial changes. |
| Qualcomm | No | . |
| MediaTek | Maybe | Current SPEC already implies the behavior proposed by P1. No strong need to have this CR or not. |
| Xiaomi | Yes | It makes the spec more clear and can be captured in Rapp CR. |
| OPPO | Maybe | We agree with Ericsson, if companies agree a clarification is needed, we’re ok to capture it in the rapp-CR. |

Q5. If “yes” on Q3, please provide detailed comments on the CRs.

| Company | Comments |
| --- | --- |
| Ericsson | Ok to add new Need N field in example, but simplify the text e.g. as - if field1 in RRCMessage-IEs is absent, UE does not modify or take any action on child fields configured within field1 (regardless of their need codes); |
| MediaTek | Ericsson wording is okay for us |

2.3	RLC-Config

R2-2302881	Correction on Need code of IE RLC-Config	Intel Corporation	CR	Rel-16	38.331	16.12.0	3969	-	F	NR_IIOT-Core

R2-2302882	Correction on Need code of IE RLC-Config	Intel Corporation	CR	Rel-17	38.331	17.4.0	3970	-	F	NR_IIOT-Core, NR_NTN_solutions-Core

Q6. Do companies agree with the intention and need of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | We agree the Need N should have been Need R, and are fine to change to this. One could expect that networks always include t-StatusProhibit-v1610 when a value from this range is used (since not clear that UE keeps the value, if rlc-Config-v1610 is included). |
| Qualcomm Inc | Yes | CR seems aligned with the previous agreement |
| MediaTek | See comment | The inter-operability analysis is not so correct, if UE implemented this as “Need M”, there may be some inter-operability issue. We can change to Need R, but prefer also saying “networks always include t-StatusProhibit-v1610 when a value from this range is used” as commented by Ericsson. In this case, Need R or Need M does not make too much difference but anyway better to change Need N. |
| Xiaomi | Yes | it is fine and the change is aligned with previous agreements. But I wonder whether there is CB issue at this time point? |
| OPPO | Yes |  |

Q7. If “yes” on Q3, please provide detailed comments on the CR.

| Company | Comments |
| --- | --- |
| Ericsson | See above. If change from Need N to Need R is not acceptable in RAN2, we should describe the expected nw workaround as above (networks always include t-StatusProhibit-v1610 when a value from this range is used). Then, change to Need M or Need R does not matter. t-StatusProhibit-v1610 can be released thanks to the Need R on rlc-Config-v1610. |

2.4	Coreset0 for PSCell

R2-2304093	Clarification on presence of Coreset0 for PSCell	Ericsson	CR	Rel-15	38.331	15.21.0	4054	-	F	NR_newRAT-Core

R2-2304094	Clarification on presence of Coreset0 for PSCell	Ericsson	CR	Rel-16	38.331	16.12.0	4055	-	A	NR_newRAT-Core

R2-2304095	Clarification on presence of Coreset0 for PSCell	Ericsson	CR	Rel-17	38.331	17.4.0	4056	-	A	NR_newRAT-Core

Q8. Do companies agree with the intention and need of the CRs above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson (proposent) | Yes |  |
| Qualcomm Inc | Yes |  |
| MediaTek | Maybe | Fine to have this CR although we think the agreement in previous meeting is enough |
| Xiaomi | Yes |  |
| OPPO | Yes |  |

Q9. If “yes” on Q3, please provide detailed comments on the CR.

| Company | Yes/No | Comments |
| --- | --- | --- |

Summary: TBD

.

TBD

Conclusion

The following is proposed as outcome of this email discussion.

Proposal 1	TBD

Appendix