3GPP TSG-RAN WG2 Meeting #121-bis electronic             	R2-23xxxxx

17th – 26th Apr. 2023

Source:	Lenovo (Rapporteur)

Title:	Summary of EMAIL DISC [121bis#xxx] on AI 7.3.5 Connected Mode Mobility

Agenda Item:	7.3.5

Document for:	Discussion and Decision

- Introduction

This contribution is to check company views on different aspects of NES Connected Mode Mobility, and accordingly formulate agreeable proposals based on submitted contributions from agenda item 7.3.5.

First, kindly fill in the contact information:

| Name | Company | Email-Id |
| --- | --- | --- |
| Shukun Wang | Xiaomi | Wangshukun3@163.com |
| Lili Zheng | Huawei, HiSilicon | zhenglili4@huawei.com |
| Peng Cheng | Apple | pcheng24@apple.com |
| Seau Sian Lim | Intel Corporation | seau.s.lim@intel.com |
| Jarkko Koskela | Nokia | Jarkko.t.koskela@nokia.com |
| Sherif ElAzzouni | Qualcomm | selazzou@qti.qualcomm.com |
| Zhe Fu | OPPO | fuzhe@OPPO.com |
| Katsunari Uemura | Fujitsu | u-katsunari@fujitsu.com |
| Ming-Hung Tao | Google | mhtao@google.com |
| Prateek Basu Mallick | Lenovo | pmallick@lenovo.com |
| Max Lu | T-Mobile USA | Kun.lu7@t-mobile.com |
| Faris Alfarhan | InterDigital | faris.alfarhan@interdigital.com |
| Lu Ting | ZTE | lu.ting@zte.com.cn |
| Maxime Grau | NEC | Maxime.grau@emea.nec.com |
| Byoung-hoon Jung | Samsung | bh14.jung@samsung.com |
| LIU Lei | Sharp | lei.liu@cn.sharp-world.com |

- Basic Understanding

We will first discuss some basic aspects that can help progress more technical issues later:

  - NES Techniques for RRC Connected UEs for this email discussion

Following techniques are mentioned in the WID and company contributions:

- Cell DTX/DRX mechanism

- Cell switch-off

- Spatial and power domain techniques

For Spatial and power domain techniques, RAN1 is the primary group (as shown below from an excerpt of RP-230566):

Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

Some companies have made proposals in this sub-area as follows:

LG Proposal 1: Do not pursue CHO enhancement (NW-triggered mobility to a preconfigured target) for spatial/power-domain NES technique and Cell DTX/DRX NES technique. CATT, Turkcell Proposal 4: CHO with CondEvent A4 can also be used to offload UEs for spatial NES technique and no other enhancement for CHO is needed.

However, RAN2 has not received any conclusive LS so far which can be used as basis for our further work as far as this sub-area is concerned. So, Rapporteur devotes this email discussion primarily to first two techniques only.

Further, Rapporteur would further like to check company opinion to see if it is acceptable to consider cell switch-off as a special case of Cell DTX/ DRX mechanism. This simplification is used here to

- if the DTX/ DRX periodic sleep time is rather small (for smaller duty cycles) and UE would not be needed to be handed over to another cell to fulfill the QoS of the already running bearer, can be managed by network implementation using legacy principles

- if the cell’s sleep time would affect QoS fulfillment of the already running bearer, the network would rather prefer to handover the UE to a suitable candidate – for not only cell switch off case but also for Cell DTX/ DRX mechanism.

Question 1: Is it acceptable (at least) for this email discussion to consider Cell DTX/ DRX as also representing cell switch off technique? If this is not so, please highlight here or in subsequent places the important differences leading to unique UE/ network behavior in each case.

| Company Name | Yes/ No | Comments |
| --- | --- | --- |
| Xiaomi | No | - Only cell DTX/DRX is considered for now. - For cell switch-off, it is not in the scope of the WI. Furthermore, cell switch-off is not a special case of Cell DTX/ DRX mechanism. In my understanding, cell switch-off means that there is no paging, SSB, SIB transmission. - For Spatial and power domain techniques, it is up to RAN1. |
| Huawei, HiSilicon | No | Activating Cell DTX/DRX does not mean the NW intends to handover the UEs. Rapp) Please see the yellow highlighted text. Rapporteur did not mean that Cell DTX/ DRX leads to a blanket handover for all UEs. Legacy UEs and non-NES capable UEs cannot recognize the R18 CHO enhancements, the only targets for this enhancement are the NES capable UEs but these UEs are exactly what Cell DRX/DRX are designed for Rapp) The legacy and non-NES capable UEs are not part of this discussions – RAN2 already discussed mechanism to restrict camping of NES capable (non-legacy) UEs, so the UEs transitioning to RRC Connected will naturally be only NES capable UEs. If in the exceptional case some UE needs to be HO’ed to a non-NES cell to fulfill the QoS requirement, this should be managed by legacy HO. We think the discussion related to source cell NES mode should focus on the switching off case (not including Cell DTX/DRX), because that scenario brings the largest gain (all UEs need to be HO’ed). The discussion related to target cell NES mode (i.e. selecting an appropriate target cell) can consider all NES techniques including Cell DTX/DRX. |
| Apple | Yes | In our understanding, there are at least below 2 cases target for CHO enhancement: - Source cell activates Cell DTX/DRX, and the activated Cell DTX/DRX will degrade QoS of served UEs (e.g. a long non-active duration) - Source cell turns off We assume Rapporteur intends to include above 2 cases and avoid unnecessary discussion on clarification on what is “cell OFF mode” and whether to specify it. We agree this is a good way-forward. |
| Intel | See comments | We are not sure of the intention of the question. To us, both techniques target different scenarios to be considered for CHO enhancement. For the cell off case, all the UEs in the cell needs to be handover/CHO while for the application of the NES techniques case (e.g. application of Cell DTX/DRX and/or spatial/domain techniques), it may not be all the UEs in the cell. Rapp) Since the specifications are written from a (per) UE perspective, the question is intended to see if the Cell DTX/ DRX and Cell switch off will impact the UE behavior differently? Since we are considering the mobility topic here, for both cases – when the need arises – the UE will be handed over to a target cell. |
| Vodafone | Yes | We agree manly with Apple. We should include both cases and also discuss both case where the outcome might be the same or different |
| Nokia | No | Turning cell of is quite different compared to activating cell DTX/DRX (which we don’t actually know what it will be in this WI) and Generally what would be benefit of “generalizing” these? Anyway we need to consider whichever method is introduced in this WI for CHO as well. Rapp) Hope the clarification provided to Intel explain the intention to generalize these two mechanisms. |
| Qualcomm | No | We already have a proposal in the [312] that specifies alignment requirement of UE CDRX and cell DTX. Cell DTX in general contains a periodicity which means that it follows a certain ON-OFF pattern. The rapporteurs suggestion seems to imply that UEs perform a CHO in the OFF period beginning, then what happens in the ON period? - Option 1: UEs are offloaded and served by another serving cell, thus the cell is ON without serving any UEs and will start getting them gradually, which does not make sense in the realm of an ON-OFF cycle. Afterall, if this is the goal, why not simply switch OFF the cell then switch it back ON without a periodic pre-determined cycle. - Option 2: UEs go back to the gNB during the ON cycle. In this case we are on-purpose introducing a ping-pong behavior in the UE which is we always try to avoid due to UE power consumption, QoS,, UPT, Service continuity, backhaul signalling, etc. Rapp) Not sure where’s this confusion coming from. In both cell switch off and cell DTX/ DRX cases, the UE might need to be handed over to another cell when/ before the source cell starts to sleep. There’s no further implication from this point. Thus, we think CHO should be kept completely separate from Cell DTX/DRX. Rapp) CHO is one potential solution which in my opinion almost all companies are considering to let the source cell enjoy some energy saving by using Cell DTX/ DRX. Not sure if by “separate” you mean that CHO is not used for Cell DTX/ DRX cases? As for Apple’s comment that DTX/DRX can get too aggressive for UE QoS, then this would be no different from any energy saving measure gNB can take e.g., Reducing Tx power or completely switching off. The gNB can use whatever scheme we develop here to offload UEs for any reason, so explicitly spelling out cell DTX/DRX in not recommended. Rapp) RAN2 is starting with cell DTX/ DRX and/ or cell switch off. The solution developed here can be used in another scenarios/ techniques, there’s no attempt to preclude anything yet. |
| OPPO | Yes | Per the Rapporteur clarification, I understand the question intends to figure out the suitable case(s) that would trigger the enhanced CHO as well as avoid the detailed discussion on cell off (e.g. what is the definition of the cell on/off, whether cell on/off is periodic, etc). We are fine to discuss the use case(s) and prefer a unified CHO enhancement mechanism if it can cover multiple use case(s). With this in mind, we understand at least those two cases below can be considered for the enhanced CHO, i.e. 1) source cell off, 2) source cell applies cell DTX/DRX if it intolerantly degrades the UE’s performance. |
| Fujitsu | See comments | Based on rapporteur’s further clarification, we think the intention is whether both cases can be used as the NES mode change for CHO enhancements. Then we can accept it as there are no different behavior from UE perspective. However, a cell can be turned off even now by NW implementation. Therefore, to turn the cell off/on, we think it should not be limited to use Cell DTX/DRX technique. |
| Google | Yes | We share the same understanding as Apple. |
| Sony | No | We understand that cell switch off and cell DTX/DRX are different techniques. |
| Lenovo | Yes |  |
| Ericsson | No | Agree with Huawei. |
| T-Mobile USA (TMUS) | No | We think these are two different concepts; there are Cell DTX/DRX On Active Duration and non-Active Duration; also we try to align Cell DTX/DRX with UE CDRX cycle. While Cell Switch Off means this cell is out of service |
| InterDigital | No | Cell switch off and Cell DTX are not the same. Though we think both should be considered for this discussion, it is not necessary to lump them together, especially since some UEs can served with Cell DTX activated. |
| ZTE | No | We also think cell switch off and cell DTX/DRX are different techniques. They may cause very different impacts on UE and therefore the resulting solutions might also be very different. So firstly we think it’s not suitable to give such statement “Cell DTX/ DRX as representing cell switch off technique”. Moreover, we disagree with this statement that “In both cell switch off and cell DTX/ DRX cases, the UE might need to be handed over to another cell when/ before the source cell starts to sleep.” This is cannot be the explanation from Rapp, it’s just company’s own view. Specifically, for cell DTX/DRX scenario, even the discussion is still ongoing, some schemes have been proposed to reduce the impact on SR/CG transmission or SPS/PDCCH reception for delay sensitive services. So we expect that the enabled cell DTX/DRX could have little impacts on UE performance and therefore handover is unnecessary. Meanwhile, for cell switch off case, we agree all the UEs in the cell needs to be handover, e.g., via legacy HO or CHO. |

| NEC | No | In the case of NES-capable UEs, some level of Cell DTX/DRX is not a deterrent to being connected to the cell. Only legacy UEs should leave the cell, where NBC Cell DTX/DRX would mean Cell switch-off. |
| --- | --- | --- |
| Samsung | Yes | We understand the cell DTX/DRX itself is to introduce radio silent periods for a cell to switch off its partial/full circuitry for power saving. |
| Sharp | Yes | Based on rapporteur’s further clarification, both cases could be supported in CHO enhancements and maybe unified method could be designed finally for CHO enhancements. |

  - Definition of NES mode

Many companies expressed views on how to define “NES mode”:

[Qualcomm] Proposal 1: Source cell NES mode means a cell that is observing low load and intends to offload all connected UEs and physically switch-off. [Huawei] Proposal 1: “NES mode” means the cell is enabling an NES technique or turning off. [Samsung] Proposal 2. RAN2 needs to discuss and determine the adequate NES modes for the CHO: A. cell DTX/ DRX B. spatial domain (e.g., adjustment of antenna ports, active transceiver chains) C. power domain (e.g., adjustment of power offset values) D. bandwidth domain (e.g., adjustment of bw, or BWP) [Lenovo] Proposal 1: RAN2 discuss if one or more of the following terms are useful and should be defined in specification: A cell in NES state A cell not in NES state A perfect target An acceptable target A sleeping target.

In the scope of this email discussion, we can limit the scope of “NES mode” to RRC Connected mobility. Later to include also the RRC Idle/ Inactive UEs, a more general definition can be agreed.

Coming to the definition itself, a definition should be meaningful from UE’s perspective and should differentiate between a cell in NES mode but in active time currently from cell not in NES mode or from a cell in NES mode and sleeping currently:

A cell is in (or will enter) NES mode means a NES cell is (or subsequently will be) saving energy but may or may not be in sleep (DTX/ DRX) “now”.

A cell is sleeping if it is in NES mode and in DTX/ DRX idle/ sleep period “now”.

Many companies have used phrases like “in NES mode, so above definitions are only used as a basis for driving this email discussion and companies may choose to use this for further work/ specification:

Question 2: Are the following definitions acceptable to your company as way forward?

A cell is in (or will enter) NES mode means a NES cell is (or subsequently will be) saving energy but may or may not be in sleep (DTX/ DRX) “now”.

A cell is sleeping if it is in NES mode and in DTX/ DRX idle/ sleep period “now”.

| Company Name | Yes/ No |  | Comments |
| --- | --- | --- | --- |
| Xiaomi | With comments |  | The cell is in NES mode when the Cell DTX/DRX is configured and activated if the activation/deactivation mechanism is defined. |
| Huawei, HiSilicon | No |  | We don’t see the necessity to “differentiate between a cell in NES mode but in active time currently from cell not in NES mode or from a cell in NES mode and sleeping currently”. The “active time” and “sleeping” states of the NES techniques are switched quite dynamically, and could even be realized by L1/L2 signaling according to the discussion in Cell DTX/DRX, they should not affect the L3 mobility which is normally based on RRC message. The definition of Nes mode should be simple and mean that the cell is enabling an NES technique or turning off. Rapp) Then how to interpret someone saying that “cell is in NES mode”? What does it mean? |
| Apple | No |  | - We agree with Huawei that no need to define “A cell is sleeping”. - We are not sure whether it is really necessary to specify an official definition of “NES mode”. In our understanding, this discussion is because terminology of “NES cell” was captured in TR 38.864 and different company have different understanding on this terminology. However, since it is WI phase, we may avoid using terminology of “NES cell” in normative spec (e.g. we can just say “a cell which is adopting NES technology”). Then this issue doesn’t exist. In summary, we suggest: 1. No need to capture “NES cell” or “NES mode” and its definition in normative spec. 2. In normative spec, if needed, we can just use “a cell which is adopting NES technology”, where NES technology can also be replaced by “Cell DTX/DRX” or others Rel-18 NES technology, depending on the text before and after the specification. |
| Intel | See comments |  | From the source cell pov, it would be good that CHO configuration is executed before the NES techniques are applied in the source cell so that the affected UEs are not impacted by the NES techniques. For the target cell pov, the UE should try to avoid selecting a target cell that has applied (or going to apply) a NES technique that may not be suitable to the UE. Hence we are not sure whether the above definitions are useful for the discussion. |
| Vodafone | No |  | It is for a reason we do not have official discussions during this meeting for NES definition and I think we should not have it here too. From our point of view, we should speak about Cell DRX/DTX capable UEs and in my view there is probably no need for NW definition. Rapp) The main necessity from Rapp’s perspective is to ease our discussion. There’s no attempt here to force these definitions to specification. Rapp thinks that “cell is in NES mode” is not just one single scenario. |
| Nokia | No |  | We don’t really see benefit of trying to decide definition now |
| Qualcomm | No |  | Unclear what any of these definitions achieve. These definitions lack specifics on what the UE is expecting and “now” emphasis seems to be an indirect agreement against introducing some CHO timer or some other proposals by companies. In this case, we can discuss timers’ proposals directly as we did in some questions below. In our view, Cell DTX/DRX applies for connected UEs and should stay this way. Source cell CHO implies that all UEs are asked to be offloaded from the cell; The exact reason the UEs are being offloaded via connected mode mobility does not need an airtight definition here. |
| OPPO | See comments |  | It may good to have a clear definition to understand what cell state would be considered for the enhanced CHO, e.g. the source cell is going to enter cell DTX/DRX non-active (or cell off), the target cell will enter or enters cell DTX/DRX non-active(or cell off). Considering there would be other cases applicable to the enhanced CHO, if we need to use a definition e.g. in the normative work, the definition would be future-proof. |
| Fujitsu | No |  | The definition should not be tied to specific NES technique. If it is coupled with, for example, Cell DTX/DRX technique, every time a new NES technique is supported in a future, we should update the definition. Then any specific NES techniques like DTX/DRX should not be included in the definition. |
| Google | No |  | There might still be some confusions even with the definitions given by the moderator. For instance, if a cell enters non-active periods in a fully dynamic way (which means the cell may not enter any non-active period eventually), is this cell in NES mode or not? Also, if a cell has been switched off entirely, can we consider the cell as a sleeping cell? Moreover, the merits of having such formal definitions are unclear. |
| Sony | See comments |  | We think it’s beneficial to clarify the NES modes but should allow more time to consider. Not sure the need to define a sleep cell. |
| Lenovo | Yes |  | This eases discussions and brings clarity to the understanding of scenarios. |
| Ericsson | No |  | We do not see the need to define NES mode, this can merely be some way to make it easier to specify NES features that we may use once we draft CRs (if easy to achieve). That is depends if the question is to define in spec or define for the discussion. Latter is fine. |
| T-Mobile USA (TMUS) | No |  | Agreed with Apple’s points |
| InterDigital | No |  | This seems to be introducing many new definitions, and “sleep” isn’t really described in agreements or the TR language. We are fine with the definition suggested by Huawei and Apple, i.e. “a cell enabling a NES technique or turning off” |
| ZTE | No |  | Agreed with Apple’s points, almost all. As it’s more and more clear to us that different NES technologies may have different impacts, we even think, for normative spec, it may not be suitable to use “a cell which is adopting NES technology” as maybe no common/concrete processes can be identified for such cell. We think only such term as “a cell which is activated Cell DTX/DRX” may be suitable. |
| NEC | No | In our understanding, the only relevant distinction is between Rel-18+ NES-cable and other non-NES-capable UEs. For NES-capable UEs, we do not see a need to define whether a cell is currently operating in legacy mode or is using Rel-18(+) NES techniques. Such information will be signalled to NES-capable UEs. |  |
| Samsung | Not sure | We have tried before but seems not be able to decide definitions at this stage. But what we understand is that “there is a cell which supports an NES technology” and it could have three different status: - The cell is activated and executing the NES technology for saving power (e.g., cell DTX/DRX is activated and the cell is in non-active time duration) - The cell is activated but not executing the NES technology (e.g., cell DTX/DRX is activated but the cell is in active time duration) - The cell is deactivated the NES technology. |  |
| Sharp | comments | OK to have terminology, but seems it is not easy. NES techniques or something like that might be used for the time being. The detailed description could be further discussed based on progress. |  |

2.3 How often is a NES Mode changing (ON <-> OFF)?

Here are excerpts of what some companies think about NES mode/ status change (changing quickly or rather stable):

Nokia Observation 1: NES mode changes can be frequent and relying on regular HO is not practical Observation 4: In case NES mode of target cell is not frequently changing then NW can updates CHO configuration to UE based on neighbour cell information exchanged via gNBs. Observation 5: If NES modes are very frequently changing exchanging this information between gNBs is not feasible. Intel Observation#8: The candidate PCells may apply the NES techniques at any time and this may change quite frequently (e.g. Cell DTX/DRX may be turned on and off dynamically). Hence Option 1 (target NES is indicated in CHO configuration) is not suitable. Sony Proposal 1: Network should notify the UE to start performing NES CHO execution conditions when the NES mode of source or candidate cells is going to change or has changed. NEC Proposal-5: Legacy CHO configuration update procedure is used to notify the UE the change of NES mode of the CHO candidate cell. Apple Observation 3: During discussion of study item phase, there were two concerns for the idea of CHO enhancement based on target cell NES mode: The neighbor cell NES mode may change dynamically, and thereby the UE may need to read SIB of the neighbor cell to identify its NES mode. Interdigital Proposal 1: The change of the serving cell’s NES mode can be used as a new CHO trigger. FFS whether L1/L2 signalling indicating an upcoming NES mode change is needed for the trigger.

The rate of change of NES mode (e.g., DRX/ DTX ON <-> DRX/ DTX OFF) may affect RAN2 solution design. If cell NES mode changes very often e.g., changes in milliseconds level or even 10s of milliseconds level is possible then RAN2 would look for a more dynamic signalling to inform UEs about the same but if a cell’s determination for power saving is based on more stable long term statistics then it is likely that once deciding to turn NES mode to ON/ OFF, the same will continue for longer time e.g., seconds or even minutes. Companies with former view may assume that energy saving can be done throughout the day/ night whereas companies with latter view may assume that energy saving mainly come from non-peak hour traffic when number of RRC Connected UEs is limited. In the below question, Rapporteur knowingly avoided a “middle-ground” option e.g., “every 100ms”, which may not help then which way to go for later discussions.

Question 3: How dynamic is an NES mode change of a NES capable cell in your view?

- Option 1: NES mode may change every 10s of milliseconds

- Option 2: NES mode may only change slower and once turned on/off remains so for seconds or longer (minutes/ hours)

| Company Name | Option | Comments |
| --- | --- | --- |
| Xiaomi | With comments | There is 2 option to explain the NES mode change: Option a: from/to cell DTX/DRX active state to/from cell DTX/DRX non active state. Option b: the cell DTX/DRX is configured/activated or the cell DRX/DRX is not configured/activated. For option a, we think option 2 is reasonable. For option b, it may be several hours. |
| Huawei, HiSilicon | Option 2 | We think the discussion of “source cell NES mode” and “target cell NES mode” should be separated. For source cell NES mode, we think the only beneficial scenario is the cell switching off (e.g., operator may want to switch off the cells during non-peak hours to achieve cost savings). For target cell NES mode, we think it mainly includes the NES techniques that are being discussed in the WI, e.g. cell DTX/DRX. But the eventual (C)HO decision is up to source cell implementation. |
| Apple | Both are possible | Option 1 can't be precluded at this stage because dynamic L1/L2 cell common / UE specific signaling to activate/deactivate Cell DTX/DRX is still on the table. |
| Intel | Both | Both Option 1 and 2 look dynamic to us. In our understanding, the application of NES techniques duration can last for 10s of milliseconds or a few seconds (or longer) and both should be supported by specifications. |
| Vodafone | Option 2 | I would also agree that cells will not change their mode of operation very fast and once changed it will remain for a while. The main point if we consider Cell DTX/DTX is that the load is not changing very dynamically and in my assumption, there will be any way a timer/threshold to control it is not switched off/on in milliseconds. For the case, the cell is going to be turned off, I think the cell will stay off at least for minutes |
| Nokia | Most likely option 2 | But it is not possible to omit option 1 now – RAN1 may come with some super frequent NES methods updates. But e.g. turning of cell is not happening every few milliseconds but more or less minutes (at most). Cell DTX/DRX may change quite often depending on the load but even that does not seem necessary to change every few tens of milliseconds. . |
| Qualcomm | 2 | CHO itself takes a while to complete including backhaul signalling so option 1 is unrealistic and generally RAN2 better stick to realistic mobility assumptions |
| OPPO | Option 2 | In our view, the change would be decided based on the statistics of system-level information or cell status, so the change may not be very frequent. |
| Fujitsu | See comments | The definition of NES mode is not stable, but we assume it is only for Cell DTX/DRX case. For Cell DTX/DRX, it depends on the traffics scenarios and it would be dynamically changed. Then for network energy consumption perspective, it is not efficient to change the NES mode few seconds later. In addition, as Cell DTX/DRX pattern is common for all UEs, we should assume it may be changed in the order of milli-seconds. For the case of cell turning off, we think once the cell is turned off, it would be turned on after few minutes or longer. |
| Google | 2 | From RAN2 perspective option 2 should be the main focus. |
| Sony | Both | We think it’s important to support fast switch of NES mode as it will allow flexibility/potentially greater energy saving from network point of view. |
| Lenovo | Option 2 | This email discussion is only for DRX/ DTX and Cell Switch off techniques – here we do not see network reconfiguring a DRX/ DTX every (some) milliseconds. |
| Ericsson | Both, with comments | The actual NES operation of e.g. a target cell may happen quite frequently/dynamically. However, NES-based CHO is configured for longer term conditions, based on the fact that e.g., a target gNB is in a situation/condition where NES can be used the coming seconds/minutes/hours. |
| T-Mobile USA (TMUS) | Both | In case we want coverage layer to be benefitted from NES techniques such as Cell DTX/DRX; also to align UE CDRX cycle with Cell DTX/DRX, it should be in ms; otherwise, it can be much longer. |
| InterDigital | comments | We think it is more important discuss whether the change is deterministic or not, rather than focusing on how periodic the NES change happens. NES is activated when cell resource load become low on average (which is not expected to change very frequently), but it is not deterministic, like the question suggests. |
| ZTE | Both | For Cell DTX/DTX, we may have similar views as Apple, Nokia, Fujitsu, Sony etc. With consideration on the trade-off among different load situations, impacts on the UE traffics etc, very frequent “DRX/ DTX ON <-> DRX/ DTX OFF” switch may be possible. However, as mentioned in Q1, we assume no handover is needed in this case. We also agree with some above comments that, for the case of cell turning off, interval for the “Cell ON <-> Cell OFF” switch may be a bit long, e.g., few minutes or longer. In this case, HO/CHO may be needed. But as HO/CHO is triggered in such longer term conditions, we assume legacy HO/CHO may be sufficient. |
| NEC | Option 2 | NES change requires a change in SI. As fast as it might be with lower layer triggers, it should still be slow, in the 100s of ms. |
| Samsung | Both | Of course we would like to have seconds of cell sleep mode but due to the previous agreements that NES shall not modify SSB/ PRACH for IDLE mode support, now it is natural to understand that NES mode may turn on/off within every 10s of milliseconds. So we cannot preclude option 1. And in order to align cell DTX and UE C-DRX, seconds of inactive duration seems not possible. (or we could just handover the UEs before cell DTX inactive duration?) |
| Sharp | Both | Option 2 is more reasonable, and Option 1 may be also kept waiting for further input from RAN1. |

- Source side

We start with a source side view where the source cell determines if it can save energy, and when so it starts handover preparation if otherwise the service maintenance to the UE will not be possible anymore without impacting the required QoS, eventually transmits a conditional RRC Reconfiguration message to a RRC Connected UE.

In this line, all companies agree to use CHO procedural framework for configuring one or more candidates, a vast majority of companies propose that some enhancement(s) for CHO procedure will be required, including likely new signalling – some companies [1, 12, 21 and 23] have concerns on pursuing enhancements. So, just to ensure that we are on the same page with respect to the work we need to do to fulfill parts of the WID, following question is being raised:

Question 4: Do companies agree that some kind of enhancement in CHO procedure, including likely new signalling in conditional RRC Reconfiguration message would be required when either source cell or at least one of the candidate cell is in NES mode?

| Company Name | Yes/ No | Comments |
| --- | --- | --- |
| Xiaomi | No | For Rel-18 NES capable CONNECTED UE(s), no matter the source gNB or targe gNB, the network can serve the UE with enhancements to ensure the performance. For legacy CONNECTED UE(s), no matter the source gNB or targe gNB, it is up to network implementation to serve the legacy UE with performance degradation. So it is not necessary to handover the UE from the cell even if the cell will enter NES mode. Furthermore, when the network decides to configure the cell DTX/DRX for NEs purpose, there are no many active UEs on network and the service of the UEs is also not so active. So it is not urgent to handover the UEs to other non-NES cell and the group based handover is also not necessary. |
| Huawei, HiSilicon |  | It depends on the exact enhancement. We cannot have an agreement now without going to the details of the solution. Also, we have the following comments on the rapporteur descriptions: 1) “In this line, all companies agree to use CHO procedural framework for configuring one or more candidates” No we think legacy HO is also feasible when the source cells determines it can save energy and plans to handover some UEs. 2) This is under the discussion of source side, why “candidate cell” is mentioned in the question? |
| Apple | Yes | First, the benefit of CHO enhancement was extensively discussed in SI and the final conclusion is captured in TR 38.864. Not sure why repeating the same discussion in normative phase: During the switching of NES modes, it is possible to handover the UEs faster by enhancing the CHO framework with: 1. Evaluation of conditional handover conditions depending on the NES mode of source/target cell, 2. How to indicate to UE the triggering of the CHO evaluation is up to the WI phase. Secondly, we think RAN2 need to respect WID objective agreed in plenary: Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] Please note that it is NOT a conditional objective (i.e. without "if necessary"). So, more or less, RAN2 should do some work to fulfill this objective. Finally, the use case of Rel-18 NES is "low load" rather than "low number of UE". It is a valid scenario that a large number of UEs are served by source cell but they have few traffic (e.g. midnight of community). |
| Intel | Yes |  |
| Vodafone | Yes | We believe the enhancements are needed for the source cell case. Then we are fine to discuss the case, the source cell is going to turn off and the source cell is going into cell DRX/DTX mode separately and see if the mechanisms applicable for one case, could be applicable for other. |
| Nokia | Yes | Agree with Vodafone comment. Most likely CHO source method needs to be discussed for each “NES method” separately in the end although the general principle is same e.g. trigger for event can be different. |
| Qualcomm | Yes | Obviously we have a decent baseline which is RRC unicast signalling and existing HO framework. However, we think that unicast RRC signalling takes too much time and causes too much overhead that the NW may prefer not to switch off to avoid the time delay and intensive RRC signalling. To give an example of the benefit, this scheme is best understood as something that can be co-deployed with cell DTX/DRX. Consider the case where the NW starts deploying some form of cell DTX/DRX at 50% load and consistently sees less and less load until it decides to offload all UEs and enter “source cell CHO mode” where “source cell CHO mode” means the cell would physically be switched off, i.e., no UEs can camp or connect. In this case, we can assume the UE is applying a very aggressive cell DTX/DRX cycle right until CHO. Now to offload all UEs via L3 HO, the Cell needs to do one of two things: 1. Perform very slow RRC signalling over multiple cycles to offload all UEs via RRC signalling. 2. Deconfigure/deactivate Cell DTX/DRX to quickly perform L3 HOs for all UEs in the NW. In this case, our argument is to bypass this L3 HO to every UE in-order to perform a HO operation by enhancing the CHO to allow for faster triggers. We understand that many companies have concerns over the perceived benefits and encroaching into the LTM territory, thus, it would be fair to use this email discussion to converge on a solution then we can rediscuss if the NES gains are worthwhile. |
| OPPO | Yes | It is a straightforward way to implement what we concluded during the SI phase and the objective approved in NESWID. |
| Fujitsu | Yes | To maximize the energy saving efficiently, we support to enhance the CHO procedure for source cell case. |
| Google | Yes | We share the same view as Vodafone. |
| Sony | Yes |  |
| Lenovo | Yes |  |
| Ericsson | ? | It should be noted that also NES UE in NES cell may move around be handed over neighbor cells using legacy means, HO or CHO. If there is also NES specific CHO as SOURCE cell may enter NES mode the candidate target cells are likely the same. There is no point to make UE evaluate the neighbor cells twice or double the configuration. Especially that it is stated that NES CHO should configure much lower RSRP threshold in order that there are suitable targets when NES trigger is applied. These issues need to be considered. |
| T-Mobile USA (TMUS) | Yes | Agree with Vodafone comments. |
| InterDigital | Yes | Agree with Apple. This is an WI objective, so no need to re-discuss it. |
| ZTE | No | As we assume HO/CHO are mainly needed for the case of cell turning off, while not needed for the case of cell DTX/DRX, we see no clear motivation to enhance them till now. Moreover, as there may be similar discussion in some other topics, e.g., mobility enhancements, it’s better not to spend too much time discussing this issue in our NES topic. |
| NEC | Yes | To clarify our position as one of the companies cited, we do see benefit in having CHO enhancements. Our main concern is regarding lower layer enhancements. |
| Samsung | Yes | We agree to enhance CHO procedure for NES mode of serving and candidate cells. |
| Sharp | Yes | Both source cell and target cell cases should be considered for CHO enhancements, and signaling needs to be enhanced. |

3.1 When to start CHO condition evaluation

Going to the next logical part of the discussion, we will touch upon when to start CHO condition evaluation at the UE side when either source cell or at least one of the candidate cell is in NES mode. Some proposals have been made in this regard:

Apple Observation 2: If UE group common L1/L2 signaling is introduced, RAN1 needs to design a new UE group common DCI, but RAN1 is not involved in the WID objective of CHO enhancement. Proposal 5: The basic procedure of CHO enhancement based on source cell NES mode is described as below: Source cell configures both legacy CHO condition and NES specific CHO condition to a group of Rel-18 NES capable UE(s) via legacy UE dedicated CHO command. Upon reception of CHO command, the UE immediately starts to evaluate legacy CHO condition, and stores the NES specific CHO condition (i.e. not evaluation of NES specific CHO immediately). Proposal 6: RAN2 discuss the following 2 alternatives on how the UE detects the source cell enters NES mode: Alt-1: via detection of signaling to apply one or more NES technique(s), e.g. RRC signaling to apply Cell DTX/DRX or SIB to indicate legacy UE barring. Alt-2: via reception of a UE group common L1/L2 signaling from gNB CMCC Observation 4: Timer-based event for NES events is useful only if the gNB has accurate prediction/information of the cell load. Proposal 5: If the CHO execution condition is not timer-based event, once UE is indicated the NES mode or cell on/off state switch, the indication can trigger the UE to perform handover immediately. Lenovo Proposal 5: UE should start the evaluation of the candidate cell upon reception of CHO. NEC Proposal-10: The CHO configuration for NES can include a timer-based condition, where the timer starts upon triggering the CHO condition evaluation. Intel Observation#7: Using dedicated/group common L1 signalling (e.g. DCI) provides a fast and in timely manner for the indication to indicate NES technique(s) is to be applied or the source PCell is to be turned off. Vivo Proposal 4: If T1 event is used as CHO execution condition for solving handover storm, the UE determines T1 event based on the preconfigured relative time offset and the reference time provided via the explicit trigger. ZTE Corporation, Sanechips Observation 1: A time based CHO triggering event could be used for triggering handover before DTX/DRX in source cell would cause the large delay of transmission. Qualcomm Proposal 2: To realize this source cell CHO, two aspects need to be enhanced over legacy CHO: - RRC configuration of CHO is extended to include the required behaviour, i.e., allowing for CHO that is explicitly triggered by signalling. - L1/L2 trigger is introduced to invoke a preconfigured CHO procedure for NES purposes. - Possible introduction of a CHO time-to-execute T that can be used to instruct the UE to: - Start evaluating CHO conditions after a time delay T. - Perform CHO to the best target cell after a time delay T. Proposal 3: RRC CHO configuration is enhanced to include a new NES-CHO configuration that can be performed upon receiving an L1/L2 trigger from gNB and optionally after a time T Huawei, HiSilicon Proposal 2: For L1/L2 triggered CHO, the benefits (e.g. faster HO) should be further justified, in the context of network energy saving. Oppo Proposal 1: RAN2 considers either of the following on how to trigger CHO execution due to the cell off/NES of the source cell. • Alt1: A UE executes the CHO once it is the time for the source cell to enter cell off/NES. The time information of the source cell can be pre-configured to the UE. • Alt2: A UE executes the CHO once it receives a specific L1/L2 UE group common signalling.

We can carry this discussion in two parts:

First, a UE may need time to evaluate HO condition(s) for included candidates, attempt to execute handover and even to possibly fallback (to source or reestablish connection) before the source cell would/ should actually start to sleep. So, CHO evaluation should start at Point A (somewhat before point B) in the below figure. In a first option, Point A can be chosen by the network and coincides with reception of CHO reconfiguration at the UE. On the other hand, if CHO evaluation is triggered at point B when the source cell is about to enter sleep then there will be time period until which the UE will not have service due to impending condition evaluation, handover execution etc.

Question 5: Do you think that handover condition evaluation for any candidate cell starts sometime before source cell enters sleep/ inactive time?

| Company Name | Yes (= Point A)/ No (= Point B) | Comments |
| --- | --- | --- |
| Xiaomi | Yes | The UE evaluate the candidate cells after the reception of the conditional RRC Reconfiguration message, i.e., no enhancement on CHO for NES. |
| Huawei, HiSilicon | Yes, but | Yes but we don’t think source cell DTX/DRX is a target scenario, we prefer to change “Cell DRX/DTX” in the figure to “Cell switching off”. |
| Apple | Yes | We think that the UE shall follow legacy CHO behavior before entering NES mode (i.e. start evaluation of the candidate cells upon reception of CHO config). And if CHO condition is satisfied, the UE shall handover to another cell (i.e. follow legacy behavior). However, this doesn't preclude enhancement for below new NES specific CHO scenario: - The source cell enter NES mode (e.g. turn off) but there is no candidate cell satisfying CHO condition For this scenario, we think a simple enhancement is that the UE can apply a relaxed condition of CHO evaluation (e.g. an offset of threshold for configured CHO A3/A5 event), so that the CHO condition can be easily satisfied. |
| Nokia | Yes | Agree with Xiaomi (and we don’t follow rapporteur “sleep” reference? Is it generally any NES mode) |
| Intel | Yes (Point A), but see comments | Firstly, we note there are two ways to model this. In our understanding, the model used in this email discussion is to do the evaluation only at that the time of the trigger in Q6. Another model (that we were using in our contribution) would be to do the evaluation from the time of reception of the CHO configuration but only perform the execution at the time of reception of the trigger. Both models are OK but it would be good to understand the model being used in relation to the comments. Our comments below are based on the assumption that the model used in this email discussion is as described above. While we don’t think evaluation and HO execution takes very long, using point A to be a little earlier than the actual ctivating of the NES will be useful to ensure all the UEs have been handed over. We also believe that CHO configuration needs to be sent in advance for the following reasons: - Prevent surge in sending CHO configuration at the point where network decide to perform NES technique (this can be avoided via sending the CHO configuration in advance and spread over time) ii. Avoid delay in the preparation of the handover (sending the CHO configuration in advance) if decided at the point where network decide to perform NES technique iii. Avoid delay in the sending of the CHO configuration due to sending at the same time, not just UE RRC processing |
| Vodafone | See R2-2303161 | We are not sure, why our document is not listed here. In our understanding the additional conditions (in our view, it is just the best RSRP level of the cells configured for CHO) associated with NES mode (switched off or entering Cell DRX/DTX mode) are evaluated once RRC message is received. |
| Qualcomm | Yes | In our view, after point B (at which the cell should just sleep as there is no point to this ON-OFF cycle except confusing UEs) does not make sense, so point A is the better option. The more interesting question, if a trigger is agreed, whether the UE needs to evaluate CHO conditions before the trigger is received. The question depends on whether an NES CHO configuration doubles as a normal CHO configuration that the UE is evaluating anyway. |
| OPPO | Yes | In our view, the UE can evaluate the candidate target cells upon receiving the enhanced CHO configuration, i.e. Point A. The UE would execute the CHO once the source cell is judged to enter cell off/NES mode (i.e. based on a T1-like event but which is associated with the source cell NES/off time duration) or the additional triggering signalling). One follow-up question: if there is a long gap between Point A and Point B and if a suitable target cell has been evaluated/found before Point B, e.g. based on RSRP-related CHO execution condition contained in the enhanced CHO configuration, whether the UE needs to hand over to that suitable target cell directly? |
| Fujitsu | Yes, but | We think a triggering of CHO evaluation is not limited to cell sleep or inactive period cases. |
| Google | Yes | Especially for the condition ‘condEventA4’, it needs to be deactivated upon being configured and then be activated sometime before the source cell enter a non-active period. |
| Sony | Yes |  |
| Lenovo | Yes | In order to ensure time for evaluation, handover execution and possible fallback. |
| Ericsson | yes | We don’t see reason why we should have different approach to evaluation than legacy CHO. It should be however ensured that “normal” mobility is ensured and that it is possible to configure UE with normal CHO as well. |
| T-Mobile USA (TMUS) | Yes |  |
| InterDigital | Yes, but | The question is only focusing on the cell DTX case, where cell turn off should also be considered for this. |
| ZTE | Yes | Agree with Huawei. |
| NEC | Yes |  |
| Samsung | Yes | We agree for Point A in the same manner that the evaluation of CHO must start before triggering CHO. To trigger CHO at B, evaluation must start before B. |
| Sharp | Yes | Evaluation should be before the actual cell off and/or cell DRX/DTX. |

Figure 1: Point A appears sometime before cell starts to sleep (Point B)

Second, if you chose point A for the above question, a second option to define Point A is by using a later trigger after the reception of the conditional RRC Reconfiguration message e.g., timer based (where timer value is signalled in CHO i.e., in conditional RRC Reconfiguration message) or L1 L2 signalling or broadcast signalling. In this option, the conditional RRC Reconfiguration has been sent to the UE sometime before point A. While generally network can choose a “right time” to transmit conditional RRC Reconfiguration to the UE, a separate point A would mean that there’s value in triggering CHO evaluation (i.e., point A) more dynamically – this may depend on your views on Question 3, discussed previously.

While “timer-based” may not allow such dynamic nature, as (and if) the timer value is to be included in conditional RRC Reconfiguration, L1 L2 based dynamic signaling can overcome this demerit. A broadcast based approach may need to overcome new hurdles to ensure SI changes are seen as-and-when and before modification boundary changes.

Accordingly, following broad options on “when to start CHO condition evaluation for NES triggering” can be seen:

  - Immediately upon receiving CHO configuration like in legacy

  - A timer based approach (in this case please also indicate how the timer value is signalled to the UE)

  - L1 L2 signalling

  - Broadcast signalling approach

  - Others (please clarify)

Question 6: Which of the above option on “when to start CHO condition evaluation for NES triggering” do you find as most sensible?

| Company Name | a/ b/ c/ d/ e |  | Comments |
| --- | --- | --- | --- |
| Xiaomi | A |  | Same as legacy CHO. |
| Huawei, HiSilicon | A, B |  | Legacy scheme (a) is of course feasible. Time-based CHO (b) is also in legacy spec. But before going for time-based CHO, we would like to check if companies have the same understanding that this applies mainly to the case when the source cell plans to switch off (e.g. during non-peak hours). |
| Apple | b, c, d (down-selection can be in future meeting) |  | We think b, c, d are solutions on the table, and RAN2 can further down-select among them. It is no hurry to make decision now because there are 5 remaining RAN2 meetings for NES. Please note that timer based approach (i.e. option b) also needs spec change at least for below highlighted part in TS 38.306: timeBasedCondHandover-r17 Indicates whether the UE supports time based conditional handover, i.e., CondEvent T1 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. And below issue needs further discussion: 1) Whether a UE which doesn’t support NTN can use event T1. 2) Event T1 is based on absolute UTC time. This is always feasible for NTN UE because they are always equipped with GNSS. However, if a NES UE doesn’t equip GNSS, whether such UE can apply event T1? |
| Intel | c (please also see comments) |  | Please also refer to our comments on Q5 on the modelling. In terms of final execution, we believe that L1/L2 trigger and the RF conditions of the CHO has to be met. Whether the RF evaluation starts at the time of the reception of L1L2 signal or from the time of reception of CHO command is a modelling aspect. |
| Vodafone | d |  | At least for the case cell switched off, the broadcast signaling is fully sufficient. We also do not think, there is any specific enhanced NES CHO configuration (and therefore we do not think Option A is applicable). The UE executes once of the CHO configurations received before once it receives the RRC message, choosing the best cell. For Cell DRX/DTX case, we should discuss it more, but also here, the RRC broadcast signaling should be sufficient. |
| Nokia | A |  | Quite confusing discussion. Why would we change legacy CHO evaluation. Only thing we need is to have additional trigger (in addition to radio condition) to trigger event in case “NES mode” is entered (I guess that is “sleep” mode mentioned by rapporteur?) |
| Qualcomm | a/c |  | Option a, if the configuration doubles as a normal CHO configuration, e.g., target cell is configured A3/A5 as a legacy CHO, but may apply a relaxed A3/A5 condition upon receiving an L1/L2 trigger. Option c, if the CHO configuration is only for NES-triggered CHO, i.e., UE does not need to evaluate target cell before the trigger. For the timer, we think it can be introduced (if need) between receiving the trigger and executing the CHO, so in this case the evaluation is done during the time the timer is running, not wait for timer expiry to start evaluation. |
| OPPO | a for evaluation, b/c for execution triggering |  | We may need to distinguish the CHO evaluation and CHO execution. In our view, the UE evaluates the candidate target cells upon receiving the enhanced CHO configuration, which is the same as the legacy. The CHO enhancement focuses on the new execution triggering, including e.g. time-based and signaling-based triggering. - If we apply time-based CHO execution triggering, which we understand is categorized into option b, the time duration is associated with the source cell. Such time duration reflects the time duration of the source cell NES/off and is contained in the enhanced CHO configuration. - If we apply signalling-based CHO execution triggering, the signaling is sent before the source cell is about to apply NES/off Answer to Apple: Generally, we agree that spec change is needed but its impact is limited. - No, we do not intend to require the UE to support NTN. That is why we think the time-based method is similar to CondEvent T1 but not the same. - It is correct, not all UEs support GNSS. We think we can use reference SFN to indicate the start time of the source cell applying NES/off. But, we are also open to other solutions. |
| Fujitsu | b, c |  | Option b can be useful for scheduling cell-off or carrier-off scenarios. We think it is beneficial to activate/deactivate Cell DTX/DRX by L1/L2 signalling, then option c is necessary to handover the UEs timely. |
| Google | b, c, d |  | It depends on which CHO evaluation condition is being discussed. If it is condEventA4 being discussed, we think UE should start the evaluation when b, c, or d is met. We can down-select among b/c/d at a later phase. If it is condEventA3/A5 being discussed, then we think the UE should start the evaluation upon receiving the CHO configuration (like in legacy). |
| Sony | c |  | UE will be configured with CHO configurations and will be triggered to start the evaluation from the network once it receives a trigger signalling e.g. L1/L2 signalling and this signalling could be group based. |
| Lenovo | A |  | We think the network can send the CHO reconfiguration at a “correct” time and then there’s no point in delaying the evaluation procedure any longer. The need for a separate L1 L2 signalling does not exist since the network will not need to dynamically change the NES mode – UE once handed over is with the target side – so L1 L2 signalling is really a one time affair from UE’s perspective and therefore the same can be considered triggered as part of CHO reconfiguration reception at the UE. |
| Ericsson | a |  | We believe that if NES specific threshold or triggering is agreed it needs to be done efficiently together with legacy CHO and normal thresholds that do not make UE to HO too early. This is possible to UE receives one configuration and evaluates candidate targets based on one configuration and only checks event entering condition based on normal or NES threshold. |
| InterDigital | c, d |  | UE should start CHO evaluation at the time of reception of the CHO configuration but only perform the execution at the time of receiving the triggers described in c or d. Option b needs some clarification as to whether the concerned time duration is indicating the amount of time the UE has to wait before starting to evaluate the conditions, or it is a window of time the UE during which the UE evaluates/executes the CHO. |
| ZTE | a, b |  | As we consider HO/CHO are mainly for the case of cell turning off, we think a, b are sufficient. |
| NEC | C or D | Both c and d would be useful depending on the network intention on NES. If the network wants to apply e.g. Cell DTX/DRX, (c) L1L2 signalling is useful for more flexible timing. If the network wants to handle both NES-capable UEs and legacy UEs, (d) broadcast signalling approach may works well. For example, the network wants to prevent the UEs from coming back and thus adjusts e.g. cell reselection parameters via SIB update, where broadcast approach may also work together. |  |
| Samsung | a,b,c,d | Support a and b for RRC based NES activation, Support c and d for L1/L2 signal based NES activation. |  |
| Sharp | b, c, d | CHO conditions for NES can be configured in advance, and the evaluation time point can be an explicit indication or an implicit timer based method. |  |

3.2 Measurement Events

Which events are used as conditions in a CHO configuration? Following events have been mentioned:

- A3 [7, 11]

- A4 [1, 7, 11, 13, 20]

- A5 [7, 13]

- A new condition “NES trigger” attached to these events [7]

- A new trigger [2]: “off duration of DTX/DRX configuration in the NR PCell is higher than a threshold1 and off duration of DTX/DRX configuration in the neighbor Cell is lower than a threshold2”

Question 7: Which of the above event(s)/ condition(s) can be used in your opinion as conditions in a CHO configuration for each/ some of the candidate cells included?

| Company Name | Option(s) | Comments |
| --- | --- | --- |
| Xiaomi |  | Same as legacy CHO. |
| Huawei, HiSilicon | A3, A4, A5 | condEvent A3 and condEvent A5 are already in the current spec. condEvent A4 is also in current spec with the restriction that it applies only to NTN or CPA and MN-initiated inter-SN CPC in terrestrial network. |
| Apple | A3, A4, A5 | See no reason to preclude any existing CHO event. |
| Intel | b)+ d). That is, A4 and new NES trigger condition | As mentioned in our response to Q5, we believe CHO configuration has to be provided in advance of the cell activating the NES technique or Cell off. As it is related to inter-frequency CHO and the serving cell may still be in good radio condition, Event A4 is needed to allow the UE to move other cell. But we also need d) to time the execution of the CHO with the cell turning on NES. |
| Vodafone | A3,A5 | We do not propose to enhance trigger events for CHO, but we need a trigger from the NW (see my previous comments) |
| Nokia | Any | We are fine to have any event – Unless there is issue for some specific event to introduce it but we don’t see now. So basically we would question that if source cell based CHO NES mode is specified why would we not allow it for any event (if it comes for free)? |
| Qualcomm | d | A cell switching off would inform the UEs via some L1/L2 trigger to execute one of the CHO configurations available. UE can perform a step of evaluation of A3-A5 before executing CHO, but our view is that the trigger would be the enhancement over legacy |
| OPPO | A3, A4 | A3/4 if we are talking about radio link-related measurement. If the question also covers other aspects to be measured, e.g. measure whether it is the time to enter source cell NES/off, we would also like to include time measure, and are open to discuss others. |
| Fujitsu | A3, A4, A5 | Using A4 is baseline, but there is no reason to exclude legacy CHO events. |
| Google | A3, A4, A5 | No reason to preclude any of the existing events. |
| Sony | - b) c) d) |  |
| Lenovo | A3, A4, A5 | Further down selection can be done in coming meeting, if required. |
| Ericsson | A,b,c | D option is not clear, does it include NES specific threshold or not? E is too complicated |
| T-Mobile USA (TMUS) | d |  |
| InterDigital | A3, A4, A5 |  |
| ZTE | b(A4), e | In the case of cell turning off, when the HO/CHO need to be triggered, the radio quality of serving cell may still be good. Therefore, we think A3 and A5 are not suitable. As proponent, for e, we assume it’s only needed only if we confirm that cell DTX/DRX will obviously deteriorate UE’s performance and HO/CHO would be needed. |
| NEC | A3,A4 | Legacy events of A3 and A4 may be used as conditions in a CHO configuration for UE evaluation. |
| Samsung | a,b,c,d | We support d) and understand such condition could be jointly configured with a), or b), or c). We do not see the need of e). |
| Sharp | A3, A4, A5 | The current CHO events are all possible for NES CHO, and may have relaxed thresholds compared with legacy CHO. |

- Target side

On legacy handover condition(s) fulfillment towards a candidate cell for conditional handover, there’s no visible difference of opinion among companies. However, for new NES state related enhancement/ aspect, following options are brought forward by companies:

- UE implementation [Nokia]

- Network provides additional prioritization for candidate cells [Fujitsu, Apple, OPPO]

- Choose candidate(s) with same NES mode as source cell [Fujitsu]

- Source cell obtains the candidate cell NES state and source cell implementation to utilize the candidate cell NES state to configure suitable candidate cells to the UE [HW]

- Source Network provides NES state flag/ information of candidate cells [QC, Ericsson P2, Sharp, Fujitsu, NEC, IDT, Lenovo, CMCC, OPPO]

  - DRX/ DTX configuration for each candidate cell in CHO command [Lenovo]

NOKIA Proposal 1: Add for events A3, A4 and A5 a additional parameter that indicates that event is triggered only if “NES trigger” is active for the source cell. Proposal 2: “The NES trigger” would be at least for the use case of turning off the cell (whether other triggers are enabled is FFS and need to wait that WI progresses on other aspects of the WI). Proposal 3: It can be left up to UE implementation to select target cell out of multiple candidate CHO cells. Fujitsu Observation 7: It is useful to select the target cell as the same cell mode as the source cell. Proposal 5: For target cell case, the priority information is additionally provided by the source cell. Apple Proposal 7: For CHO enhancement based on target cell NES mode, introduce a gNB configured priority value for each candidate cell based on their NES mode. The UE is not required to detect NES mode change of the candidate cell(s). Ericsson Proposal 2: Enhance CHO procedure to enable priorization of candidate target cells by the UE based on NES mode.

4.1 Finding right target cell

There was some initial online discussion on this from the last [#121] meeting. From some of the contributions it is not fully clear how a sensible UE implementation makes consistent decision on mobility without any information provided by the source and/ or reading system information from the candidate cell – which is generally avoided for handover execution.

Further, it seems a majority of companies believe that some sort of information/ aid from the source cell needs to be provided to the UE to help it decide on a target cell among candidate cell(s) that have fulfilled handover condition(s) included in conditional RRC Reconfiguration message.

While additional prioritization per candidate cell can be easy from UE implementation perspective, it is unclear how network would prioritize e.g., two cells on the same frequency that are in NES mode but have very different duty cycle and/ or how would network prioritize among cells where some of the cell is/ are in “active” time but the others are sleeping – at the time of sending the conditional RRC Reconfiguration message to the UE!

As another option brought forward by some company “Choose candidate(s) with same NES mode as source cell”, is simple and requires only a Boolean indication per candidate cell but suffers from the same demerits as for the previous option. The same is perhaps also true of “NES state flag” like proposals.

However, if the UE were to be provided with DRX/ DTX configuration of each candidate cells, where the offset is aligned with the source cell’s downlink timeline, UE itself can plot the DRX/ DTX cycle of a candidate cell and see if the cell is sleeping or active “now”.

Question 8: Which of the following option do you think will ensure better chances of UE finding service immediately after executing conditional handover?

- UE implementation

- Network provides additional prioritization for candidate cells

- Choose candidate(s) with same NES mode as source cell / Source Network provides NES state flag of candidate cells

- DRX/ DTX configuration for each candidate cell in CHO command and implicitly/ explicitly NES mode of source as well as included candidate cells

- Network can provide a subset of CHO candidates as part of the CHO trigger

- Network implementation to (re)configure the candidate cells

| Company Name | a/ b/ c/ d/ e/ f | Comments |
| --- | --- | --- |
| Xiaomi | a) with comments | If the question 8 is based on CHO case, then I think it is not possible to configure the NES mode of target cell in CHO if the NES mode changes frequently. So option a) is feasible. For legacy HO case, it is up to RAN3. Anyway, the target can serve the UE no matter the UE is NES capable UE or legacy UE. If the target cell think it cannot meet the Qos of the UE , the network can leave the NES mode. |
| Huawei, HiSilicon | f | As indicated in our paper R2-2303102, we think it’s better to leave it to NW implementation. If the source cell does not filter the candidate cells by the NES mode and configure more candidates cells for UEs to choose from, all the configured candidate cells need to reserve resources, especially considering the candidate cells may be an NES cell whose main motivation is to save power and avoid excessive wireless resources consumption. |
| Apple | a) | We can leave it to UE implementation. |
| Intel | e) | If multiple candidate cells satisfied the execution condition for NES, it can be left to the UE implementation, but we are also fine if some priority is provided for this case. |
| Vodafone |  | NW should ensure the Handovers do not take place to the cell going to be switched off or where cell DRX/DTX is switched on. No enhancements for this case are needed in RAN2. Moreover the target NW should not configure CHOs once in Cell DRX/DTX mode or going to be switched off. |
| Nokia | a/f | A might be needed if there is possibility to have multiple candidate target cells in CHO events. We are also OK to limit only to one. Then no need to have A => then it means network needs to apply f) solution in some scenarios |
| Qualcomm | a+b | When CHO conditions are true for multiple cells, the UE can leverage information from source cell about the NES mode of the target cell to select a target cell. We don’t think this should be hard-coded into CHO configuration so as to not to complicate UE implementation of CHO evaluation by enforcing too many checks before/during CHO, but we foresee this as useful information for UE implementation in target cell selection. |
| OPPO | b/d/a | b/d has a benefit if more than one cell fulfils the CHO execution condition. But a is also acceptable to us. |
| Fujitsu | a and b | In general, NW tries to avoid configuring CHO candidate cell which is mismatched with the UE’s QoS. In addition, NW can provide the priority information to reduce the QoS mismatching and the UE respects this priority information. However, it is up to UE implementation how to use this. |
| Google | f |  |
| Sony | c)/e) | We think this information would be beneficial for the UE to make the decision. |
| Lenovo | d or f | We do not see how a UE implementation can ensure that a target cell is not sleeping when the UE wants to execute handover towards it and send handover complete, without having to read SI of the target side! So, we prefer that source provides the necessary information – this is not really new considering e.g., CFRA from target side can be anyway signalled in the legacy HO command. |
| Ericsson | b | If more than one target cell fulfils the CHO triggering, UE should oprioritize based on network preference. Can be indication of NES mode of target, or a more general priority value which can be used as NES indication or for any load balancing. |
| T-Mobile USA (TMUS) | f | We believe it is better to leave it to network implementation |
| InterDigital | b, d, or e | it can be useful for the network to guide UEs to handover to a subset of candidates. If not needed, UE implementation is fine. |
| ZTE | f | We also think it’s better/sufficient to leave this to network implementation |
| Samsung | d) | We agree to rapporteur that RRC message based candidate cell’s NES mode configuration can be modified at each candidate cell via L1/L2 signaling. If a candidate cell activates/deactivates its NES mode, then should all the neighbor cells retransmit modified CHO trigger RRC message to all the UEs within the cell? We do not think this is the way to go. So basically we think UE needs to observe SIB of each candidate cell to determine whether the cell is currently in or activated NES mode or not. In case for NES mode of candidate cells to be included in RRC, we support d) the RRC message to carry the ‘pattern’ of NES mode at each cell, not the current status. |
| Sharp | b, c, e | Source cell can provide information to UE to help UE to know more about candidate target cells, and the detailed information can be further discussed. |

4.2 Failure case

Some companies mentioned that it may not always be possible to find perfect target cells, fulfilling not only handover condition(s) but also one that is in active time when the UE is about to execute handover.  What can be done in such situation? Some proposals were made as follows:

Ericsson Proposal 1: Network needs to know if there are no good enough candidate target cells for CHO at the time cell is going to deactivate or enter cell DTX/DRX. Proposal 3 If the UE is performing HO or CHO and experiences HO failure due to cell being in NES mode, UE should inform the network about the HO failure cause. Lenovo Proposal 7: RAN2 kindly discuss further how to handle cases when there’s still no perfect target or an acceptable target when the source cell is about to enter sleep. Vivo Proposal 5: After receiving the explicit signaling to trigger CHO, if the UE cannot find a candidate cell satisfies the execution condition (e.g., A4 event), the UE should notify the source cell within a certain time. Oppo Proposal 2: If CHO execution is triggered due to the cell off/NES of the source cell and if the network also configures the UE with the CHO execution event (e.g. A3 or A4), the UE needs to select the target cell from candidates for which the CHO execution event related to channel quality is satisfied. If such a CHO execution event cannot be satisfied for any of the candidate cells, the UE needs to choose the candidate cell with a better channel quality.

So, mainly two options emerge from the proposals made by companies:

- UE reports to source cell if there are no good enough candidate target cells for CHO at the time cell is going to deactivate or enter cell DTX/DRX. Rapporteur thinks that the source cell may possibly stretch its active time (i.e., postpone sleep when e.g., user is on a voice call with you know who…) or may just ignore the UE’s situation (e.g., if user is surfing 6G videos).

- UE chooses a candidate cell with best radio quality among all candidates evaluated even if the said cell has not fulfilled the stipulated radio condition(s). Rapporteur thinks that this may not avoid choosing a sleeping target though.

Question 9: If there are no good enough candidate target cells for CHO at the time cell is going to deactivate or enter cell DTX/DRX, the UE shall:

- Report the situation to source cell

- Choose the best among worst candidate cells

- The UE applies a relaxed condition of CHO evaluation (e.g. a threshold offset for configured CHO A3/A5 event). If still no candidate target cell satisfies the condition, follow legacy procedure.

| Company Name | Option | Comments |
| --- | --- | --- |
| Xiaomi | - With comments | The UE can report the situation to the source cell. |
| Huawei, HiSilicon | None | We think in this case, the legacy behavior should apply, e.g. UE triggers RRC re-establishment. Alternatively, the NW can configure RRM measurements to UE (this is common practice for RRC_CONNECTED mode UEs) and know there is no good enough candidate via the measurement report by the UE, and NW uses legacy HO command to handover the UE to a relatively good target cell. |
| Apple | c), b) can be further considered | We disagree a) because it can be achieved by NW implementation. Note that in legacy CHO, source cell can still configure measurements towards candidate target cells after CHO is configured. Therefore, based on UE measurement reporting, the source cell can always know the radio condition of candidate cells, and can do anything accordingly (e.g. postpone cell sleep). We have sympathy with intention of b), but it has 2 issues: - The best candidate cell may still have poor radio condition, so the mandating UE to choose such cell will result in RLF. - If source cell just activates Cell DTX/DRX with a long non-active duration, the UE can still stay in source cell if no candidate cell with good radio condition is available. Thus, we are fine to continue discuss b) and improve it. For c), we think it can resolve the issues with minor spec change: - It can resolve issue 1) of b), i.e. we still have a RSRP/RSRQ threshold to restrict UE to select candidate cell with poor radio condition. - It can also resolve issue 2) of b), i.e. source cell can (by its implementation) configure different threshold offset depending on whether it plans to tun off or activate cell DTX/DRX. - It is on top of existing CHO framework with minor spec change. |
| Intel | None | We think this will not be a normal case but a corner case. We can leave this to existing RLF handling. |
| Vodafone | See my previous comment | Happy R2-2303161 is also included into this summery as we clearly propose not to introduce any new mechanism for the target cell case |
| Nokia | None of proposed solutions | We tend to agree with Intel i.e. if no candidate cell fulfills condition then we can rely on basic RLF handling |
| Qualcomm | A | Obviously, we would like to avoid RLF for the UE based on that so we can discuss how the gNB can confirm that the UE has performed a successful CHO, e.g., either via UE signalling or legacy backhaul HANDOVER SUCCESS message from target cell |
| OPPO | b | The intention of option b is to provide an additional chance to switch the UE when the source cell is about to enter NES/off. In our view, sometimes the best cell among the worst candidates can serve the UE (temperately) and may hand over that UE to a more suitable cell if needed. If the best cell among the worst candidates is not good enough, the UE would trigger RRC re-establishment. On option c, we are open to further discussing it. |
| Fujitsu | None | If there are no good enough candidate cells, NW should not turn the cell off or activate Cell DTX/DRX. We think it is a corner case then no need to specify the UE behaviors for the failure case. Even if it happens, the UE simply reuses the legacy behavior, i.e. RLF and re-establishment. |
| Google | None | Agree with Intel. |
| Sony | None | Legacy procedure will be applied in this case. |
| Lenovo | A | - We think this is quite possible that a UE has not found a suitable target by the time the source plans to sleep. The fact that source plans to sleep does not alter UE’s geometry i.e., radio conditions. - RLF handling must be avoided – in numerous cases we have tried to reduce RLFs – these affect user experience. |
| Ericsson | A, c with comments | C is basically covered by the earlier questions. Normal mobility needs to be ensured where UE can go to neighbor cell even source did not enter NES mode and hence there is no specific trigger. But also that UE does not move there too early due to low threshold configured. A is another thing which we believe is needed for good network operation. |
| T-Mobile USA (TMUS) |  | We believe the coverage layer should benefit from NES Techniques as well; in case this is the coverage cell and there is voice call ongoing, Cell DTX/DRX can be delayed; or Cell DTX/DRX cycle can align with voice burst cycle to support voice service. |
| InterDigital | a), c) | Reporting using (a) can be useful for the source cell to avoid turning off and also avoiding RLF for some UEs that don’t have any alternatives due to the geometry. (c) ensures that some UEs have handed over to a good enough cells instead of triggering RRC re-establishment |
| ZTE | None | Similar view as Huawei and Intel. |
| Samsung | None. | If there is no candidate cell which is good enough to trigger CHO, then the CHO shall not be triggered. CHO is only for the quick handover within the network configured conditions met. Even if the CHO is not triggered, the UE will trigger HO or perform cell reselection to other cell so no need to introduce such new behavior. |
| Sharp | None | Similar view as Huawei and Intel. |

- References

- R2-2302764	CHO enhancement for NES	CATT, Turkcell	discussion	Rel-18	Netw_Energy_NR-Core

- R2-2302837	Further discussion on connected mode mobility	ZTE Corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core	Late

- R2-2302925	NES Connected mode mobility	Qualcomm Incorporated	discussion	Rel-18

- R2-2303077	CHO for NES	Ericsson	discussion	Rel-18	Netw_Energy_NR-Core

- R2-2303080	Handover enhancement for NES	Sony	discussion	Rel-18	FS_Netw_Energy_NR

- R2-2303102	Discussion on CHO enhancement for NES	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR

- R2-2303128	CHO on NES	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_Netw_Energy_NR

- R2-2303146	Discussion on CHO enhancements for NES	Sharp	discussion

- R2-2303161	Triggering conditions and other aspects of the Handover to/from DTX/DRX cells	Vodafone España SA, Apple	discussion	Rel-18

- R2-2303259	Discussion on Connected mode mobility for network energy savings	Fujitsu Limited	discussion	Rel-18	Netw_Energy_NR-Core

- R2-2303311	Discussion on connected mode mobility	OPPO	discussion	Rel-18	Netw_Energy_NR

- R2-2303317	CHO procedure enhancement to support NES mode	NEC Telecom MODUS Ltd.	discussion

- R2-2303370	Discussion on CHO enhancement in NES	Apple	discussion	Rel-18	Netw_Energy_NR-Core

- R2-2303481	DRAFT LS for Enhanced handovers towards cells with activated cell DTX/DRX or cells which are going to be switched off 	Vodafone	LS out	Rel-18	To:RAN3

- R2-2303512	CHO procedure enhancements for NES	Intel Corporation	discussion	Rel-18	Netw_Energy_NR-Core

- R2-2303602	NES mobility aspects	InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

- R2-2303654	CHO Procedure in NES Mode	Lenovo	discussion	Netw_Energy_NR-Core

- R2-2303749	Discussion on Connected mode mobility for NES	Samsung	discussion	Rel-18

- R2-2303793	Discussion on Connected mode mobility enhancement for NES	CMCC	discussion	Rel-18	Netw_Energy_NR-Core	Late

- R2-2303824	Conditional handover enhancement for network energy saving	vivo	discussion	Rel-18

- R2-2303853	Discussion on UE mobility due to NES cell	Xiaomi	discussion

- R2-2304155	Discussion on CHO procedure enhancements in case source/target cell is in NES mode	Turkcell	discussion	Rel-18

- R2-2304180	Connected Mode Mobility	LG Electronics Inc.	discussion	Rel-18	Netw_Energy_NR-Core

- Conclusion

Point A and B are drawn in Figure 1