3GPP TSG-RAN WG2 Meeting #123	R2-2307106

Toulouse, France, August 21-25, 2023

Agenda item:	7.6.2.2

Source:	vivo

Title:	Discussion on GNSS Operation for IoT NTN

Document for:	Discussion and Decision

Introduction

In the past meetings, RAN2 has discussed the GNSS operation enhancement for R18 IOT NTN and achieved lots of agreements. In this contribution, we will continue to discuss the remaining issues for this topic.

Discussion

New MAC CE for GNSS operation

In the RAN2#122 meeting, RAN2 achieved the following working assumption as below.

Working Assumptions [1]:

1. 	An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution

2.	A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

For the UL MAC CE for GNSS validity duration reporting, the main concern is that the UL MAC CE may not be successfully received by the NW due to the possible maximum number of retransmissions on the NW side. However, this issue has gone through serval meetings and cannot reach a consensus. Based on our understanding, it can be solved by NW implementation. As the GNSS measurement gap is configured by NW, NW knows that UE has UL MAC CE for reporting the GNSS validity duration after the end of the GNSS measurement gap, and will try its best to guarantee the reliability of the UL MAC CE. Considering the limited time for R18 IOT NTN, to make progress, it is suggested to follow the RAN1 agreement and confirm the work assumption on the new UL MAC CE for GNSS validity duration reporting.

For the new DL MAC CE for GNSS measurement, RAN1 has achieved further progress in the meeting in May and obtained the following agreements. Considering RAN1 has done a lot of standard work, RAN2 should consider RAN1's conclusion and follow RAN1's design. Even though there are security concerns on MAC CE from some RAN2 companies, there is not a specific issue for GNSS measurement as several MAC CEs (e.g., DRX command MAC CE) in the current spec also will cause a UE to stop communicating for a long time. Thus, it is suggested to confirm the working assumption on the new DL MAC CE for GNSS measurement.

RAN1#123 Agreement [2] For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, down select one of the alternatives for the start time of the gap: - Alt 1: should be at n+ X, where n is the end of MAC CE receiving subframe/slot and X>= 12ms for NB-IoT, X>= 3ms for eMTC - Note: X is one value regardless of HARQ feedback enabled or disabled for the MAC CE - FFS: details, e.g. X is predefined value or configured value - Alt 2: should be at n+ X1, where n is the end of MAC CE receiving subframe/slot when HARQ feedback for the MAC CE is disabled and X1>= 12ms for NB-IoT, X1>= 3ms for eMTC, or should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled - FFS: details, e.g. X1 and X2 are predefined value or configured value, including whether X1 and X2 can be the same - Alt3: should be at p+ X, where p is the end of HARQ feedback transmission subframe/slot, where HARQ feedback for the MAC CE is always enabled - FFS: details, e.g. X is predefined value or configured value

Proposal 1: Confirm the working assumption that An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well and A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

For the new UL MAC CE for GNSS validity duration reporting and the new DL MAC CE for GNSS measurement, it is still under discussion in RAN1. For the detailed content of the MAC CEs, RAN2 should wait for further input from RAN1.

Proposal 2: The detailed format of UL MAC CE for GNSS validity duration reporting and DL MAC CE for GNSS measurement wait for more input from RAN1.

FFS on GNSS measurement

In the RAN2#121bis meeting, there is an FFS left for further discussion.

RAN2#121bis Agreement [3]

- UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry

During the RAN2#121bis email discussion, one company raised an issue that there will be a gap between the timer expiry and the start of the GNSS measurement gap if the GNSS measurement is started after the timer expiry and was wondering if such a gap really exists. Then, the above FFS is left for further discussion.

Before solving this FFS, we can look at the RAN1 agreement first. The related RAN1 agreements are listed below.

RAN1#112bis Agreement [4] On when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot - FFS: details of X, e.g. predefined value or configured value, considering HARQ feedback for the MAC CE, etc RAN1#113 Agreement [2] From RAN1 perspective, at least for the case when frequency error and timing error are within frequency and timing error requirements with legacy closed loop time correction, UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition. RAN1 will decide further details of the above.

Based on the RAN1 agreement, RAN1 has agreed that UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition. Based on our understanding, the duration X refers to the time gap between the reception of MAC CE for GNSS measurement and the start time of the GNSS measurement gap. Then, from the RAN1 perspective, the start of the GNSS measurement gap can be after the current GNSS validity duration expiry. Considering that NW has the idea of the remaining time of GNSS validity duration, it is up to NW implementation to configure the duration X to guarantee that UE starts the GNSS measurement gap before, upon or after the current GNSS validity duration expiry.

Proposal 3: GNSS measurement can be started before, upon or after the current GNSS validity duration expiry, which is up to NW implementation.

Proposal 4: UL transmission can be allowed in the time gap between the current GNSS validity duration expiry and the start of the GNSS measurement gap.

Collision between GNSS measurement and SIB31

For the collision between GNSS measurement and SIB31, RAN2 has achieved the following agreement. However, how to handle the timer T318 is still a question.

RAN2#121bis Agreement

- For a UEs that cannot acquire system information and GNSS position at the same time, acquisition of SIB31 may be postponed until GNSS measurement is completed if the UE cannot complete acquisition of SIB31 before the start of GNSS measurement gap

In the R17 IOT NTN, a guard time (i.e., T318) is introduced to avoid UE taking forever to acquire SIB31. The UE will declare RLF and initiate RRC connection re-establishment RLF if UE cannot acquire the SIB during T318. For the use case that UEs cannot acquire system information and GNSS position at the same time, if the acquisition of SIB31 may be postponed until GNSS measurement is completed, the timer T318 shall not run during the GNSS measurement gap. Otherwise, the timer T318 may expire during the GNSS measurement gap, which will trigger RLF. The other case is that UE has less time to acquire the SIB31 when the GNSS measurement is completed, which will against the original intention of the design of the timer T318.

Proposal 5: The timer T318 shall not run during the GNSS measurement gap if the acquisition of the SIB31 is postponed until the GNSS measurement is completed.

GNSS measurement failure

According to the RAN1#112bis agreement, the GNSS measurement gap can be larger than the latest UE-reported GNSS position fix duration. Then, there may be a case that UE successfully completes the GNSS measurement before the end of the GNSS measurement gap. Under this case, what‘s the UE behavior in the left time of the GNSS measurement gap is under discussion in RAN1. For this issue, RAN2 can wait for RAN1's conclusion. There also may be a case that UE unsuccessfully completes the GNSS measurement during the GNSS position fix duration. Under this case, what‘s the UE behavior in the left time of the GNSS measurement gap has not been discussed in RAN1. From the RAN2 perspective, UE can try another attempt for GNSS measurement until the end of the GNSS measurement gap, which does not need additional standard work. If UE cannot successfully complete the GNSS measurement until the end of the GNSS measurement gap, how to handle the GNSS measurement failure shall be discussed and solved. Based on our understanding, the GNSS measurement gap is the maximum time that allows UE to perform GNSS measurement in the connected state. Then, if UE cannot acquire the valid GNSS during the measurement gap, as legacy, UE shall go to idle state.

RAN1#112 Agreement On the length of GNSS measurement gap, which is aperiodically triggered by eNB, the gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration. FFS: whether the gap duration is configured by eNB, or the gap duration is equal to the latest reported GNSS position fix time duration.

Proposal 6：UE goes to idle state if UE cannot acquire the valid GNSS during the GNSS measurement gap in the connected state.

Conclusion

In this contribution, we discussed the GNSS operation enhancement for R18 IOT NTN. This discussion includes the following proposals.

Proposal 1: Confirm the working assumption that An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well and A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

Proposal 2: The detailed format of UL MAC CE for GNSS validity duration reporting and DL MAC CE for GNSS measurement wait for more input from RAN1.

Proposal 3: GNSS measurement can be started before, upon or after the current GNSS validity duration expiry, which is up to NW implementation.

Proposal 4: UL transmission can be allowed in the time gap between the current GNSS validity duration expiry and the start of the GNSS measurement gap.

Proposal 5: The timer T318 shall not run during the GNSS measurement gap if the acquisition of the SIB31 is postponed until the GNSS measurement is completed.

Proposal 6：UE goes to idle state if UE cannot acquire the valid GNSS during the GNSS measurement gap in the connected state.

Reference

- RAN2#122 chairman note.

- RAN1#113 chairman note

- RAN2#121bis chairman note.

- RAN1#112bis chairman note