3GPP TSG-RAN WG2 Meeting #123	R2- 2307135

Toulouse, France, Aug 21 - Apr 25, 2023

Agenda Item:	7.11.2.1

Source:	MediaTek Inc.

Title:	Control plane discussion for multicast reception in RRC INACTIVE

Document for:	Discussion, Decision

Introduction

In the last meeting, RAN2 made progress on PTM configuration structure, initial config, notification for session activation/deactivation for multicast reception in RRC INACTIVE. In this contribution, we further discuss the RRC resumption threshold and cause, the aspect of detailed UE behaviour for session activation and the MCCH monitoring time.

Discussion

RRC resumption

Resume threshold due to bad reception quality

In RAN2 #121bis, RAN2 reached agreements on UE triggered RRC resumption due to the bad reception quality:

UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

The intention for this threshold is used to detect whether UE is in a bad coverage area (e.g., cell edge) and needs to switch back to RRC CONNECTED state for higher reliability (e.g., HARQ feedback and PTP transmission). So the similar threshold (e.g., rsrp-ThresholdSSB-SUL) used in RRC INACTIVE state can be considered for detecting cell edge and reception quality.

Considering that the existing UE measurements for RRC INACTIVE are limited, the threshold type should be limit on existing UE measurement in RRC INACTIVE, i.e., RSRP and RSRQ, to avoid extra UE measurement requirement and RAN4 impact.

Hence, the proposal is:

Proposal 1: The threshold used to trigger RRC resumption is based on existing UE measurement in RRC INACTIVE, i.e., RSRP or RSRQ.

Resume cause for RRC resumption

There may be multiple scenarios when UE need to request RRC resumption for multicast reception:

- Scenario 1: UE triggered RRC resumption due to poor reception quality, as defined by the threshold discussed previously

- Scenario 2: UE triggered RRC resumption due to the MCCH unavailability after cell reselection in RRC INACTIVE state.

- Scenario 3: UE triggered RRC resumption due to the MCCH unavailability after receiving a session activation notification

- Scenario 4: NW triggered RRC resumption (via unicast paging or legacy group paging)

For network triggered RRC resumption (scenario 4), it is natural that the resume cause for RRC Resume should be legacy “MT-Access”, as this connection is initiated and scheduled by the network.

For UE triggered RRC resumption (scenario 1~3), no matter the exact reason for the triggering, network is not aware of the UE’s request until it receives the RRC signaling. Existing resume causes (e.g., mo-xxx) do not align with the intention for RRC resumption for multicast and are not beneficial for the network scheduling of the ongoing multicast session based on current system load. This is because network cannot judge whether UE is triggering RRC resumption for unicast or multicast service. If all UE trigger legacy mo-xxx for multicast, NW cannot afford the burden and do not know there are issues for multicast (e.g., MCCH unavailability).

Observation 1: Existing resume causes do not align with the intention for the UE triggered RRC resumption for multicast (scenario 1~3) and not beneficial for the network scheduling of the ongoing multicast session.

Besides, RAN2 has discussed the behavior that UE triggers RRC resumption and NW indicates the updated PTM config via RRCRelease message, then UE do not need to fully switch back to CONNECTED state. This behavior also needs the support of new resume cause for multicast.

Therefore, new resume cause should be introduced for multicast reception in RRC INACTIVE.

Considering the limited spare bits for IE resumecause and the various potential resume causes from UE (at least scenario 1~3), we suggest using a single bit in resumecause (e.g., mo-r18Multicast) to cover all UE triggered RRC resumptions for R18 multicast. No matter whether the resumption is due to MCCH unavailability or bad quality, network need to determine whether to allow UE to connect based on current system load for the cell. The detailed reason can be deduced by network implementation based on other information (e.g., a sudden increase in the number of resuming UEs could indicate an issue with the multicast MCCH)

Proposal 2: Use one bit in resumecause (e.g., mo-r18Multicast) to cover UE triggered RRC resumption for R18 multicast.

Session activation

In the past several meetings, RAN2 reached agreements to use group paging with enhancement to notify a session activation and can be received in RRC INACTIVE state:

- RAN2 #119bis: As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification). - RAN2 #121bis: Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS. - RAN2 #122: Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

Based on the above discussion, we can further discuss the UE’s behaviours when it receives a session activation notification. Given that multicast reception in RRC INACTIVE is important for the high congestion scenarios, and considering that simultaneous RRC resumes by many UEs could overload the system, it's clear that UEs with valid PTM configurations should continue to receive multicast in RRC_INACTIVE after receiving the session activation notification via enhanced group paging.

Proposal 3: UEs with valid PTM configuration can receive multicast in RRC_INACTIVE state when receiving session activation notification.

However, if a UE does not have UE valid PTM configuration, it should first try to obtain the updated PTM configuration via multicast MCCH instead of initiating RRC resumption. If a multicast session is deactivated and then resumed after a period, it's highly likely that the original PTM configuration is outdated. This is mainly because the PDCP window is shifted during the session activation period, so the previous initial RX_DELIV is outdated.

Observation 2: The PTM configuration outdate is common when data transmission resume, since the PDCP window had been shifted during the session activation period and initial RX_DELIV is outdated.

Therefore, if we let a UE without valid PTM configuration to directly initiate RRC resumption, then most of UEs need to initiate RRC resumption simultaneously if one multicast session has been deactivated and resumed again. This could overwhelm the congestion system where most UEs should receive multicast in RRC INACTIVE state.

Observation 3: Letting all UE to initiate RRC resumption when data transmission resume will impose a great burden on the network

Proposal 4: UEs without valid PTM configuration should first obtain PTM configuration via multicast MCCH when receiving session activation notification.

Time for MCCH monitoring

Compared with Rel-17 multicast or broadcast, the main difference in PTM configuration delivery for Rel-18 multicast in RRC INACTIVE is that the PTM configuration will be delivered to UE in 2 ways, i.e., dedicated signalling, and multicast MCCH. Given that UE can receive multicast sessions in RRC INACTIVE even without the information in multicast MCCH (at least in the beginning), a question is raised that when should UE monitor the multicast MCCH.

In the last meeting, RAN2 reached agreements to use group paging with enhancement to notify a session activation, and use multicast MCCH for session deactivation notification. So it is not necessary to monitor MCCH before session start. Once UE starts receiving multicast in RRC INACTIVE, it needs to monitor MCCH for potential PTM configuration update and session deactivation in the future.

Accordingly, UE should stop monitoring multicast MCCH if the session is deactivated, released, or the UE is no longer interested in the service. In our view, monitoring multicast MCCH for an inactive session is not power-friendly, and all notifications in this stage should be indicated via paging message instead.

Observation 4: Monitoring multicast MCCH for an inactive session will lead to much more power consumption, which is undesirable for UE in RRC INACTIVE state.

Proposal 5: UE is not required to monitor multicast MCCH if its interested multicast session is not activated.

Conclusion

In this contribution, the following observations and proposals are made:

RRC resumption threshold

Proposal 1: The threshold used to trigger RRC resumption is based on existing UE measurement in RRC INACTIVE, i.e., RSRP or RSRQ.

RRC resumption cause

Observation 1: Existing resume causes do not align with the intention for the UE triggered RRC resumption for multicast (scenario 1~3) and not beneficial for the network scheduling of the ongoing multicast session.

Proposal 2: Use one bit in resumecause (e.g., mo-r18Multicast) to cover UE triggered RRC resumption for R18 multicast.

Session activation

Proposal 3: UEs with valid PTM configuration can receive multicast in RRC_INACTIVE state when receiving session activation notification.

Observation 2: The PTM configuration outdate is common when data transmission resume, since the PDCP window had been shifted during the session activation period and initial RX_DELIV is outdated.

Observation 3: Letting all UE to initiate RRC resumption when data transmission resume will impose a great burden on the network.

Proposal 4: UEs without valid PTM configuration should first obtain PTM configuration via multicast MCCH when receiving session activation notification.

Time for MCCH monitoring

Observation 4: Monitoring multicast MCCH for an inactive session will lead to much more power consumption, which is undesirable for UE in RRC INACTIVE state.

Proposal 5: UE is not required to monitor multicast MCCH if its interested multicast session is not activated.

References

R2_122 MBS session notes (Dawid) 2023-05-26 EOM_rm

R2_121bis MBS session notes (Dawid)_2023-04-25 1550 UTC

[AT121bis-e][604] UP issues for Multicast in RRC Inactive (Apple)