3GPP TSG-RAN WG2 Meeting #123	 R2-2307191

Toulouse, France, August 21-25, 2023

Agenda Item:	7.6.3.1

Souce:	MediaTek Inc.

Title:	Remaining Enhancements on Neighbor Cell Measurements in IoT-NTN

Document for:		Discussion and Agreement

# 1 Introduction

After RAN2#122, a posting meeting discussion was arranged to Discuss remaining issues related to SIB handling.

[Post122][112][IoT NTN Enh] Mobility enhancements (Mediatek)

Scope: Discuss remaining issues related to SIB handling(i.e. for neighbor cell/satellite information and for triggers for neighbor cell measurements)

Intended outcome: Summary of the email discussion

Deadline (for companies’ comments and responses): August  9th, 10:00 UTC

In this contribution, we further discuss remaining issue on Enhancements on neighbour cell measurement other from the offline discussion.

# 2 Discussion

In the RAN2-121 meeting, for quasi-earth-fixed cell and earth-moving cell, a serving cell reference location and a distance threshold/radius are specified for detecting when to trigger connected mode measurements. FFS on whether the R17 IEs are reused or not.

The Rel-17 footprint IE(footprintInfo-r17) was introduced for discontinuous coverage in SIB32. It includes referencePoint-r17(including longitude-r17 and latitude-r17), elevationAngles-r-17,radius-r17. Since the elevation angles is not used in rel-18, it is better not to reuse the footprintInfo-r17. But the referencePoint-r17 and radius-r17 can be reused.

Proposal 1: For earth fixed and earth moving cell, serving cell reference location uses the Rel-17 footprint IE (i.e. referencePoint-r17 and radius-r17)

For the new SIB, it has been agreed in the last RAN2 meeting:

RAN2#122 Agreement: In RRC IDLE, how to (re-)acquire neighbour cell assistance information is up to UE’s implementation.

How the new SIB to be acquired and reacquired in the connected mode has not been discussed. The new SIB contains the neighbor satellite ephemeris, which has validity duration. UE needs the information in new SIB for neighbor cell measurement. So that the UE needs to acquire new SIB before connection establish, and UE needs to update new SIB during the connection once the validity duration expires.

For IoT NTN, the SIB31 is reacquired before accessing the cell. In RRC connected mode, T317 and T318 controls the updating of SIB31. For new SIB, the simplest solution of (re)acquisition is follow the SIB31. No need to introduce new mechanism for acquisition, and it minimized the interruption of connection. However, it only works when the new SIB’s validity duration is aligned with SIB31’s.

Proposal 2: If the new SIB’s validity duration is aligned with SIB31’s, in RRC connected mode, UE acquires the new SIB in the same way as SIB31(-NB).

# 3 Conclusions

Proposal 1: For earth fixed and earth moving cell, serving cell reference location uses the Rel-17 footprint IE (i.e. referencePoint-r17 and radius-r17)

Proposal 2: If the new SIB’s validity duration is aligned with SIB31’s. in RRC connected mode, UE acquires the new SIB in the same way as SIB31(-NB).

# 4 References