3GPP TSG-RAN WG2 Meeting #123                                                      R2-2307278

Toulouse, France, August 21-25, 2023

Source:              CATT

Title:	             Discussion on FR2 unknown SCell activation enhancement

Agenda Item:	7.25.1

Document for:	Discussion and Decision

Introduction

In last meeting, based on the LS [1] from RAN4, RAN2 discussed the signaling design to support FR2 unknown SCell activation enhancement, and “RAN2 thinks that the existing L3 measurement framework can be reused for the unkown SCell measurement reporting based on SCell activaton command” according to reply LS [2]. In this paper, we continue discussing the details on measurement reporting and provide our analysis based on the new incoming LS [3] from RAN4.

Discussion

L3 measurement framework

According to the approved LS reply [2] as below in last meeting, RAN2 will reuse the legacy RRC MeasurementReport message to report the measurement result. In legacy MeasurementReport, there are multiple types of information element, such as the measurement result for neighbor cells and location information, these information are not related to the activated unknow SCell, RAN4 only ask RAN2 to report the L3 measurement result for the activated unknow FR2 SCell. Meanwhile, these information is not beneficial for network to shorten the SCell activation delay. Hence, it is suggested the MeasurementReport will include only the measure results associated to activated unknown FR2 SCell, including the measurement quality on beam level and beam index information, and this can be up to network implementation to configure the corresponding reportconfig.

Question 3: Contents of the report of L3 measurement result for unknown FR2 SCell activation enhancement and how to send the report [RAN2 Answer to Q3] - Legacy RRC MeasurementReport message is used for the unkown SCell measurement report, which can already carry the beam level L3 measurement result and beam index. RAN2 will further discuss the detailed signaling design and spec impact in next meeting.

Proposal 1: MeasurementReport for unknown FR2 SCell activation enhancement includes only the measurement results on beam level and beam index information associated to the activated unknow FR2 SCell.

For the measurement report, based on current mechanism, the network will send the reporting configuration to UE to indicate the condition to trigger the report, and also what information is needed in measurement report and some details on how to perform measurement. In our view, for unknown FR2 SCell activation enhancement, there is no significant requirement to introduce cell-specific configuration, the report configuration should be common to all FR2 SCell(s) unless RAN4 send LS to us, which is also aligned with legacy rule.

Proposal 2: The report configuration should be common to all unknown activated FR2 SCell(s).

Based on current measurement mechanism, network always needs to configure reportType IE, such as periodical or eventTriggered, the UE will report the measurement results once the report entering/leaving condition is met. According the LS reply, the unkown SCell measurement report is triggered by SCell activation command MAC CE, but this MAC CE only could be interpreted by MAC Layer. Besides, in order to align with the legacy framework for measure report, a new reportType needs to be introduced for unknown Scell report, the reportType could be receiving an indication from MAC layer to indicate UE has received the SCell activation command MAC CE.

Question 2: How to trigger the report of L3 measurement result for unknown FR2 SCell activation enhancement [RAN2 Answer to Q2] - The unkown SCell measurement report is triggered by SCell activation command MAC CE.

Based on below content in LS [3], it can be seen that, UE will report L3 measurement reporting after receiving SCell activation command MAC CE if the measurement results is valid, the UE will not report the L3 measurement reporting if UE has no valid measurement results. The definition of valid measurement result needs to refer to the content in TS38.133. It is suggested RAN2 spec consider whether the measurement results is valid before UE reporting the L3 measurement result.

Enhancement 1: network triggering report (of L3 measurement result) during FR2 unknown SCell activation - Agreement: - The report of L3 measurement result after SCell activation command needs to be valid. - The report of L3 measurement result is considered as valid only if it fulfils the measurement requirement for a deactivated SCell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) and accuracy requirements in TS 38.133 Clause 10. - Agreement: - UE does not need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results.

Proposal 3:  Define a new reportType to trigger the report of L3 measurement result for unknown FR2 SCell activation, i.e., receiving an indication from MAC layer to indicate UE has received the SCell activation command MAC CE for an unknown FR2 SCell, and UE has valid L3 measurement results.

UE capability

Based on below content in LS [3], RAN4 also proposed to introduce new UE capability for Rx beam sweeping factor less than 8 for cell detection part (X1) and SSB based L1-RSRP measurement (X2). And beam sweeping factor capability X1/X2 are two independent capabilities, RAN2 needs to capture it in the spec and the details are FFS.

Enhancement 2: Beam sweeping factor reduction in FR2 unknown SCell activation Agreement in RAN4#106-bis: - For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement. - if UE has full set (N=8) of beam sweeping during AGC settling part during FR2 unknown SCell activation procedure. - Introduce beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement - Beam sweeping factor capability X1/X2 are two independent capabilities - Note: above enhancement only applies for FR2 unknown SCell activation enhancement - Note: How to capture in spec can be discussed in CR stage - The candidate values for X1/X2 are FFS Agreement in RAN4#107: - Candidates of X1 are {1,2,4,6} - Candidates of X2 are {0,1,2,3,4,5,6,7} - If X1 is absent, beam sweeping factor for cell detection part is 8 - If X2 is absent, beam sweeping factor for SSB-based L1 measurement is 8

Besides, UE capability to support the L3 measurement report for FR2 unknown SCell activation also needs to be introduced, and it is the prerequisite of 	Beam sweeping factor capability X1/X2. But so far, it is not clear about the granularity of this capability based on RAN4 LS, RAN2 still needs to wait for further RAN4 conclusion.

Proposal 4: Introduce optional UE capability to support the L3 measurement report for FR2 unknown SCell activation enhancement, the details could be FFS.

Proposal 5: Introduce optional UE capabilities to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement respectively, the details could be FFS.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: MeasurementReport for unknown FR2 SCell activation enhancement includes only the measurement results on beam level and beam index information associated to the activated unknow FR2 SCell.

Proposal 2: The report configuration should be common to all unknown activated FR2 SCell(s).

Proposal 3:  Define a new reportType to trigger the report of L3 measurement result for unknown FR2 SCell activation, i.e., receiving an indication from MAC layer to indicate UE has received the SCell activation command MAC CE for a unknown FR2 SCell, and UE has valid L3 measurement results.

Proposal 4: Introduce optional UE capability to support the L3 measurement report for FR2 unknown SCell activation enhancement, the details could be FFS.

Proposal 5: Introduce optional UE capabilities to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement respectively, the details could be FFS.

Reference

- R2-2304636, LS on FR2 unknown SCell activation enhancement, Apple

- R2-2306863, LS reply on FR2 unkonwn SCell activation enhancement, Apple

- R2-2307039, LS on FR2 SCell activation enhancements, Apple