3GPP TSG RAN WG 2 Meeting #123		R2-2307389

21st - 25th August, 2023

Title:	[Draft] Reply LS on beam application time for LTM

Response to:	LS R2-2307020/R1-2306259 on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM

Release:	Release 18

Work Item:	NR_mob_enh2-Core

Source:	Fujitsu, MediaTek, CATT (to be RAN2)

To:	RAN1, RAN4

Cc:	RAN3

Contact person:	Takako Sanda

sanda.takako<atmark>fujitsu.com

Li-Chuan Tseng

Li-Chuan.Tseng<atmark>mediatek.com

Rui Zhou

Zhourui<atmark>catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	<38.300 running CR> (To be attaced)

1	Overall description

RAN2 thanks RAN1 for their LS on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM.

RAN2 would like to provide the following feedback for RAN1’s question:

RAN1 respectfully asks RAN2 to take the RAN1 agreements into consideration for their work and kindly asks to provide their feedback on the RAN1 requests described in section A.

A. Decision of the value for beam application time

RAN2 assumes “Cell switching time” mentioned in the LS R1-2306259 refers to Components of Mobility Latency captured in TS 38.300 running CR or LTM supervisor timer which is used to detect LTM switch failure.

For the case of Components of Mobility Latency, the beam application time is a part of the components, i.e., beam application time should at least include T_processing2 (“This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc.”, see the attachment). However, RAN2 will not intend to decide exact value of each component.

For the case of LTM supervisor timer, the value range of the beam application time should be considered when RAN2 decides the value range of LTM supervisor timer.

Based on above, RAN2 prefers that RAN1 and RAN4 decide the value of the beam application time first and then input the decided value to RAN2.

B. Potential RAN3 related issue

When deciding the value of beam application time, network interface delay for inter-DU LTM case may need to be considered as the target cell should be ready on the selected beam when beam application time arrives at UE side.

In the RAN2#122 meeting the following agreement was made:

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell:

- - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch.

- - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant.

If the beam application time is much smaller than network interface delay, after beam application completion, the UE may need to monitor PDCCH longer time on the selected beam at the inter-DU target cell until receiving the first dynamic UL grant scheduling.

If RAN1 and RAN4 find network delay needs to be considered, RAN3 should be involved.

2	Actions

To RAN1 and RAN4 groups

ACTION: 	RAN2 kindly asks RAN1 and RAN4 to decide the value of beam application time and inform RAN2 of the decided value. Also, RAN2 kindly asks RAN1 and RAN4 to take the information described in section B into consideration.

3	Dates of next TSG RAN WG 2 meetings

TSG RAN WG2 Meeting #123 bis	9 – 13 October 2023		Xiamen, CN

TSG RAN WG2 Meeting #124		13 – 17 November 2023		Chicago, US