3GPP TSG-RAN WG2 #123 	R2-2307473

Toulouse, France 21 – 25 August 2023

Agenda Item:	7.14.3

Source:	NEC

Title:	Discussion on buffer level threshold-based triggering

Document for:	Discussion, Decision

Introduction

RAN2#122 meeting further discussed Rel-17 leftover topics for QoE and achieved the following agreements [1]:

Agreements - RAN2 confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by application layer. - RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting. - RAN2 will align with RAN3’s agreement on support of radio-related event triggers for RVQoE reporting in Rel-18. - Application layer measurement configuration and reporting for shared spectrum channel access is not supported in R18.

This contribution further discusses buffer level threshold-based triggering for Rel-18 RVQoE report.

Discussion

Regarding triggers for RVQoE reporting, recent RAN3 meetings achieved following agreements [2]:

RAN3#119bis-e meeting Triggers for RVQoE reporting - Radio-related event triggers for RVQoE reporting is not supported in Rel-18. - If a UE is configured with periodic RVQoE reporting that automatically starts at the beginning of the application session or immediately upon reception of RVQoE configuration, it cannot be configured with a threshold-based trigger at the same time. - Discuss whether threshold-based buffer level reporting starts: i) when buffer level is greater than a threshold or ii) when buffer level is below a threshold or iii) when buffer level is between two thresholds. - RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options: Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports) Option 2: Periodically based on a gNB configured reporting periodicity Option 3: A certain number of times based on gNB configured report amount - Further discuss whether to introduce TTT (time to trigger) for threshold-based triggers. RAN3#120 meeting The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

The agreement (2) at the RAN3#119bis-e meeting is on the relationship between Rel-17 periodic RVQoE report and Rel-18 event-based RVQoE report. From configuration point of view, ran-VisibleParameters-r17 shown below contains periodicity, number of buffer level entries and report of playout delay. Moreover, the number of buffer level entries is also used by application layer to calculate the interval of RAN visible buffer level measurement, which equals to the periodicity of RAN visible application layer measurements reporting divided by numberOfBufferLevelEntries.

Compared to Rel-17, Rel-18 event-triggered RVQoE introduces new buffer level threshold-based triggers for RVQoE reports while the RVQoE measurement itself is the same as in Rel-17. Therefore, ran-VisiblePeriodicity-r17 and numberOfBufferLevelEntries-r17 are still needed to determine the interval of RAN visible buffer level measurement.

After checking the RAN3 discussion summary in [2], the intention of agreement (2) seems to clarify that the periodic RVQoE report and event triggered RVQoE report should not be allowed to be activated together. Therefore, we propose RAN2 to confirm that when Rel-18 event triggered RVQoE report is configured, the App layer will stop to report Rel-17 periodic RVQoE report of buffer level.

RAN2 confirm that when Rel-18 event triggered RVQoE report is configured, the Rel-17 periodic RVQoE report of buffer level is deactivated/stopped.

Another issue discussed in RAN3 was how the UE to send the RVQoE reports after the threshold is met. So far, the following options are listed up, however no consensus were achieved:

Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports)

Option 2: Periodically based on a gNB configured reporting periodicity

Option 3: A certain number of times based on gNB configured report amount

In our understanding all three options can work. RAN2 can wait for further RAN3 agreements to conclude how UE sends the event triggered RVQoE reports. From RAN2 perspective, when the threshold-based RVQoE report is configured, the network needs to configure proper threshold values. However, network may not have full knowledge on doing that. When thresholds are configured too low or too high, it may result in no RVQoE report can be collected at all within a certain period. In such cases, we think it would be beneficial to let the UE inform the network that all RVQoE results collected in the UE side do not fulfill the configured threshold values. Then the network can take consequent actions, e.g., adjust threshold values, etc.

RAN2 to discuss whether it is beneficial to inform the network that within a certain period, all RVQoE results collected in the UE side do not fulfill the configured threshold values.

On the configuration of buffer level threshold triggers, whether to introduce TTT (time to trigger) is still under RAN3 discussions. Besides TTT, whether to introduce hysteresis for RVQoE report may also need to be discussed. So far, the range of hysteresis is [0,30]. If hysteresis for RVQoE is needed, the enhancement of the hysteresis range or a RVQoE specific hysteresis with different range may be needed.

RAN2 to discuss whether to introduce hysteresis for threshold-based triggers.

Summary

This contribution discussed buffer level threshold based triggering for Rel-18 RVQoE report.

Proposal 1 RAN2 confirm that when the Rel-18 event triggered RVQoE report is configured, the Rel-17 periodic RVQoE report of buffer level is deactivated/stopped.

Proposal 2 RAN2 to discuss whether it is beneficial to inform the network that within a certain period, all RVQoE results collected in the UE side do not fulfill the configured threshold values.

Proposal 3 RAN2 to discuss whether to introduce hysteresis for threshold-based triggers.

- References

[1] R2-2306542 Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)

[2] R3-232046 Summary of Offline Discussion on CB: # QoE4_Others