3GPP TSG RAN WG2#123	R2-2307519

Toulouse, France, 21st - 25th August, 2023

Agenda item:	7.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On NES SSB-less SCell operation

WID/SID:	Netw_Energy_NR-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

# RAN has approved WI on NW energy saving (RP-223540) with one of the objectives:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

In this contribution we discuss SSB-less SCell operation based on RAN4 progress.

# 2	Discussion

RAN4 has discussed (R4-2306369) the following scenarios for SSB-less SCell operation:

- Agreements

  - The following scenarios are considered for inter-band SSB-less SCell requirements definition

    - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell

      - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation

    - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell

    - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell.

    - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

RAN4 have the following FFS for the scenarios:

Scenario 1:

    - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

    - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations.

    - FFS if requirements will be defined subject to feasibility analysis

Scenario 2:

    - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

    - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations.

    - FFS if requirements will be defined subject to feasibility analysis

Scenario 2a:

    - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

    - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations.

    - FFS if requirements will be defined subject to feasibility analysis

RAN4 has not concluded whether it is feasible to specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells.

Proposal 1: Before progressing RAN2 should wait RAN4 to conclude on SSB-less SCell operation

# 3	Conclusion

Based on the discussion following is proposed:

Proposal 1: Before progressing RAN2 should wait RAN4 to conclude on SSB-less SCell operation