3GPP TSG-RAN WG2 Meeting #123	R2-2307608

Toulouse, France, August 21-25, 2023

Agenda Item:	7.5.4.2

Source: 	Meta

Title:  	Discussion on PDU Discard Operation for XR

Document for:	Discussion and Decision

# Introduction

In the RAN2#122 meeting, the following agreement was made:

- PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e., whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity. - Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. - FFS how/whether to minimize additional UL signalling after this indication. - FFS if the NW indication is a one-shot or also subsequent packets.

In RAN#120,

- RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.

In RAN#121

- RAN2 thinks PSI can be useful for PDU set-based discard. RAN2 aims to introduce a mechanism to allow UE to handle discarding of packets with different PSI in case of congestion. FFS for other cases.

In RAN#121bis-e

- PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

In this paper, we consider PDU Discard Operation based on the PDU Set framework for XR.

# Discussion

In the RAN2#122 meeting, PSI-based XR discard mechanism was discussed. When congestion occurs, the network sends PDU-set discard indication for UL via RRC signaling. The UE performs PDU discard to mitigate UL congestion.

In terms of the effectiveness of UL congestion mitigation, PSI indication alone may not be sufficient. This is because the UL congestion is commonly caused by large data sizes or high network loads. For example, for XR traffic, UL pose control or control data may be classified as low-importance data. However, the data packets are small compared to videos and audios. Discarding low-importance but small-sized data may turn out to be not effective in solving congestion issues. The more effective way to improve congestion performance is to reduce the network load by discarding large packets. By discarding the PDU sets with high data rates, the network congestion will be improved quickly.

Observation 1: PSI alone is not sufficient for PDU discard to mitigate UL congestion.

Proposal 1: Under UL congestion, discard PDU sets with high data rate (regardless of their importance) first and discard PDU sets with high importance last.

The UL channel condition and network congestion states can vary rapidly. When network congestion improves, it is important for the network to send subsequent signaling to the UE to disable PDU discard operation.

Proposal 2: When UL congestion improves, the network sends subsequent signaling to the UE.

# Conclusion

In this paper, we discuss PDU Discard Operation in the presence of UL congestion.

Observation 1: PSI alone is not sufficient for PDU discard to mitigate UL congestion.

Proposal 1: Under UL congestion, discard PDU sets with high data rate (irrespective of their importance) first and discard PDU sets with high importance last.

Proposal 2: When UL congestion improves, the network sends subsequent signaling to the UE.

# References

[1] RAN2 Chairman Notes from RAN2#122, May 2023.

[2] R2-2306402 Discussion on PDU Discard Operation for XR, Meta USA