3GPP TSG RAN WG2 Meeting #123       	      	                	               R2- 2307685

Toulouse, FR, 21st – 25th Aug, 2023

Agenda Item:	7.16.2.3

Source:	Intel Corporation

Title:	Architecture impact on model transfer method

Document for:	Discussion and Decision

# Introduction

During RAN2 #121 meeting, following solutions were agreed and their analyzed pros/cons are summarized in the table.

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

Considering the selection of solutions are highly related to the architecture assumption and LCM entity mapping within the network. In this contribution, we further analysis the architecture assumption impact to model transfer method.

# Discussion

## General Assumptions

#### Model Format

Proprietary model format and open format are discussed in RAN1.

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion, Proprietary-format models; ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format. Open-format models; ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive. From RAN1 discussion viewpoint, RAN1 may assume that: - Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared. - Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

It is understanded that the proprietary model may not support interoperability, while the open format allows model formats (e.g. ONNX, etc) that can be shared across different vendors. For proprietary model, during model transfer, the network can only transfer the model as a whole to the UE. However, for open format, it allows a possibility to transfer the model associated with its model parameters and model structure to the UE. The model parameter and model structure may not be transferred together with the model itself.

Hence, from RAN2 point of view, the model itself being transferred can be treated as a container, and it is equivalent between open format and proprietary model.

Observation 1: RAN2 can discuss model transfer/delivery solution independent from what model format is used assuming the model itself is transferred as a container.

## Further Analysis and Down-Selection on Model Transfer/Delivery Solutions

As agreed in RAN2 #122 meeting, functional framework for AI/ML for NR air interface is captured in [4]. To facilitate the discussion of model transfer/delivery, model storage is agreed as a reference point for protocol termination of model delivery/transfer. However, this does not limit where models are actually stored.

Functionality mapping was discussed in [Post122][060][AIML] Mapping of functions to physical entities. Below table is a summary of model transfer/delivery options based on our observation and understanding in outcome of [Post122][060][AIML] :

| Use case/sub-use case |  | Training Entity | Inference Entity | Model Transfer/Delivery |
| --- | --- | --- | --- | --- |
| Two-sided CSI compression | Training Type 1 (joint training at a single side/entity) | gNB | gNB | None |
|  |  |  | UE | gNB -> UE |
|  |  | OAM | gNB | OAM -> gNB (out of RAN2 scope) |
|  |  |  | UE | OAM -> gNB (intermediate) -> UE |
|  |  | UE/OTT server | gNB | UE/OTT server -> gNB |
|  |  |  | UE | None or OTT server -> UE (out of RAN2 scope) |
|  | Training Type 3 (separate training) | gNB & UE/OTT server | gNB | None |
|  |  |  | UE | None or OTT server -> UE (out of RAN2 scope) |
|  |  | OAM & UE/OTT server | gNB | OAM -> gNB (out of RAN2 scope) |
|  |  |  | UE | None or OTT server -> UE (out of RAN2 scope) |
| UE-sided CSI prediction/UE-sided beam management |  | gNB | UE | gNB -> UE |
|  |  | OAM | UE | OAM -> gNB (intermediate) -> UE |
|  |  | UE/OTT server | UE | None or OTT server -> UE (out of RAN2 scope) |
| NW-sided beam management |  | gNB | gNB | None |
|  |  | OAM | gNB | OAM -> gNB (out of RAN2 scope) |
| Positioning accuracy enhancement | Case 1: UE-based with UE-side model | LMF | UE | LMF -> UE |
|  |  | OAM | UE | OAM -> gNB (intermediate) -> UE |
|  |  | UE/OTT server | UE | None or OTT server -> UE (out of RAN2 scope) |
|  | Case 2a: UE-assisted/LMF-based with UE-side model | LMF | UE | LMF -> UE |
|  | Case 2b: UE-assisted/LMF based with LMF-side model | LMF | LMF | None |
|  | Case 3a: RAN-assisted with gNB-side model | LMF | gNB | LMF -> gNB |
|  |  | OAM | gNB | OAM -> gNB (out of RAN2 scope) |
|  |  | gNB | gNB | None |
|  | Case 3b: RAN-assisted with LMF-side model | LMF | LMF | None |

It is observed that if both model training and model inference are located at the same entity or at network side (i.e. the grey highlighted options), there’s no need to consider model transfer/delivery over the air interface, hence there is no RAN2 impact. As summarized in above table, from RAN2 study point of view, RAN2 can focus on the following three over-the-air model transfer/delivery options in downlink towards UE and one over-the-air model transfer/delivery options in uplink.

Downlink:

1) gNB -> UE

suitable use cases: CSI compression, UE-side CSI prediction, UE-side beam management

not applicable use case: positioning accuracy enhancement

2) OAM -> gNB (intermediate) -> UE

suitable use cases:  CSI compression, UE-side CSI prediction, UE-side beam management, positioning accuracy enhancement

not applicable use case: none

3) LMF -> UE

Suitable use case: positioning accuracy enhancement

Not applicable use cases: CSI compression, CSI prediction, beam management

Uplink:

1) UE -> gNB

Suitable use case: CSI compression

Not applicable use cases: CSI prediction, beam management, positioning accuracy enhancement

Observation 2: Considering the functionality mapping of three use cases, for model training at network side, model inference at UE side, the model can be transferred from gNB/OAM/LMF to UE over the air interface.

A table of relation between model transfer/delivery solutions and use cases was agreed during RAN2 #121 meeting. Based on above functionality mapping of model training and model inference entity, CN (except LMF) is not a suitable network entity for the studied three use cases. Hence, for solutions where model is transferred from CN to UE, i.e. solution 2a and solution 2b can be deprioritized.

Proposal 1: Considering the functionality entity mapping of all three use cases, Solution 2a and Solution 2b, where model is transferred from CN (except LMF) to UE, can be deprioritized .

## Further analysis on Solution 1a

During RAN2 #121 meeting discussion, all other solutions except solution 1a has dependency with other WGs. Therefore, in this contribution, we mainly focus on solution 1a, where RAN2 could further progress and consider how to support model transfer with limited optimization.

It was captured in R2-2302268 [1], solution 1a has following limitations:

1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes)

2. Maybe high control plane overhead, as a large model size may need segmentation/transmission/acknowledgment. This consumes critical configuration time for model transfer/delivery

3. An incomplete control plane model transfer has to be restarted upon mobility, as there are no current procedures to resume transmission across gNBs. Some companies wonder whether it is critical or not as it depends on how frequent the gNB to send new/updated AI/ML to the UE

For the 1st and 2nd limitation, as discussed earlier in above table, the model can be either stored at OAM or gNB. Though the existing RRC signaling cannot support large size of AI/ML model’s transfer due to limited segmentation number, we think increasing the support the number of RRC segmentation is not a challenging issue.

Furthermore, whether to use existing SRB or a new SRB was also discussed. As mentioned earlier, the AI/ML model is not an urgent task and does not require time critical  transmission. Similar as QoE, a new low priority SRB is used to carry AI/ML model. The priority of this new SRB can be the same as SRB4 for Rel-17 QoE. For the case where AI/ML model is initially transmitted from OAM, if segmentation is needed, similar as OAM, the segmentation can be done at RRC layer.

For 3rd limitation, to support continuity during handover, it can be solved by simply extending PDCP reestablishment to SRB and retaining the segments that were transferred successfully before HO without fundamentally changing NG-RAN architecture or introducing a new concept of DRBs to carry user plane data only to RAN.

As captured in TS38.323 [2], current PDCP entity reestablishment does not support for SRB. To achieve continuity during handover, this could be simply achieved by extending current PDCP entity reestablishment to this new SRB used for AI/ML model.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall: - for SRBs (except SRBx for AI/ML model) and UM DRBs, set TX_NEXT to the initial value; - for SRBs (except SRBx for AI/ML model), discard all stored PDCP SDUs and PDCP PDUs; - for SRBx (for AI/ML model) and AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below: - … - submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1. When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall: - process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1; - for SRBs (except SRBx for AI/ML model), discard all stored PDCP SDUs and PDCP PDUs; - for SRBs (except SRBx for AI/ML model), UM DRBs and UM MRBs, if t-Reordering is running: - stop and reset t-Reordering; - for UM DRBs and UM MRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression; - for SRBs (except SRBx for AI/ML model), UM DRBs and UM MRBs, set RX_NEXT and RX_DELIV to the initial value;

Furthermore, as discussed in the companion contribution [3], if the model is not fully transmitted to the UE and the UE is handed-over to another cell, the model might need continue transmitted to the UE if the network required so. Therefore, to support model transfer/delivery continuity when RRC segmentation is needed for model transfer/delivery, a UE needs to temporarily store the received RRC segments, and reassemble if needed after HO until all segments are received.

Proposal 2: RAN2 confirms that Solution 1a is feasible and following observations for Solution 1a are proposed to be captured in the TR:

- model can be stored at OAM, and transmitted to NG-RAN for model transfer

- increase the number of supported RRC Segmentation to support large model size.

- A new low priority SRB is introduced to carry AI/ML model.

- during HO, PDCP reestablishment is applicable to this new SRB.

- RRC segments storage during HO and reassemble after HO is needed in RRC.

## Specification Effort

With above analysis for solution 1a, below we provide an initial view on the specification effort of different solutions with reasons:

|  | Pros | Cons | Specification Effort |
| --- | --- | --- | --- |
| Solution 1a | 6. The existing RRC signaling solutions can be reused as baseline, at least including delta signaling and egmentation 9. Additional security and verification may not be necessary as the UE already established security before the transfer is initiated 11. Gnb can take the control of the AIML model transfer itself, which can not be achieved by traditional UP based solution | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 2. Maybe high control plane overhead, as a large model size may need segmentation/transmission/acknowledgment. This consumes critical configuration time for model transfer/delivery 3. An incomplete control plane model transfer has to be restarted upon mobility, as there are no current procedures to resume transmission across gNBs. Some companies wonder whether it is critical or not as it depends on how frequent the Gnb to send new/updated AI/ML to the UE | Small - increase RRC segment number, support RRC segment storage and reassemble - introduce a new low priority SRB, where PDCP reestablishment is support for service continuity |
| Solution (2a and )3a | 5. Service continuity on model transfer/delivery is easy to achieve compared with Solution 1a 6. Impacts on RAN2 may be limited (some companies think that LPP signalling is in RAN2 scope) | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 3. If NAS does the segmentation, it may introduce some overhead 4. (only valid for Solution 2a) CN is not a good option for later on model monitoring/activation/deactivation/fallback/update that requires less latency. The model transfer/delivery is transparent to Gnb, it could be tricky to get Gnb involved in the AI model LCM. It could be problematic when the network needs to be in control of what happening at the UE side and especially in two-sided models where one side of the model is intended to be located at the network side | Medium - may involve other WGs, e.g. SA2 - CN is not considered as training entity for three use cases |
| Solution 1b | 1. The network can provide different 5Qis for model transfer/delivery with different QoS requirements (e.g. can support large model size) 2. Compared with CP-based solutions, this Solution 1b can reduces control plane overhead, reduces overhead at Gnb for model delivery/transfer 5. Compared with CP-based solutions, it may not need to consider CP message segmentation, CP message blocking issue | 5. Not compatible with current mobility procedure. Supporting model transfer during mobility is not so straightforward | Large - the solution requires architecture changes, e.g. introducing new protocol layer; or change DRB establishment fundamental rules |
| Solution (2b and )3b | 1. The network can provide different 5Qis for model transfer/delivery with different QoS requirements (e.g. can support large model size) 5. Compared with CP-based solutions, it may not need to consider CP message segmentation, CP message blocking issue | 2. CP signalling is needed to configure and initiate the model transfer from the CN 4. May be unable to support delta-model transfer/delivery based on current user plane framework | Large - it’s unclear the benefit of using CP signaling if model storage is not at CN - CN is not considered as training entity for three use cases |
| Solution 4 | 2. If 3GPP network can be aware of AI/ML model in this Solution 4, the network can provide different 5Qis for model transfer/delivery with different QoS requirements (e.g. can support large model size). How to synchronize 3GPP and server so that the network can take appropriate actions is not clear, and it may not be fully under 3GPP control | 2. There may be inter-operability issues, such as: a) Different implementations may lead to different model performances and a huge burden of model management (e.g., frequent model activation/deactivation) b) Massive offline coordination is needed or requires lots of coordinations among vendors, especially for the CSI compression use case 4. When network cannot control the model transfer/delivery, the transfer of large model may impact important and delay sensitive user data traffic | No impact - no 3GPP impact |

Proposal 3: RAN2 to adopt above table with specification effort for different solutions in the TR.

# Conclusion

In this contribution, we analyze the general architecture impact to model transfer method:

Observation 1: RAN2 can discuss model transfer/delivery solution independent from what model format is used assuming the model itself is transferred as a container.

Observation 2: Considering the functionality mapping of three use cases, for model training at network side, model inference at UE side, the model can be transferred from gNB/OAM/LMF to UE over the air interface.

Proposal 1: Considering the functionality entity mapping of all three use cases, Solution 2a and Solution 2b, where model is transferred from CN (except LMF) to UE, can be deprioritized .

Proposal 2: RAN2 confirms that Solution 1a is feasible and following observations for Solution 1a are proposed to be captured in the TR:

- model can be stored at OAM, and transmitted to NG-RAN for model transfer

- increase the number of supported RRC Segmentation to support large model size.

- A new low priority SRB is introduced to carry AI/ML model.

- during HO, PDCP reestablishment is applicable to this new SRB.

- RRC segments storage during HO and reassemble after HO is needed in RRC.

Proposal 3: RAN2 to adopt below table with specification effort for different solutions in the TR.

|  | Pros | Cons | Specification Effort |
| --- | --- | --- | --- |
| Solution 1a | 6. The existing RRC signaling solutions can be reused as baseline, at least including delta signaling and egmentation 9. Additional security and verification may not be necessary as the UE already established security before the transfer is initiated 11. Gnb can take the control of the AIML model transfer itself, which can not be achieved by traditional UP based solution | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 2. Maybe high control plane overhead, as a large model size may need segmentation/transmission/acknowledgment. This consumes critical configuration time for model transfer/delivery 3. An incomplete control plane model transfer has to be restarted upon mobility, as there are no current procedures to resume transmission across gNBs. Some companies wonder whether it is critical or not as it depends on how frequent the Gnb to send new/updated AI/ML to the UE | Small - increase RRC segment number, support RRC segment storage and reassemble - introduce a new low priority SRB, where PDCP reestablishment is support for service continuity |
| Solution (2a and )3a | 5. Service continuity on model transfer/delivery is easy to achieve compared with Solution 1a 6. Impacts on RAN2 may be limited (some companies think that LPP signalling is in RAN2 scope) | 1. Face challenges to convey large size or “no upper limit size” AI model by RRC message (e.g. >45kBytes) 3. If NAS does the segmentation, it may introduce some overhead 4. (only valid for Solution 2a) CN is not a good option for later on model monitoring/activation/deactivation/fallback/update that requires less latency. The model transfer/delivery is transparent to Gnb, it could be tricky to get Gnb involved in the AI model LCM. It could be problematic when the network needs to be in control of what happening at the UE side and especially in two-sided models where one side of the model is intended to be located at the network side | Medium - may involve other WGs, e.g. SA2 - CN is not considered as training entity for three use cases |
| Solution 1b | 1. The network can provide different 5Qis for model transfer/delivery with different QoS requirements (e.g. can support large model size) 2. Compared with CP-based solutions, this Solution 1b can reduces control plane overhead, reduces overhead at Gnb for model delivery/transfer 5. Compared with CP-based solutions, it may not need to consider CP message segmentation, CP message blocking issue | 5. Not compatible with current mobility procedure. Supporting model transfer during mobility is not so straightforward | Large - the solution requires architecture changes, e.g. introducing new protocol layer; or change DRB establishment fundamental rules |
| Solution (2b and )3b | 1. The network can provide different 5Qis for model transfer/delivery with different QoS requirements (e.g. can support large model size) 5. Compared with CP-based solutions, it may not need to consider CP message segmentation, CP message blocking issue | 2. CP signalling is needed to configure and initiate the model transfer from the CN 4. May be unable to support delta-model transfer/delivery based on current user plane framework | Large - it’s unclear the benefit of using CP signaling if model storage is not at CN - CN is not considered as training entity for three use cases |
| Solution 4 | 2. If 3GPP network can be aware of AI/ML model in this Solution 4, the network can provide different 5Qis for model transfer/delivery with different QoS requirements (e.g. can support large model size). How to synchronize 3GPP and server so that the network can take appropriate actions is not clear, and it may not be fully under 3GPP control | 2. There may be inter-operability issues, such as: a) Different implementations may lead to different model performances and a huge burden of model management (e.g., frequent model activation/deactivation) b) Massive offline coordination is needed or requires lots of coordinations among vendors, especially for the CSI compression use case 4. When network cannot control the model transfer/delivery, the transfer of large model may impact important and delay sensitive user data traffic | No impact - no 3GPP impact |

# References

- R2-2302268, Report of Offline 027 model transfer delivery

- TS38.323, NR; Packet Data Convergence Protocol (PDCP) specification

- R2-2307684, AI/ML locality and capability: RAN2 impact, Intel Corporation

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface