3GPP TSG-RAN WG2 Meeting #123                                                      R2-2307707

Toulouse, France, August 21-25, 2023

Source:	CATT

Title:	Further discussion on SPR

Agenda Item:	7.13.4

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, SPR related issues were discussed and achieved the following agreements [1].

=>	SPR except the critical issues will not be further enhanced from this meeting until the end of R18.

Agreements:

SPR

1	 For values of triggering conditions of SPR, Percentage based threshold variables for SHR (T310/T312/T304) can be reused for SPR is applied.

As agreed above, SPR except the critical issues will not be further enhanced from RAN2#122 meeting until the end of R18.

So, in this contribution, we will discuss the critical issues for SPR we identified in section 2.

Discussion

- SPR configuration

In RAN2#120 meeting, SPR configuration was discussed and made the following agreements [2]:

4 For Q8, RAN2 agree following options: depends on which of nodes initiates SPR, i.e.: For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE For the SN-initiated PSCell Change, the source-SN sends the Successful PSCell Change configuration within the container through MN. T304 trigger needs to be configured by the target SN node. 1 UE stores both SPCR and SHR configuration (one for each type at most) if received from NW.

Based the agreements above, the UE only stores one SPR configuration, for legacy PSCell change procedure, we consider the following issues which need to be further discussed and addressed.

If SN hopes to configure SPR configuration (T310/T312 triggering conditions) and trigger PSCell change procedure, SN will send the SPR configuration within the container through MN before sending SN-initiated PSCell change command, then UE could receive MN-initiated PSCell change command, UE has no idea about the PSCell change type (i.e. MN-initiated PSCell change or SN-initiated PSCell change), UE cannot determine whether it should record SPR or not. Furthermore, UE could record the SPR and report it to SN based on SN configured SPR configuration even if the PSCell change is triggered by MN which is not what we expect.

Observation 1: The UE cannot determine whether it should record SPR or not and furthermore it could record the SPR and report the SPR to SN based on SN configured SPR configuration when receives MN-initiated PSCell change command as UE has no idea about the PSCell change type.

If the UE can get to know the PSCell change type (e.g. by network indication), the UE will record SPR based MN/SN configured SPR configuration only when MN/SN triggers the PSCell change. In case of UE is configured with MN/SN configured SPR configuration but receives SN/MN triggered PSCell change command, the UE will not record the SPR even if the SPR trigger conditions are met. The SPR configuration will be released after successful PSCell change which means the SPR configuration is inefficient.

Observation 2: If the UE can get to know the PSCell change type, the SPR configuration is inefficient in case of UE is configured with MN/SN configured SPR configuration but receives SN/MN triggered PSCell change command.

In addition, if SN sends SPR configuration to UE, the MN could also send SPR configuration to UE since the MN doesn’t know the UE has stored SN configured SPR configuration, the MN configured SPR configuration will override SN configured SPR configuration, and vice versa.

Observation 3: MN/SN configured SPR configuration can override SN/MN configured SPR configuration as MN/SN doesn’t know the UE has stored SN/MN configured SPR configuration.

For CPC scenario, MN-initiated CPC and SN initiated CPC could be configured to UE simultaneously, if both MN and SN hope to configure SPR configuration to UE, it cannot be achieved based current agreement, i.e.  the UE only stores one SPR configuration. Furthermore, whether MN-initiated CPC or SN-initiated CPC is performed is unpredictable, if only one SPR configuration is stored in UE, there is a possibility that the SPR configuration is useless, e.g. the SPR configuration stored in UE is configured by MN, but the UE executes SN-initiated CPC.

Observation 4: MN and SN cannot configure SPR configuration to UE simultaneously in case of both MN-initiated CPC and SN-initiated CPC are configured to UE.

Observation 5: The SPR configuration could be useless as it is unpredictable for MN-initiated CPC or SN-initiated CPC is performed.

From our perspective, the issues mentioned above is serious especially for CPC case, considering above, we prefer to store two SPR configurations in UE to address the issues mentioned above.

Proposal 1: UE should be allowed to store two SPR configurations configured by MN and SN respectively.

- PSCell change/CPC type indication information

In RAN3#119bis-e meeting, RAN3 discussed the PSCell change type indication information issue and achieved the follow agreement.

Agreements: To assist in the forwarding of SPR, UE may include the following in SPR - CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed) - WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

According to RAN3’s agreement, the PSCell change type should be included in SPR. But how UE knows this information needs to be considered.

From RAN2’s perspective, for legacy PSCell change, MN needs to indicate the PSCell change type to UE as the UE may not know the PSCell change type (i.e. MN-initiated inter-SN PSCell change or SN-initiated inter-SN PSCell change). For intra-SN PSCell change without MN involvement, the source SN sends the PSCell change command by SRB3. The UE could get to know the PSCell change type based on the receiving path.

Observation 6: For legacy PSCell change, the PSCell change type indication information needs to be indicated to UE.

For MN-initiated CPC and SN-initiated CPC, the UE can get to know the CPC type by measId as the execution conditions configured for MN-initiated CPC are based on MN configured measConfig and the execution conditions configured for SN-initiated CPC are based on SN configured measConfig. UE can differentiate SN-initiated intra-SN CPC from SN-initiated inter-SN CPC by the receiving path, i.e. SN-initiated inter-SN CPC configuration is configured in MN RRCReconfiguration message, however, SN-initiated intra-SN CPC configuration is configured in SN RRCReconfiguration meesage which as container in MN RRCReconfiguration message. Therefore, no extra indication information is needed for UE to identify the CPC type.

Observation 7: For CPC, no extra indication information is needed for UE to identify the CPC type as the UE can get to know the CPC type by implicit way.

Based on the above, we propose,

Proposal 2: For legacy PSCell change, the PSCell change type indication information should be configured to UE to indicate the executed PSCell change type.

Conclusion

According to the discussion in section 2, we propose:

Observation 1: The UE cannot determine whether it should record SPR or not and furthermore it could record the SPR and report the SPR to SN based on SN configured SPR configuration when receives MN-initiated PSCell change command as UE has no idea about the PSCell change type.

Observation 2: If the UE can get to know the PSCell change type, the SPR configuration is inefficient in case of UE is configured with MN/SN configured SPR configuration but receives SN/MN triggered PSCell change command.

Observation 3: MN/SN configured SPR configuration can override SN/MN configured SPR configuration as MN/SN doesn’t know the UE has stored SN/MN configured SPR configuration.

Observation 4: MN and SN cannot configure SPR configuration to UE simultaneously in case of both MN-initiated CPC and SN-initiated CPC are configured to UE.

Observation 5: The SPR configuration could be useless as it is unpredictable for MN-initiated CPC or SN-initiated CPC is performed.

Observation 6: For legacy PSCell change, the PSCell change type indication information needs to be indicated to UE.

Observation 7: For CPC, no extra indication information is needed for UE to identify the CPC type as the UE can get to know the CPC type by implicit way.

Proposal 1: UE should be allowed to store two SPR configurations configured by MN and SN respectively.

Proposal 2: For legacy PSCell change, the PSCell change type indication information should be configured to UE to indicate the executed PSCell change type.

Reference

- Draft_R2_122_meeting_report_v2

- R2-2300002 Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France