3GPP TSG-RAN WG2 Meeting #123	R2-2307737

Toulouse, France, 21st – 25th Aug. 2023

Source:	vivo, Huawei, HiSilicon, Apple, MediaTek Inc., Qualcomm Inc., Ericsson, Intel Corporation, CMCC, Guangdong Genius

Title:	Discussion on RAN1 LS on Msg.4 PDSCH transmission

Agenda Item:	7.19.1

Document for:	Discussion and Decision

- Introduction

In RAN2#122 meeting, an LS [1] on Msg.4 PDSCH transmission to Rel-18 eRedCap UEs has been received from RAN1.  In this contribution, we will discuss the issue in LS from RAN2 persepctive.

- Discussion

RAN1 sent an LS[1] to RAN2 regarding Msg4 PDSCH transmission to Rel-18 eRedCap UEs. In the LS, the RAN1 agreement and a related case are indicated, which are quoted as follows. The intention for this LS is to request RAN2 to discuss UE behaviour in case that when UE with BB reduction capability detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process.

Agreement Confirm the following working assumption by assuming that Msg3 indication is available Working Assumption - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.. RAN1 would like to inform RAN2 about the following case, to specify, if needed, the UE behavior in the RAN2 specifications, and ask RAN2 for feedback if any: - For UE BB complexity reduction, the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process

In RAN1#112bis-e meeting, following three options have been discussed:

Option 1: The UE considers the contention resolution as not successful.

Option 2: The UE discards the DCI and continues monitoring the DCI until ra-ContentionResolutionTimer expires.

Option 3: The UE behavior is up to the UE implementation.

These options were discussed in RAN2#122 meeting. It is obvious that Option 2 can be supported by the current RAN2 specification without any change, while Option 1 may need some specification change to stop contention resolution timer.

During the discussion in RAN2#122 meeting, some companies argue option 1 is an optimization. But we think it follows the same logic of existing case in current specification.

For the contention resolution of 4-step RACH,  the existing UE behavior defined in TS 38.321 as below could be used as the reference:

*************************************TS38.321 sart*****************************************

2>	else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its TEMPORARY_C-RNTI:

3>	if the MAC PDU is successfully decoded:

4>	stop ra-ContentionResolutionTimer;

4>	if the MAC PDU contains a UE Contention Resolution Identity MAC CE; and

4>	if the UE Contention Resolution Identity in the MAC CE matches the CCCH SDU transmitted in Msg3:

5>	consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;

5>	if this Random Access procedure was initiated for SI request:

6>	indicate the reception of an acknowledgement for SI request to upper layers.

5>	else:

6>	set the C-RNTI to the value of the TEMPORARY_C-RNTI;

5>	discard the TEMPORARY_C-RNTI;

5>	consider this Random Access procedure successfully completed.

4>	else:

5>	discard the TEMPORARY_C-RNTI;

5>	consider this Contention Resolution not successful and discard the successfully decoded MAC PDU.

*************************************TS38.321 end******************************************

In the current specification, termination of Contention Resolution before contention resolution timer expires is already specified. To be more specific, if one UE finds the Msg.4 is not for itself, i.e., the UE Contention Resolution Identity MAC CE in Msg.4 doesn’t match with the CCCH SDU transmitted in its Msg.3, the UE will consider this contention resolution not successful.

For R18 eRedCap UE, Msg.3 based early indication is introduced, so that network can differentiate R18 eRedCap UE from R17 RedCap UE and non-Redcap UE after Msg.3 is received. In case the network identifies the R18 eRedCap UE, the network should schedule a Msg.4 PDSCH transmission with a bandwidth not larger than 5MHz. In other words, if network schedules a Msg.4 PDSCH transmission with a bandwidth larger than 5MHz, the R18 eRedCap UE can consider the scheduled Msg.4 is not for this eRedCap UE. In this case, Contention Resolution is terminated as in legacy, i.e. eRedCap UE considers this Contention Resolution not successful and stops monitoring  PDCCH for Contention Resolution rather than continuing monitoring the PDCCH until contention resolution timer expires. In this way, R18 eRedCap UE can at once perform the Random Access Resource selection procedure after considering this Contention Resolution not successful, which is beneficial for reducing the latency of RACH procedure and the UE power consumption.

In the below table, the pros/cons of different options are summarized.

| Options | Pros | Cons |
| --- | --- | --- |
| Option 1 | - UE power saving and latency reduction - Same logic as legacy behaviour for termination of contention resolution | Minor specification change is needed |
| Option 2 | - No specification change is required | More power consumption and longer latency wrt. the case in Option 1 due to unnecessary monitoring |
| Option 3 | Flexibility |  |

With the above analysis, we think option 1 (i.e. the UE considers the contention resolution not successful when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process) is more reasonable and efficient.

Proposal: A eRedCap UE considers the contention resolution not successful and stop the ra-ContentionResolutionTimer, when the UE detects a PDCCH transmission addressed to its TEMPORARY_C-RNTI with a DCI that schedules a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process, i.e. option 1 is adopted.

Regarding how to capture the corresponding behaviour in specification, e.g. only in MAC, or in both PHY and MAC, could be further proceeded after we agreed the above proposal.

- Conclusion

In this contribution, we discuss the LS on Msg.4 PDSCH transmission to Rel-18 eRedCap UEs from RAN1. Based on the discussion, we have the following observation and  proposal:

Proposal: A eRedCap UE considers the contention resolution not successful and stop the ra-ContentionResolutionTimer, when the UE detects a PDCCH transmission addressed to its TEMPORARY_C-RNTI with a DCI that schedules a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process, i.e. option 1 is adopted.

- References

- R2-2304619_R1-2304262, LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs, RAN1/Ericsson