3GPP TSG-RAN WG2 #123					      		 R2-2307750

Toulouse, France – August 21 - 25, 2023		                   Revision of R2-2305874

Agenda item:	7.9.2

Source: 	Kyocera

Title: 	Considerations for U2U L2 relay operations

Document for:	Discussion

# Introduction

During RAN2#122 meeting, the following agreements were reached:

Agreements:

For Model A discovery, the relay UE should only announce the neighbour UEs for which the SD-RSRP/SL-RSRP between the relay UE and the neighbour UE is above a configured threshold in a discovery announcement message. LS is sent to SA2.

For Model A discovery, upon discovery message reception, remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

For integrated-discovery, when receiving DCR message from one or multiple relay UEs, the target remote UE should consider candidate relay UEs towards which the SL-RSRP is above a configured threshold to respond and that satisfy upper-layer criteria, and select a relay UE from among them.

For non-integrated U2U relay discovery model B, when relay (re)selection is triggered at the remote UE, the discovery transmission may be triggered at the same time to search for candidate relay UEs.

Separate thresholds for SL-RSRP and SD-RSRP are configured for the trigger of U2U relay (re)selection.

Besides the PC5 link quality, RAN2 does not pursue other AS criteria for relay (re)selection.

For the E2E SL-SRB configuration of U2U relay, specified PDCP configuration is used. FFS for the SRAP and PC5 RLC channel configuration for SL-SRB.

AS layer is responsible for QoS split in L2 U2U relay.

Relay UE is responsible for AS layer QoS split in L2 U2U relay.

For OOC U2U relay/remote UE, pre-configuration is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

For RRC_IDLE/INACTIVE U2U relay/remote UE, SIB is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

Authorization for L2 U2U relay operation is supported.

The support of authorization for L2 U2U relay operation does not mean the dedicated configuration for U2U relay has to be supported. Whether the dedicated configuration for U2U relay is supported or not is FFS.

Authorization for L2 U2U relay operation includes: 1) whether the UE is authorized to act as a 5G ProSe Layer-2 U2U Relay UE; 2) whether the UE is authorized to act as a 5G ProSe Layer-2 U2U Remote UE.

Authorization for L3 U2U relay operation is not supported.

The legacy authorization for “5G ProSe Direct discovery” and “5G ProSe Direct communication” can be reused for L3 U2U remote/relay UE.

For the possible use of a short ID in U2U relay, RAN2 will downselect between the following options for identifying the source and destination remote UEs at the SRAP layer:

a)	Single ID, identifying the source and destination remote UEs

b)	Source ID and Destination ID

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

These agreements do not imply agreement to use a short ID.

In this contribution, we consider the some of the remaining issues that need to be addressed for U2U operation, including issues related to discovery and U2N vs U2U relaying.

# Discussion

Based on the conclusions from RAN2#122, certain issues related to U2U discovery and communications were discussed, but majority of the proposals were not treated during online discussions.  These issues are further discussed in the subsequent sections.

2.1.	U2U discovery

One issue that needs to be addressed for U2U discovery is whether an in-coverage relay UE is allowed to transmit both U2N discovery and U2U discovery assuming the respective discovery transmit criteria and upper layer authorization is satisfied. Based on SA2 LS responses on U2N/U2U coexistence [3], SA2 indicated that there is no specific restriction on a relay UE to operate both U2U discovery and U2N discovery as long as the relay UE is capable of both; thus, it also means a relay UE may simultaneously support both types of discoveries, since it’s up to the relay UE’s upper layer decision. In our understanding, since a relay UE can serve as both a U2N relay and a U2U relay, the support for coexistence between the two types to relay UE is already supported, so the issue is whether any AS layer functionalities are needed. Although the choice of discovery procedure may be viewed as completely an upper layer decision, we think there are certain issues that are worth considering.  In particular, the choice for the discovery type may depend on whether the target remote UE is also in-coverage and/or whether the target remote UE is reachable directly by the relay UE. For example, an OOC remote UE that is connected to the gNB via a U2N relay UE may also decide to communicate with a peer UE through a U2U relay connection, using the same relay UE.  The question is whether the remote UE would send a DCR to a relay UE when it is already PC5-RRC connected to the relay UE.

- Proposal 1	RAN2 should consider if the remote UE is expected to send a DCR to a U2U relay when the remote UE is already PC5-RRC connected to the same relay UE for U2N relay connection.

2.2.	U2U relay under PC5-RRC connection

One issue that was discussed in the previous meeting is related to the possible use of a short ID in U2U relay as highlighted in the above agreements.  Based on the offline discussions in [1], the ID used for the SRAP header should not only be applicable to Rel-18 but it should be extendable to multihop relays in the future. In Rel-17, for U2N relay, the local ID was generated by the gNB, so the issue with ID collision isn’t an issue.  For Rel-18 U2U relay, it was proposed in [1] that at least for single hop U2U relaying, the U2U relay performs the ID assignment, which is reasonable since the relay UE has direct access to both hops.  And it’s also reasonable to consider that local ID can be adopted since the relay UE can help to ensure that ID collision won’t occur.  However, with multihop relay, it isn’t clear which entity should assign the short ID and how ID collision is supported; therefore, the baseline assumption for ID assignment should be to use L2ID unless short ID and the entity that performs assignment can be extended to multihop relay.

- Proposal 2	For U2U relay, unless there is further clarification of which entity should perform ID assignment in the case of multihop relays, the baseline assumption should be to use L2ID for Rel-18.

RAN2 has already agreed the following, and it is still FFS how PC5 RLC channel for SL-SRB should be configured.

For the E2E SL-SRB configuration of U2U relay, specified PDCP configuration is used. FFS for the SRAP and PC5 RLC channel configuration for SL-SRB.

In our understanding, once the PC5-RRC connections of the 1st hop and the 2nd hop are established, a DCR message is sent from the source remote UE to the target remote UE to establish PC5 unicast link and this triggers the establishment of the E2E PC5-RRC connection between the two remote UEs. And once the PC5-RRC connection is established, it is necessary to perform HbH and E2E SLRB configuration and RLC channel mapping for both HbH and E2E to support the control plane protocol that was already agreed and captured in Figure 5.5.1-2 of [2]:

Figure 1: Control plane protocol stack for L2 UE-to-UE Relay

In Rel-17, for U2N relay, it is assumed that the SLRB and SL-RLC channel configurations and mappings are handled by the gNB since the relay UE and the remote UE are RRC_CONNECTED, while for SL communication in Rel-16, it is configured by the serving gNB when the UE is in-coverage and by the Tx UE in the OOC scenario.  Although another option would be for the relay UE to handle the configurations, both indirect path and direct path communications need to be supported and for the OOC scenario, it would simplify the procedures by allowing the Tx UE to configure the SLRB and SL-RLC channels.  Therefore, for U2U relay, we should follow the Rel-16 procedure.  There have been discussions that gNB should not be involved in such SLRB configurations for U2U relay for the in-coverage scenario, but since Mode 1 resource allocation is already agreed, it should be reasonable to also allow the gNB to provide the necessary configurations. In case the gNB prefers not to handle these configurations, it should be possible for the gNB to allow the source remote UE to perform the configurations.

- Proposal 3	After E2E PC5-RRC connection establishment, the SLRB and SL-RLC channel configurations and mappings should be handled by the source remote UE during OOC scenario.  It should be further discussed whether gNB is responsible for the configurations when one or more UEs are in-coverage.

In addition to the configuration of SLRB and SL-RLC channels, our understanding is that the SL-RSRP thresholds used by the remote UEs and relay UE are either configured by the serving gNB (if in-coverage) or pre-configured.  However, it’s not clear if we should always expect the configuration and pre-configurations of the SL-RSRP thresholds to be the same and across different UEs.  It’s also unclear if the gNB would configure the SL-RSRP threshold differently depending the QoS flow which may impact the PER.  In particular, since RAN2 has already agreed “RAN2 confirms that multiplexing of the different bearers from the different source remote UEs into the same RLC channel in the second hop is supported”, if we assume the following model in Figure 2 for multiplexing data from different source remote UEs towards the same target remote UE.

For example, in case source remote UE 1 receives configuration from its serving gNB with SL-RSRP threshold as part of the SLRB configuration, it’s not clear if this threshold be applicable to the relay UE and target remote UE.  Additionally, it’s also not yet clear the same also applies to the source remote UE 2 which could have a different SL-RSRP threshold and which threshold the relay UE should use since it’s serving as relay UE for two source remote UEs. We also assume if the SL-RSRP thresholds are applied independently by the remote UEs and relay UE, the radio condition for triggering SL-RLF for the two hops may differ.

- Proposal 4	For the case of multiplexing data from different source remote UEs RAN2 should consider whether the SL-RSRP thresholds used by the remote UEs and relay UE should be applied consistently, and how to handle the case when the SL-RSRP threshold configured by the gNB is different from the pre-configured threshold(s).

With respect to SL-RLF detection, it is already agreed that “Each Remote UE can trigger Relay reselection based at least on current hop quality”; however, it will also be necessary for the relay UE to assist the source remote UE (and target remote UE) with regards to the quality of the second hop, whether the relay UE should inform the source remote UE upon SL-RLF with the target remote UE or whether it should be based on a threshold level.  The legacy notification message could be reused for this purpose.

- Proposal 5	RAN2 should consider which metric should be used by the relay UE to inform the source remote UE/target remote due to drop in quality on the second hop.

Based on the above agreements, U2U relay reselection can be triggered based on the PC5 RSRP between a remote UE and the relay UE falling below a threshold.  Similar U2U relay reselection can also be triggered in case SL-RLF is detected at the remote UE or informed by the relay UE. According to Solution #7 from SA2 [4], relay reselection may be negotiated:

Figure 3: UE-to-UE Relay Reselection Procedure [4]

The relay reselection procedure is useful for reselecting a candidate relay UE that may have a better signal level than that with the existing relay UE.  There are at least two issues that should be considered as part of such a negotiated relay reselection:

- In case U2U service continuity isn’t supported, switching to a new relay UE when the existing relay UE can still support ongoing communication will result in disruption of service between the source and target remote UEs.

- In case on SL-RLF in any of the two hops such a negotiated procedure will not be possible.  An alternate procedure will be needed for relay reselection.

Regarding Case 1, our understanding is that the decision for triggering relay reselection is based on the proper configuration of SL-RSRP threshold and the decision by the upper layer.  The configuration of the SL-RSRP threshold will have a big impact on whether there would be too early or too late triggering of relay reselection. Since the remote UEs and the relay UE are assumed to be non-stationary, the negotiated candidate relay UE may not always work due to dynamic changes in signal level.

During RAN2#121-bis discussions, the following agreement was reached there was also the FFS:

Proposal 16 (modified): When the remote UE receives PC5-RLF indication from the U2U relay UE, it would inform upper layers and rely on upper layers to trigger relay reselection (or not).  FFS if there would be any constraints on the remote UE implementation behaviour to keep or release the PC5 link with the relay UE.

To allow for improved relay reselection, both source and target remote UEs should be allowed to connect with the candidate relay UE without releasing the connection to the existing relay UE (i.e., make-before-break PC5 connection to the two relay UEs). To assist with the reselection of candidate relay UEs with minimal interruption, the neighbor list in the Model A discovery transmitted by the candidate relay UE may also include the PC5 connection status of the remote UEs; otherwise, the two remote UEs may confirm with one another the successful PC5 connections to the same candidate relay UE over the existing relay UE if it’s still available.  In case a candidate relay UE cannot be found, the PC5 communication may continue using the existing relay UE (assuming SL-RLF has not been declared by either of the remote UEs).

- Proposal 6	The remote UE is allowed to be support make-before-break relay reselection (i.e., release the existing relay UE after PC5 connection is established with the candidate relay UE).

Regarding Case 2, when the remote UE detected SL-RLF on the first hop or receives notification from the relay UE on the SL-RLF from the second hop, relay reselection should be triggered. In this situation, negotiated relay reselection will not work.  Furthermore, since the two remote UEs may not detect SL-RLF simultaneously, the source remote UE may reselect a candidate relay UE, which should in turn inform the target remote UE via discovery message.  Upon receiving the discovery message from the candidate relay UE, the target remote UE may release the existing relay UE and establish connection with the candidate relay UE. It may be further discussed what message is used to inform the target remote UE regarding the new candidate relay UE.

- Proposal 7	In case of SL-RLF, the source remote UE should inform the target remote UE via the candidate relay UE of the new path.

Previously, for V2X communication it is assumed once the two UEs moves away from one another, there’s no point to try and reestablish the sidelink connection. However, with U2U relay, even if the U2U connection experiences SL-RLF, it may be possible for the source remote UE and target remote UE to be reconnected via a different U2U relay UE, esp. since the E2E PC5-RRC connection has already been established. For example, the source remote UE may send a PC5-RRC message (SL re-establishment msg) towards the target remote UE either via a newly selected candidate relay UE or directly between the two remote UEs depending on the PC5 radio conditions.

- Proposal 8	RAN2 should consider whether SL re-establishment of the E2E PC5 link should be supported for U2U relay.

In our understanding, SA2 is currently discussing whether U2U path switch from one relay UE to another could be supported.  Although, some companies’ understanding is that there’s no specific requirement in the WID to support path switch, it could be considered if at least the path switch between direct path and indirect path could be supported; otherwise, it means the E2E configuration would need to be released and reconfigured even though the E2E configuration is separate from the configuration between the source remote UE to relay UE (target remote UE to relay UE).

- Proposal 9	RAN2 should consider if U2U path switch between direct path and indirect path can be supported.

# Conclusion

In this contribution, remaining issues for SL U2U issues are highlighted.  RAN2 is kindly asked to take into account the proposals below:

- Proposal 1	RAN2 should consider if the remote UE is expected to send a DCR to a U2U relay when the remote UE is already PC5-RRC connected to the same relay UE for U2N relay connection.

- Proposal 2	For U2U relay, unless there is further clarification of which entity should perform ID assignment in the case of multihop relays, the baseline assumption should be to use L2ID for Rel-18.

- Proposal 3	After E2E PC5-RRC connection establishment, the SLRB and SL-RLC channel configurations and mappings should be handled by the source remote UE during OOC scenario.  It should be further discussed whether gNB is responsible for the configurations when one or more UEs are in-coverage.

- Proposal 4	For the case of multiplexing data from different source remote UEs RAN2 should consider whether the SL-RSRP thresholds used by the remote UEs and relay UE should be applied consistently, and how to handle the case when the SL-RSRP threshold configured by the gNB is different from the pre-configured threshold(s).

- Proposal 5	RAN2 should consider which metric should be used by the relay UE to inform the source remote UE/target remote due to drop in quality on the second hop.

- Proposal 6	The remote UE is allowed to be support make-before-break relay reselection (i.e., release the existing relay UE after PC5 connection is established with the candidate relay UE).

- Proposal 7	In case of SL-RLF, the source remote UE should inform the target remote UE via the candidate relay UE of the new path.

- Proposal 8	RAN2 should consider whether SL reestablishment of the E2E PC5 link should be supported for U2U relay.

- Proposal 9	RAN2 should consider if U2U path switch between direct path and indirect path can be supported.

# References

- R2-2306692 [AT122][419][Relay] Short ID in U2U relay, Ericsson

- TR38.836, “Study on NR sidelink relay”

- R2-2302445 Reply LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U

- TR 23.700-33, “Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)”