3GPP TSG-RAN WG2 Meeting #122	R2-2307772

Toulouse, France, August 21-August 25 2023

Agenda item:	7.6.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On remaining issues for mobility enhancements for IoT-NTN

WID/SID:	IoT_NTN_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

Rel-18 WID [1] for IoT-NTN enhancements includes one objective for mobility enhancements as given below.

Mobility Enhancements: - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC.

Following are the RAN2 agreements in previous meetings related to measurement triggering for IoT-NTN and CHO enhancements to eMTC (see [2], [3] and [4]).

RAN2#122 Agreements: - Extend the neighbour cell information in existing SIBs (not SIB31) to include satellite ID - The system Information modification procedure is not triggered for an update of new SIB on neighbor-cell assistance information. - For NB-IoT, SIBxx is not an essential SIB. UE does not need to consider the cell barred if it is unable to acquire the SIB when scheduled. FFS for eMTC - In RRC IDLE, how to (re-)acquire neighbour cell assistance information is up to UE’s implementation. - The satellite ID in the new SIB is an integer of X bits wherein X depends on the maximum number of satellites to be considered for mobility. - The satellite ID is defined as Radio resource control information element to be used in other configurations. - If a parameter in the common TA parameters is absent, then the value of the parameter is assumed zero. - If Kmac is absent, then the value of Kmac for the neighbor satellite in the list is assumed zero. FFS on further optimization on signaling, e.g., signalling explicit value 0 of Kmac. Agreements: - Reference location and distanceThresh in SIB31. A change of reference location does not trigger SI modification. A UE does not need to get a new reference location as long as ephemeris and Epoch time are valid (in Connected mode the UE relies on T317) Agreements: - For earth-fixed cells, introduce t-ServiceStart for neighbor cells. If UE is aware of the t-ServiceStart of the neighbour cell then may be used (up to UE implementation) to determine when to start measurements of that neighbor cell. - If the serving cell t-service expires, stop T310 (if running) and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310). FFS on discontinuous coverage. - The distance between the UE and a second reference location (e.g. within a neighbour cell) is not taken into account. - R18 location and time based trigger for measurements (for connected mode and for idle) apply to both NB-IoT and eMTC. RAN2#121bis Agreements: - New SIBxx is introduced to broadcast the neighbor cell/satellite information. Agreements via email – from offline 114: - Common TA parameters are broadcast as assistance information for neighbor cell measurements. - Kmac is broadcast as neighbor cell assistance information. - For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN). Agreements online: - Introduce satellite ID for the satellite in a list in new SIB-xx. FFS on the details of the new IE - For eMTC NTN, for fixed cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). - For eMTC NTN, for moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode. - SIB3 is extended to include the reference location and distanceThresh.

RAN2-121 Agreements: 1. Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN). 2. Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN). Agreements: 3. We don’t introduce any new low mobility criterion for enhanced mobility in Rel-18.

RAN2-120 Agreements on NB-IoT: 1. For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion). 2. At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage) 3. RAN2 will not specify the condition of stopping UE measurement before t-Service. 4. For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS) 5. For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation. Agreements on eMTC: 6. CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.: UE is allowed to perform HO only during T1 to T2. 7. For eMTC NTN, introduce a location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold. (similar to condEventD1 in NR) 8. For eMTC NTN, introduce event A4 based conditional trigger (similar to condEventA4 in NR). 9. FFS whether time and location-based trigger conditions may be configured independently (i.e., without a jointly configured event A4 measurement condition) for eMTC NTN.

# 2  Discussion

## 2.1 New SIB for neighbor cell assistance information

In RAN2-121-bis-e meeting new SIB will be introduced that included neighbor cell assistance information. RAN2 also agreed to include common TA and Kmac parameters in the new SIB.  Whether the new system information is modified using regular system information modification procedure or timer-based modification as similar to SIB31 is not concluded.

The common TA and ephemeris information of neighbor cells needs to be regularly updated when the information is updated at the neighbor cell.  For serving-cell ephemeris and TA parameters, UE in idle mode will read SIB-32 prior to system access and can maintain the information based on SIB31 validity duration before trying to access the same information prior to the next RACH access.  But the neighbor-cell assistance information is needed only when the UE starts neighbor-cell measurements which happen during the cell-reselection situation. Hence the UE can acquire the new SIB for neighbor-cell assistance information whenever needed for cell reselection measurements without maintaining any timer for the same.  This timer may be needed only if the UE needs to re-acquire the system information if the cell-reselection procedure does not result in the switching of cells. As this is not a frequent event the validity timer for new SIB containing neighbor-cell system information is not needed.

Observation 1: New SIB for neighbour-cell assistance information can be read by UE implementation whenever cell-reselection measurement starts without any dependency on the validity timer.

Proposal 1: The validity timer parameter is not included in the new system information for neighbor-cell assistance information.  It is up to network implementation to maintain valid information about neighbor cells in the new system information.

For connected mode neighbor-cell measurements, if the new system information contents are needed for optimum measurements, the UE may need to switch to system information acquisition prior to obtaining the new system information. Because the values acquired from the new SIB are valid anymore at the time of connected mode measurement triggering. Similar to the timer started for acquiring SIB31 in connected mode, the new timer may be needed to resume connected mode operation. Whether UE can continue to monitor downlink along with the re-acquisition of new SIB needs to be decided based on UE’s capability. In the case of NB-IoT, UE may have the connected mode operation in the non-anchor carrier and it may need to switch to an anchor carrier to acquire the new SIB.

Proposal 2: RAN2 to discuss additional UE capability needed for acquiring the new SIB for connected mode measurements.

Regarding the content of the new SIB, during the email discussion in [5] the following proposals were not agreed:

Proposal 3 (10/13): For fixed cell, the cell start time of the neighbor cell is broadcast. Proposal 5 (8/13): For fixed cell, the cell stop time of the neighbor cell is not broadcast. Proposal 6 (8/13): for moving cell, reference location and distance threshold of neighbor cell are broadcast. Proposal 9 (13/14): For fixed cell, same mechanism of location-based connected mode measurement initiation can also be used in RRC_IDLE (like in NR-NTN). Proposal 10 (12/14): For moving cell, same mechanism of location-based connected mode measurement initiation can also be used in RRC_IDLE. FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode.

In a similar manner, Proposal#6 had broad support, but it was not agreed. In this case, the proposal aimed at including neighbor cell reference location and distance threshold in the new SIB.

For moving cells, IoT UEs can make use of the serving cell reference location and the corresponding threshold to anticipate when the serving cell will stop serving the area. Using satellite ephemeris, cell reference location and epoch time in system information, the UE can propagate over time the satellite’s motion and the corresponding cell reference locations. Based on this reasoning, the following agreement was taken at the last RAN2#121-bis-e meeting via email discussion (see [5]):

Agreements via email – from offline 114: For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN).

At RAN2#120 meeting (November 2022), the following agreement was made for eMTC NTN:

Agreement at RAN2#120 (November 2022) For eMTC NTN, introduce a location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold. (similar to condEventD1 in NR).

At least for eMTC NTN, this agreement considers that eMTC UEs could be configured with a location-based trigger based on condEventD1 in NR. Furthermore, the agreement has the following implications for eMTC UEs:

- UEs should have valid information of the neighbor cell’s reference location and the corresponding distance threshold. This feature is only possible if the location information of the neighbor cell is broadcast.

- UEs should be capable of monitoring the distance between its own position and the serving cell’s reference location without compromising UE’s power consumption. This feature is only possible if the UE can derive the trajectory of the serving cell.

- UEs should be capable of monitoring the distance between its own position and the neighbor cell’s reference location without compromising UE’s power consumption. This feature is only possible if the UE can derive the trajectory of the neighbor cell.

Observation 2: eMTC UEs cannot acquire valid location information of the neighbor cells to comply with RAN2 agreements.

Considering the current RAN2 agreements, only the second point is possible. Thus, proposals 5 and 6 should be agreed in order to comply with RAN2 agreements and the listed points above.

## 2.2 Connected mode measurements for NB-IoT-NTN

In RAN2#122 and RAN2#121 meetings the following agreements, that are relevant for connected mode measurements, were decided:

Agreements: - The distance between the UE and a second reference location (e.g. within a neighbour cell) is not taken into account. - R18 location and time based trigger for measurements (for connected mode and for idle) apply to both NB-IoT and eMTC.

Agreements: 1. Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN) 2. Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN).

For quasi-Earth-fixed cells (EFC) and Earth-moving cells (EMC) deployments, it has been agreed that the NB-IoT UEs can acquire the serving cell reference location and a distance threshold for cell measurement initiation. Particularly for EFC, location-based measurement triggers can be beneficial for IoT UEs moving away from serving cell. A UE close to the cell edge, and moving towards another cell, could evaluate its own distance to the serving cell’s reference location and, based on a given threshold, determine if further actions should be taken. However, a UE solely configured with a location-based trigger cannot determine when the serving cell is going to stop serving the area. If serving cell and neighbor cell are about to disappear, a smart UE could jointly evaluate time-based and location-based conditions and decide to wait and handover towards the new cell.

Proposal 3: For quasi-Earth fixed cells, connected mode measurements to benefit from jointly use of time-based and location-based triggers.

In RAN2#120 meeting and RAN2#121-bis meeting, the following was agreed regarding connected-mode measurements in Earth-moving cells:

Agreement in RAN2#120 For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation. Agreement in RAN2#121bis For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN).

Later during RAN2#122 meeting, and only for Earth-fixed cells, the agreement below was made:

Agreement in RAN2#122 For earth-fixed cells, introduce t-ServiceStart for neighbor cells. If UE is aware of the t-ServiceStart of the neighbour cell then may be used (up to UE implementation) to determine when to start measurements of that neighbor cell.

The agreement defines the UE can consider the neighbor cell t-ServiceStart to determine when to start measurements towards the target cell, which is useful to ensure the measurements are timely i.e. not occurring before the cell is available and also not occurring too late. However, the “(up to UE implementation)” makes it rather unclear whether RAN4 shall proceed with defining requirements related to the measurements and therefore we think it would be good to consider clarifying the intention and potentially omit the implementation aspect.

Proposal: RAN2 to consider to clarify the intention of “(up to UE implementation)” in the RAN2#122 agreement related to use of t-ServiceStart of the neighbor cell measurements.

In RAN2#121-bis meeting, it was agreed that for moving cells, the UE can propagate over time the trajectory of the serving cell based on satellite ephemeris and epochTime (similarly as in NR-NTN). The agreement allows the UE to estimate how the serving cell’s reference location moves during the time that the UE is camping in that cell and solves the issue of the cell reference location becoming outdated. In our view, we think that this knowledge could be further exploited. Figure 1  show an example on how the estimation of the cell movement can be used. In this case, the UE could determine not only when to start neighbor cell measurements but also when to relax or pause serving cell measurements. This mobility issue was barely addressed in RAN2#120 meeting and the details were left open up to UE implementation. The specification should define when the UE relaxes serving cell and starts neighbor cells measurements given that UE’s power consumption is a critical feature for IoT-NTN.

Observation 7: For Earth-moving cells, even though the UE can derive the movement of the serving cell, it is not specified whether the UE could initiate, relax or pause cell measurements based on that estimation.

Proposal 4: For Earth-moving cells, the UE can relax/pause serving cell measurements and stop/initiate neighbor cell measurements based on the trajectory of the serving cell.

Figure 1UE performing cell measurements relaxation using the knowledge on the end of coverage of the serving cell.

For Earth-fixed cells, it was agreed that the UE could use t-ServiceStart to determine when to start measurements of the target cell (if t-ServiceStart is available and valid). However, similarly as in Earth-moving cells, nothing was agreed on how this parameter could be used for serving cell measurements. Similarly as in Earth-moving cells, if the UE is aware of when the target cell becomes available, the UE could relax or pause serving cell measurements until t-ServiceStart. For that reason, we propose the following:

Proposal 5:  For Earth-fixed cells, the UE can relax/pause serving cell measurements until t-ServiceStart (if available).

## 2.3 RLF Enhancements for IoT-NTN

For moving cell scenarios when radio link problem is detected at the source cell, the chances for recovering back on the source cell is minimum considering the predictable coverage change. The current UE behaviour related to restarting the timer based on serving cell measurements can be enhanced considering the availability of target cell measurements. If connected mode measurements for the target cells are available at the time of start of T310 timer UE should be allowed to skip further monitoring for recovery and declare early RLF to start the re-establishment procedure earlier.

Proposal 6: Fast RLF declaration based on measurement availability is considered for LEO scenario for NB-IoT NTN.

## 2.4 Idle mode measurements for eMTC-NTN

Regarding eMTC NTN and idle mode, the following agreements were made in the last RAN2#121-bis-e meeting:

Agreements online: - For eMTC NTN, for fixed cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). - For eMTC NTN, for moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode.

Similar as in NR NTN, RAN2 agreed for eMTC UEs in idle mode to re-use the location-based measurements framework for cell re-selection mechanism. In the email discussion in [5], several companies showed concerns about the UE updating its GNSS position frequently, which can alter UE’s power consumption. In our view, the location information should be used as assistance information for cell reselection and be used optionally by UE if it has valid GNSS location as per its regular GNSS acquisition. Thus, UE should not be mandated to acquire GNSS locations for the purpose of cell re-selection.

Proposal 8: RAN2 to confirm that UE does not require to update GNSS location for the purpose of cell re-selection.

Proposal 9: For idle mode cell re-selection, the network can provide information to enable location-based measurement triggering, but it is up to the UE whether to use it or use legacy solution.

# 3	Conclusion

Observation 1: New SIB for neighbour-cell assistance information can be read by UE implementation whenever cell-reselection measurement starts without any dependency on the validity timer.

Proposal 1: The validity timer parameter is not included in the new system information for neighbor-cell assistance information.  It is up to network implementation to maintain valid information about neighbor cells in the new system information.

Proposal 2: RAN2 to discuss additional UE capability needed for acquiring the new SIB for connected mode measurements.

Observation 2: eMTC UEs cannot acquire valid location information of the neighbor cells to comply with RAN2 agreements.

Proposal 3: For quasi-Earth fixed cells, connected mode measurements to benefit from jointly use of time-based and location-based triggers.

Proposal: RAN2 to consider to clarify the intention of “(up to UE implementation)” in the RAN2#122 agreement related to use of t-ServiceStart of the neighbor cell measurements.

Observation 7: For Earth-moving cells, even though the UE can derive the movement of the serving cell, it is not specified whether the UE could initiate, relax or pause cell measurements based on that estimation.

Proposal 4: For Earth-moving cells, the UE can relax/pause serving cell measurements and stop/initiate neighbor cell measurements based on the trajectory of the serving cell.

Proposal 5:  For Earth-fixed cells, the UE can relax/pause serving cell measurements until t-ServiceStart (if available).

Proposal 6: Fast RLF declaration based on measurement availability is considered for the LEO scenario for NB-IoT NTN.

Proposal 8: RAN2 to confirm that UE does not require to update GNSS location for the purpose of cell re-selection.

Proposal 9: For idle mode cell re-selection, the network can provide information to enable location-based measurement triggering, but it is up to the UE whether to use it or use legacy solution.

# References

- RP-223519, WID on IoT NTN enhancements, 3GPP TSG RAN Meeting #98, December, 2022.

- R2-2300002, “Report of 3GPP TSG RAN2 Meeting #120”,  3GPP TSG RAN2 Meeting #120 Meeting, 14-18 November, 2022.

- R2-2301901, “Report from Break-out session on NR-NTN, IoT-NTN and RedCap” 3GPP TSG RAN2 Meeting #121 Meeting, February 27th-March 3rd 2023.

- R2-2304201, “Report from Break-out session on NR-NTN and IoT-NTN” 3GPP TSG RAN2 Meeting #121bis-e Meeting, April 17th-26th 2023.

- R2-2304257, “Report of [AT121bis-e][114][IoT NTN Enh] Neighbour cell measurements (Qualcomm)” 3GPP TSG RAN2 Meeting #121bis-e Meeting, April 17th-26th 2023.

- 3GPP Technical Specification 38.331, version 17.1.0, June 2022.