3GPP TSG-RAN WG2 Meeting #123	R2-2307788

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.5.3

Source:	Nokia, Nokia Shanghai Bell, Continental Automotive

Title:	DRX enhancements for XR

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

DRX cycle alignment and SFN wrap around issues for XR power saving enhancements have been discussed in the previous RAN2 meetings with the following agreements reached:

Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications. Not use broadcast signalling for counter and reference SFN in XR.

We further discuss those issues in this contribution.

# 2	DRX cycle alignment

Based on the agreement of defining DRX cycle based on rational numbers, considering we do not use REAL value for ASN.1, it could be enough to configure an integer hyper cycle (e.g. 50ms) and number of cycles within the hyper cycle (e.g. 3), then with the floor operation as proposed to the MAC formula it would result in the pattern of 17, 17, 16 as shown in the table above without introducing the counter to check at which cycle when an adjustment is needed. According to the typical frame rate captured in the stage 2 running CR (R2-2304393), possible combinations of hyper cycle and number of cycles per hyper cycle (100, 3), (200, 9), (50, 3), (125, 9), (100, 9), (25, 3) could cover the frame rate of 30fps, 45fps, 60fps, 72fps, 90fps, 120fps. We could also consider configure the hyper cycle and number of cycles as separate parameters to allow any combination and be future proof. It is left to NW implementation to configure the combination based on the traffic characteristic.

Proposal 1: Configure hyperCycle and numberOfCycles per hyper cycle as separate parameters with possible values of{ms25, ms50, ms100, ms125, ms200, spare3, spare2, spare1} for hyper cycle and {ms3, ms9, spare2, spare1} for numberOfCycles per hyper cycle. DRX cycle is defined as hyperCycle / numberOfCycles.

Proposal 2: DRX start offset is separately configured from the DRX cycle and only the values from 0 to floor(hyperCycle/numberOfCycles) – 1 are applicable depending on the configured hyperCycle and numberOfCycles values.

TP for RRC:

Configuring DRX hyper cycle requires enhancements to the DRX-Config IE defined in TS 38.331 as follows: DRX-Config information element -- ASN1START -- TAG-DRX-CONFIG-START DRX-Config ::= SEQUENCE { -- Legacy DRX parameters skipped ..., drx-CycleRational-r18 SEQUENCE { hyperCycle-r18 ENUMERATED {ms25, ms50, ms100, ms125, ms200, spare3, spare2, spare1}, numberOfCycles-r18 ENUMERATED {ms3, ms9, spare2, spare1} } drx-StartOffset-r18 INTEGER(0..65), ..., } } -- TAG-DRX-CONFIG-STOP -- ASN1STOP drx-CycleRational Used to configure DRX short and long cycles with non-integer values. drx-LongCycle or drx-ShortCycle corresponds to hypercycle/ numberOfCycles. When this field is configured, drx-LongCycleStartOffset and drx-ShortCycle in drx-Config shall be ignored.. drx-StartOffset Used to configure drx Start Offset when the DRX cycle is with non-integer values. Only the values from 0 to floor(hypercycle/ numberOfCycles) – 1 are applicable depending on the configured hypercycle and numberOfCycles values. When this field is configured, drx-LongCycleStartOffset shall be ignored..

For DRX cycles represented as rational numbers, there are different ways to implement the modulo operation without producing rounding errors. One such example is to apply the definition as discussed in [3]:

x mod y=x-y×floorxy

which is a well-established formula that can be found in many authoritative references on mathematics and computer science as well as online resources such as MathWorld and Wikipedia. However, the modulo operation by using this equation requires the multiplication of a fractional number with an integer number. Depending on the numerical precision used for the representation of the fractional number in a given computation platform, e.g., MATLAB or Excel, the formula output different results.

To better illustrate the problem statement, hereafter, we present a detailed analysis of the formula specification and potential issues if modulo operation is implemented as suggested by Qualcomm et al in [3]. Without loss of generality, we describe the example for the long cycle case, and the target formula is

floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset

Now for the sake of convenience, let us define the following shorthand:

- A = [(SFN × 10) + subframe number]: an integer number.

- B / C = drx-LongCycle: a rational number represented by the ratio of two integers (B and C).

- Z = drx-StartOffset: an integer number.

Then the above formula reduced to

floorA modulo BC=Z

Now if we apply the formula proposed by Qualcomm et al in [3], we have

floorA-BC×floorA×CB=Z                                                       (1)

Therefore, we analyze equation (1) as follows. Firstly, A×C is an integer since it is the product of two integers, and B is an integer, which results in that A×CB is a rational number as well. Thus, floorA×CB is an integer number. However, the fraction BC is still a rational number, in particular, multiplying an integer floorA×CB by a fractional number BC result in different values depending on the numerical precision used to represent the fractional number in a given computation platform. To be more explicit, the rational number BC and recurrent decimal (e.g., 50/3 = 16.6666…) represented in machine can cause rounding errors. The numerical representation of the fractional number may cause the product to wrap up the nearest integer toward infinity. This is seen as the machine-dependent problem which cannot be overcome in the practical implementation.

- For example, let F be a fractional number and I be an integer number. The product of F×I can result in different values depending on the representation of the fractional F in a computer machine. Let I=50 and F=13 (note that this is the typical example 503=16.6666…):

- If the fractional number F is represented as F=13=0.3 by a computer machine, we get F×I=50×0.3=15, which result in floorF×I=floor50×0.3=15.

- If the fractional number F is represented as F=13=0.33 by a computer machine, we get F×I=50×0.33=16.5, which result in floorF×I=floor50×0.33=16.

- If the fractional number F is represented as F=13=0.333 by a computer machine, we get F×I=50×0.333=16.6, which result in floorF×I=floor50×0.333=16.

As discussed above, to avoid the numerical errors due to the numerical precision of fractional numbers by a computer machine, the DRX formula with rational numbers may adopt the following implementation:

floorA×C-B×floorA×CBC=Z                                                                  (2)

In this way, the rounding error caused by floor operation of a rational number BC multiplied by an integer number floorA×CB will be avoided.

To fully demonstrate the rounding errors caused by using (1), we present the numerical results in Figure 1 below. In this numerical example, DRX hyper cycle is B = 50 ms, and number of cycles per hyper cycle C = 3, so drxLongCycle is represented as a rational number 50/3 ms. In this case, it is clearly show that there are 3 numbers calculated as -1 for drx-StartOffset, which is in theory impossible. However, by using the proposed formula in (2), the numerical errors can be overcome.

Figure 1: Numerical illustrations of rounding errors by using equation (1) for DRX long cycle

For Short Cycle case, denote B / C = drx-ShortCycle, which is a rational number represented by the ratio of two integers (B and C), the following formula is obtained based on the same methodology:

floorA×C-B×floorA×CBC=floorZ×C-B×floorZ×CBC                                                 (3)

Proposal 3:  in addition to the the MAC formula change with floor operation, capture that Floor operation of modulo of a rational number, floorA modulo BC shall be implemented as floorA×C-B×floorA×CBC to avoid rounding error, where B = hyperCycle and C = numberOfCycles in the hyper cycle:

- if drx-CycleRational is configure: 2> if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset: 2> if the Short DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-ShortCycle)) = floor ((drx-StartOffset) modulo (drx-ShortCycle)): Floor operation of modulo of a rational number, floorA modulo BC shall be implemented as floorA×C-B×floorA×CBC to avoid rounding error, where B = hyperCycle and C = numberOfCycles in the hyper cycle.

## 2.2	SFN wraparound

It was agreed to use a counter which is increased at every SFN wrap around.

The remaining question is how to make sure the initial counting starts from the same SFN at the UE and the NW side considering the RRC message processing time and potential retransmissions.

Some possible options have been proposed:

-	Option 1: indicate Hyper-SFN in SIB as done for eDRX.

-	Option 2: indicate Hyper-SFN number with dedicated RRC signalling when DRX is configured;

-	Option 3: parameter drxReferenceSFN similar to timeReferenceSFN introduced for Rel-16 CG is used as reference;

Option 1 was excluded from the previous RAN2 meeting.

The main difference between option 2 and option 3 are the value range needed for the reference point. Option 2 would be with the range of e.g. (0..1023), while option 2 with only one code point of sfn512 and 0 is assumed if not present. Option 3 seems to be simpler and enough as it does not matter which Hyper SFN it really is as long as the NW and UE using the same timing reference.

timeReferenceSFN-r16 ENUMERATED {sfn512} OPTIONAL -- Need S timeReferenceSFN Indicates SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration, see TS 38.321 [3], clause 5.8.2. If the field timeReferenceSFN is not present, the reference SFN is 0.

Proposal 4: option 3 with drxReferenceSFN is used as the reference for SFN wrap around.

Proposal 5: The formula is changed as follows if drxReferenceSFN is configured:

- if drx-ReferenceSFN is configured: 2> if the Long DRX cycle is used for a DRX group, and [(N × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) = (drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-LongCycle): 2> if the Short DRX cycle is used for a DRX group, and [(N × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-ShortCycle):

Proposal 6: When both hyper cycle and drxReferenceSFN are configured, the formula for Long DRX Cycle would be as follows:

- if drx-ReferenceSFN and drx-CycleRational are configured: 2> if the Long DRX cycle is used for a DRX group, and floor([(N × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle)) = floor ((drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-LongCycle)): 2> if the Short DRX cycle is used for a DRX group, and floor ([(N × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) = floor ((drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-ShortCycle)):

Proposal 7: the legacy formula should be left untouched and which new one is used depends on whether either hyper cycle or drxReferenceSFN or both are configured.

TP for RRC for drxReferenceSFN:

Configuring DRX Reference SFN requires enhancements to the DRX-Config IE defined in TS 38.331 as follows: DRX-Config information element -- ASN1START -- TAG-DRX-CONFIG-START DRX-Config ::= SEQUENCE { ..., Legacy DRX parameters drxReferenceSFN ENUMERATED {sfn512} OPTIONAL -- Need S } -- TAG-DRX-CONFIG-STOP -- ASN1STOP

# 3	Conclusion

The issues on DRX cycle alignment and SFN wraparound is discussed in this contribution with the following proposals proposed:

Proposal 1: Configure hyperCycle and numberOfCycles per hyper cycle as separate parameters with possible values of{ms25, ms50, ms100, ms125, ms200, spare3, spare2, spare1} for hyper cycle and {ms3, ms9, spare2, spare1} for numberOfCycles per hyper cycle. DRX cycle is defined as hyperCycle / numberOfCycles.

Proposal 2: DRX start offset is separately configured from the DRX cycle and only the values from 0 to floor(hyperCycle/numberOfCycles) – 1 are applicable depending on the configured hyperCycle and numberOfCycles values.

Proposal 3:  in addition to the the MAC formula change with floor operation, capture that Floor operation of modulo of a rational numbers floorA modulo BC shall be implemented as floorA×C-B×floorA×CBC to avoid rounding error, where B = hyperCycle and C = numberOfCycles in the hyper cycle:

- if drx-CycleRational is configure: 2> if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset: 2> if the Short DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-ShortCycle)) = floor ((drx-StartOffset) modulo (drx-ShortCycle)): Floor operation of modulo of a rational number, floorA modulo BC shall be implemented as floorA×C-B×floorA×CBC to avoid rounding error, where B = hyperCycle and C = numberOfCycles in the hyper cycle.

Proposal 4: option 3 with drxReferenceSFN is used as the reference for SFN wrap around.

Proposal 5: The formula is changed as follows if drxReferenceSFN is configured:

- if drx-ReferenceSFN is configured: 2> if the Long DRX cycle is used for a DRX group, and [(N × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) = (drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-LongCycle): 2> if the Short DRX cycle is used for a DRX group, and [(N × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-ShortCycle):

Proposal 6: When both hyper cycle and drxReferenceSFN are configured, the formula for Long DRX Cycle would be as follows:

- if drx-ReferenceSFN and drx-CycleRational are configured: 2> if the Long DRX cycle is used for a DRX group, and floor([(N × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle)) = floor ((drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-LongCycle)): 2> if the Short DRX cycle is used for a DRX group, and floor ([(N × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) = floor ((drx-ReferenceSFN × 10 + drx-StartOffset) modulo (drx-ShortCycle)):

Proposal 7: the legacy formula should be left untouched and which new one is used depends on whether either hyper cycle or drxReferenceSFN or both are configured.

# Reference

[1] R2-2302514, DRX enhancements for XR, Qualcomm Incorporated, MediaTek, CATT, vivo, NEC

[2] R2-2303861, DRX cycle alignment for XR, Nokia, Nokia Shanghai Bell, CMCC, China Unicom, DENSO CORPORATION, Ericsson, Intel, Google Inc., Huawei, HiSilicon, Samsung, Xiaomi

[3] R2-2304709, DRX enhancements for XR, Qualcomm Incorporated, MediaTek, CATT, vivo, NEC, Meta