3GPP TSG-RAN WG2 Meeting #123	R2-2307914

Toulouse, France, August 21-25, 2023

Source:	III

Title:	Discussion on BSR enhancements for XR

Agenda item:	7.5.4.1

Document for:	Discussion

- Introduction

During the RAN#99 meeting [1], the following objectives were noted:

Specify the enhancements related to capacity: - Delay reporting of buffered data in uplink (RAN2);

Topics relative to XR-specific capacity issues was raised as mentioned above. In this contribution,”How will the delay/remaining time reporting work?” is addressed.

- Discussion

In the RAN#99 meeting [1], the following objectives were noted:

Specify the enhancements related to capacity:

-	Delay reporting of buffered data in uplink (RAN2);

To support delay reporting of buffered in uplink, the following issues need to be further considered:

- New BSR trigger event for BSR with DSR,

- The urgent BSR reporting,

- Impact from intra-UE prioritization on DSR.

  - New BSR trigger event for BSR with DSR

XR traffic from the application layer will be processed into several PDU sets. Each PDU set can consist multiple IP packets for XR traffic, i.e. I-frame/I-slice(s) or P-frame/P-slice(s). Each IP packet will become a PDU set and every PDU Session has its QoS rule, including the requirement of PDU Set Delay Budget (PSDB) and PDU Set Error Rate (PSER). To increase capacity, UE/gNB can discard PDU sets after any of them have violate the PSDB constrain based on the evaluation of XR traffic.

Hence, add a new BSR entry/table to share the remaining delay time information of each PDU set between UE and gNB side (e.g. Delay Status Reporting, DSR), which can also help data scheduling to improve the XR capacity, the accurate remaining delay budget information of PDU(s) could be derived from the time stamp of the input XR PDU set(s).

Since the transmitting of the DSR needs to be timely, otherwise the remaining delay time may be inaccurate for gNB, so how to trigger the BSR becomes very important. In the current release, there are three types of BSR trigger events:

- Regular BSR: the UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or none of the logical channels which belong to an LCG contains any available UL data.

- Padding BSR: UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its sub-header,

- Periodic BSR: the periodicBSR-Timer expires.

However, the above three triggering events cannot guarantee that the BSR will be triggered when the remaining delay time is critical. Therefore, a new BSR trigger event is required.

Observation 1: A new BSR trigger event for BSR with DSR is required.

In order for the BSR with DSR to be sent in time, the BSR should be triggered immediately when the remaining delay time is critical. Therefore, the threshold of remaining delay time can be used to trigger the event of BSR. When the least remaining delay time in LCG is below a threshold, BSR with DSR should be triggered. Moreover, when transmitting BSR with DSR, the content of BSR should not be the data size of high priority in LCG, instead, it should be the data size of the least remaining delay time.

Proposal 1: The threshold of remaining delay time can be used to trigger the event of BSR. When the least remaining delay time in LCG is below a threshold, BSR with DSR should be triggered.

Proposal 2: When transmitting BSR with DSR, the content of BSR should not be the data size of high priority in LCG. Instead, it should be the data size of the least remaining delay time.

  - The urgent BSR reporting

The existed BSR report is either dynamically allocated by the gNB or periodically scheduled. When a UE has a PDU set and its remaining delay time is almost exhausted, in such urgent situation, the UE needs to quickly notify the gNB, to let the gNB knows that the UE has an urgent PDU set to be scheduled for transmission.

For this reason, a straightforward solution is to make a threshold of the remaining delay time to decide the urgent PDU set, and transmit the “urgent data indication” which can piggyback on the SR or HARQ feedback. As a result, the gNB can get the information in time to schedule the urgent PDU set for transmission.

Observation 2: If a UE has a PDU set and its remaining delay time is almost exhausted, the UE needs to quickly notify the gNB, to let the gNB knows that the UE has an urgent PDU set to be scheduled for transmission.

Proposal 3: Make a threshold of the remaining delay time to decide the urgent PDU set, and transmit the “urgent data indication” which can piggyback on the SR or HARQ feedback.

  - Impact from intra-UE prioritization on DSR

In R16 IIoT, the "autonomous transmission (autonomousTx)" mechanism has been well discussed. The purpose of the autonomousTx is to solve the intra-UE prioritization issue, that is, when two uplink grants conflict, if one of the uplink grant is a low priority configured grant, another uplink grant with high priority will be successfully transmitted, while the low priority configured grant must wait until the next cycle of the configured grant with the same TB size and HARQ process ID for uplink transmission (as Fig 1.).

Fig 1. Autonomous transmission

If the UE perform the autonomousTx, and DSR is triggered for the low priority configured grant, there are two situation for the remaining delay time:

- MAC PDU is not delivered to the lower layer:

  - If DSR is not triggered due to urgency (remaining delay time is almost exhausted), the DSR must be recalculated, and the remaining delay time need to be updated by subtracting the time of the configured grant period.

  - If DSR is triggered due to urgency (remaining delay time is almost exhausted), the low priority configured grant should be prioritized.

- MAC PDU is delivered to the lower layer:

  - In this situation, the DSR cannot be recalculated.

Proposal 4: When the UE performs autonomousTx, if DSR is not triggered due to urgency (remaining delay time is almost exhausted) the remaining delay time is recalculated only if the MAC PDU is not delivered to the lower layers.

Proposal 5: When the UE performs autonomousTx, if the MAC PDU is not delivered to the lower layers, and DSR is triggered due to urgency (remaining delay time is almost exhausted), the deprioritized configured grant should be prioritized.

Proposal 6: When two uplink grants conflict, if the low priority CG’s MAC PDU is delivered to the lower layers, the DSR cannot be recalculated.

- Conclusion

The related proposals and observations from above discussions are below:

Observation 1: A new BSR trigger event for BSR with DSR is required.

Proposal 1: The threshold of remaining delay time can be used to trigger the event of BSR. When the least remaining delay time in LCG is below a threshold, BSR with DSR should be triggered.

Proposal 2: When transmitting BSR with DSR, the content of BSR should not be the data size of high priority in LCG. Instead, it should be the data size of the least remaining delay time.

Observation 2: If a UE has a PDU set and its remaining delay time is almost exhausted, the UE needs to quickly notify the gNB, to let the gNB knows that the UE has an urgent PDU set to be scheduled for transmission.

Proposal 3: Make a threshold of the remaining delay time to decide the urgent PDU set, and transmit the “urgent data indication” which can piggyback on the SR or HARQ feedback.

Proposal 4: When the UE performs autonomousTx, if DSR is not triggered due to urgency (remaining delay time is almost exhausted) the remaining delay time is recalculated only if the MAC PDU is not delivered to the lower layers.

Proposal 5: When the UE performs autonomousTx, if the MAC PDU is not delivered to the lower layers, and DSR is triggered due to urgency (remaining delay time is almost exhausted), the deprioritized configured grant should be prioritized.

Proposal 6: When two uplink grants conflict, if the low priority CG’s MAC PDU is delivered to the lower layers, the DSR cannot be recalculated.

References

[1] 	RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm.