3GPP TSG-RAN WG2 #123	Tdoc R2-2307934

Toulouse, France, August 21st – 25th, 2023

Agenda item:	7.18.3

Source: 	Ericsson, T-Mobile USA, Deutsche Telekom

Title:  	Handling BWP restrictions in MT-SDT

Document for:	Discussion and Decision

# Introduction

In the WID [1] for MT-SDT it is stated that the objective is

“Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3]

- MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RACH procedure based and CG-SDT procedure based UL response;

MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.”

This contribution primarily discusses the scheduling restrictions owing to bandwidth part constraints in MT-SDT. We further propose some solutions to enhance the transmission capability for MT-SDT.

# Discussion

## BWP Restriction in MT-SDT

Rel-17 agreements restrict small data transmissions over initial Bandwidth part (BWP); the reason being to render SDT procedure as close to legacy initial access procedure as possible.

For UL transmissions initiated as part of MO-SDT, frequency resources across the entire initial UL BWP are available. However, for DL transmission, initial BWP operation restricts the frequency domain resource allocation to the size of CORESET 0. In specification, this is detailed shown below.

For first DL MT-SDT data, the frequency domain resource assignment is provided in TS 38.212 [3] as follows.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment – bits - is the size of CORESET 0

The same restriction is applicable for subsequent transmissions as well. In TS 38.212, this is provided as follows.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment – bits where is given by clause 7.3.1.0 Where clause 7.3.1.0 is defined as follows: Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

According to TS 38.331 [2], the UE applies this restriction until after the reception of RRCSetup/RRCResume/RRCReestablishment when it switches to Connected mode.

Therefore, we make the following observation.

MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC Inactive state.

For 30kHz subcarrier spacing, this means the scheduler can allocate a maximum of up to 48 PRBs within CORESET 0 [4]. If no DL quality indication is introduced in MT-SDT, a conservative MCS should be used to schedule MT-SDT PDSCH and PDCCH, e.g with low MCS or even MCS index 0 and high CCE aggregation level (this can be seen from Figure 1, in Sec 2.2). Note that at MCS 0, the TB allocation can be as small as 1320 bits for 48 PRBs. This results in a larger number of subsequent transmissions as can be seen in Figure 2, Sec 2.2.

MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions.

Since higher number of subsequent transmissions considerably reduce the energy efficiency of SDT, RAN2 should discuss solutions to handle the BWP restriction in MT-SDT. There are different options to handle this, as listed below.

Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data

This option involves considerable changes to RAN 1 specifications: especially in TS 38.212 and TS 38.214, and some minor modifications to TS 38.331. In addition, this option would require BWP switching and it is not clear how this could be performed prior to reception of Msg4.

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

An indicator of the channel quality from the UE allows the scheduler to adapt to better modulation and coding rates at good radio channel conditions. Even with a restriction to 48 PRBs, this allows for considerably larger TB sizes. The details of this option are discussed in section 2.2.

If no standard changes are done, it would require MT-SDT data volumes to be substantially lower than that for MO-SDT (MO-SDT allowed DVT to be as high as 96000 bytes) to be efficient compared to going to connected mode. We think that the restriction reduces the usefulness of the MT-SDT procedure since the gNB will move the UE to connected mode at smaller data volumes than would be needed if some enhancements described in options 1 - 2 are made.

RAN2 handles BWP restriction in MT-SDT with option 2, introducing a Channel Quality Indication (CQI) in Message 3 or Msg A.

## CQI reporting in Msg3

In LTE, Bandwidth reduced low complexity (BL) UEs or UEs in enhanced coverage may report Downlink Channel Quality Report (DCQR) in Msg3. Without any CQI reporting, the gNB always has to adapt the transmission to reach cell edge, which especially for SDT users in the cell center is inefficient. For the majority of UEs which are not at the cell edge, larger TBs could be scheduled with sufficient reliablity, which is especially important considering the constraint of 48 PRBs remaining for MT-SDT operation within CORESET 0. Additionally, this would be beneficial also in case of MT-SDT to enable better gNB control, e.g. to decide whether to do MT-SDT or go to RRC CONNECTED mode.

Introduction of a DCQR for MT-SDT would enable proper link adaptation for MT-SDT DL data transmission.

In LTE the DCQR is implemented as a MAC CE. For MT-SDT, a similar MAC CE could be defined and multiplexed into Msg3, MsgA or into the initial CG-SDT transmission. Introducing DCQR containing channel condition information, allows adapating the link towards a higher MCS or lower CCE aggregation level as seen in Figure 1 below (Simulation assumptions provided in Annex A.1), thereby allowing larger TB allocation.

Figure 1: A snapshot of MCS and aggregation level distribution at larger data volumes. As stated in Observation 2, without quality reports, MT-SDT performs conservative link scheduling at low MCS values at higher aggregation levels.

Introduce a DCQR MAC CE which can be multiplexed into Msg3, MsgA or the initial CG-SDT transmission in the MT-SDT procedure.

Simulation in Annex A is done with different DL buffer size. Figure 2 shows that the number of PDSCH transmssion can be considerably reduced depending on MT-SDT user’s channel condition and downlink buffer size.

Figure 2: Number of PDSCH transmissions increase considerably in baseline MT-SDT at larger data volumes if its operation is restricted to CORESET#0 bandwidth. Introducing quality report allows better link adaptation, thereby facilitating better TB scheduling and reducing the number of PDSCH transmissions.

Reducing the PDSCH transmissions by enabling better link adaptation reduces the total time the network spends on transmission, thereby reducing the overall latency as can be seen in Figure 3.

- Figure 3: Higher number of PDSCH transmissions incur larger latency. This results in low energy efficiency and reduces the overall gain for the MT-SDT feature.

From a UE perspective, this results in lesser UE power consumption and from the network perspective, this leads to more efficient cell resource utilization, thereby improving network energy efficiency. Therefore, introduction of DCQR MAC CE can improve the overall gain achieved as part of MT-SDT.

Introducing DCQR MAC CE improves the achieved overall MT-SDT performance; resulting in lower latency, lower UE power consumption, more efficient cell resouce utilization, and better network energy efficiency.

# Conclusion

In the previous sections we made the following observations:

Observation 1	MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC Inactive state.

Observation 2	MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions.

Observation 3	Introduction of a DCQR for MT-SDT would enable proper link adaptation for MT-SDT DL data transmission.

Observation 4	Introducing DCQR MAC CE improves the achieved overall MT-SDT performance; resulting in lower latency, lower UE power consumption, more efficient cell resouce utilization, and better network energy efficiency.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 handles BWP restriction in MT-SDT with option 2, introducing a Channel Quality Indication (CQI) in Message 3 or Msg A.

Proposal 2	Introduce a DCQR MAC CE which can be multiplexed into Msg3, MsgA or the initial CG-SDT transmission in the MT-SDT procedure.

# References

3GPP RP-222993. “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR”

3GPP 38.331, 17.2.0, NR; Radio Resource Control (RRC); Protocol specification

- [3]	3GPP 38.212, 17.2.0, NR; Multiplexing and channel coding

- [4]	3GPP 38.214, NR; Physical layer procedures for data

# Annex A

## Annex A.1

Set of simulation assumptions that were used to evaluate MT-SDT performance and CQI report in Msg3 gains is summarized below.

| Parameter | Value |
| --- | --- |
| User configuration | 1 user in a cell (measurements averaged over 100 randomly distributed UEs) User not moving, but environment is changing |
| Cell configuration | FDD 40MHz with SCS 30kHz |
| DL data buffer | Varying as show in the simulation curve |
| UL data buffer | 0 |
| SDT RSRP threshold | -120dB |
| MCS assumption | MCS 0 is assumed before CQI report |