3GPP TSG RAN WG2 #123    											R2-2307940

Toulouse, France, August 21-25, 2023

Agenda Item:	7.9.3

Source: 	NEC

Title:	Discussion on Remaining Issues of Service Continuity

Document for:	Discussion

- Introduction

This contribution provides our further considerations on the leftover issues of service continuity enhancements.

- Discussion

  - Basic Procedures of for Inter-gNB I2D Path Switching and Inter-gNB I2I Path Switching

When considering the procedure of inter-gNB I2D path switching including the releasing of indirect path, one issue is when to reconfigure Relay UE to release the configuration of relay channel(s).

For intra-gNB I2D path switching, the gNB is aware of the access of Remote UE for the reason that the gNB is performed as both source gNB and the target gNB. So the reconfiguration of Relay UE can be executed at a proper time based on gNB implementation, such as the RRCReconfiguration message to the L2 U2N Relay UE can be sent anytime after step 3 [1].

However, for inter-gNB I2D path switching, the path switching command is sent via the source Relay UE. Due to the per-hop RLC scheme being adopted, the source gNB is not aware of whether the path switching command is sent to Remote UE successfully.  Furthermore, Remote UE will access to the target gNB while the source gNB is in charge of the reconfiguration of Relay UE. Therefore, the source gNB is not aware of whether the path switching of Remote UE is finished. Thus, it may happen that the source gNB reconfigures the source Relay UE to release the PC5 relay RLC channel/the PC5 connection between Remote UE before the successful delivery of path switching command to Remote UE. For example, the path switching command is waiting for RLC retransmission at the source Relay UE, but the PC5 relay RLC channel of Relay UE is released as requested by the source gNB. Moreover, a similar situation may happen to inter-gNB I2I path switching.

For inter-gNB I2D path switching and inter-gNB I2I path switching, when to reconfigure the source Relay UE should be considered to guarantee the successful delivery of the path switch command.

To solve this problem, one simple method is the target gNB be notified by the source gNB with the access of Remote UE. After that, the source Relay UE can be reconfigured. So we propose:

RAN2 to consider when to reconfigure Relay UE to release the configuration of relay channel(s) for inter-gNB I2D path switching and inter-gNB I2I path switching.

  - Signaling for handling I2I path switching

In Rel-17, the existing IE reconfigurationWithSync is used to indicate direct-to-indirect path switch to Remote UE. Additionally, one new IE sl-PathSwitchConfig is introduced to configure T420 and the target Relay UE to Remote UE [2]. For I2I path switch discussed in Rel-18, the target node is also a Relay UE. As a result, the same IE sl-PathSwitchConfig can be utilized to indicate indirect-to-indirect path switch. Moreover, if Remote UE originally has an indirect path, it can identify that this signaling is for I2I path switch. Then we propose:

RAN2 to reuse IE sl-PathSwitchConfig to indicate indirect-to-indirect path switch to Remote UE without any enhancement.

  - Failure handling for inter-gNB path switching

During D2I or I2I path switching, the RRC layer of Remote UE may trigger the upper layer to establish the PC5 unicast link with the target Relay UE. The RRCReconfigurationComplete message can only be sent if this link is successfully established. Since extra time is needed for the preparation of the target Relay UE between two gNBs compared to intra-gNB case, the channel condition between Relay UE and Remote UE may be changed during that long time period, leading to the failure of the PC5 unicast link establishment. Therefore, the RRCReconfigurationComplete message will not be sent, let alone sending it successfully to stop timer T420 [2]. This failure of path switching can be detected by Remote UE once the PC5 unicast link is not established successfully. Thus Remote UE can solve the failure in time by executing the connection re-establishment procedure, rather than waiting for the expiration of timer T420. So we propose:

RAN2 to consider the failure of PC5 unicast link establishment during path switching to trigger the RRC Reconfiguration failure procedure.

- Conclusion

We have the following proposals:

Observation 1:	For inter-gNB I2D path switching and inter-gNB I2I path switching, when to reconfigure the source Relay UE should be considered to guarantee the successful delivery of the path switch command.

Proposal 1:	RAN2 to consider when to reconfigure Relay UE to release the configuration of relay channel(s) for inter-gNB I2D path switching and inter-gNB I2I path switching.

Proposal 2:	RAN2 to reuse IE sl-PathSwitchConfig to indicate indirect-to-indirect path switch to Remote UE without any enhancement.

Proposal 3:	RAN2 to consider the failure of PC5 unicast link establishment during path switching to trigger the RRC Reconfiguration failure procedure.

Reference

- TS 38.300 V17.5.0

- TS 38.331 V17.5.0