3GPP TSG-RAN WG2 #123	R2-2308002

Toulouse, France, 21st – 25th August 2023

Agenda item:	   7.4.3

Source:		Lenovo

Title:		Discussion on issues related to subsequent CPAC

Document for:		Discussion and Decision

# Introduction

In this paper, we further discuss the relevant issues related Subsequent CPAC.

# 2	Discussion

## 2.1 Scenario

RAN2#119bis Agreements: Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

In the RAN2#119bis meeting, RAN2 has confirmed the need to support “CPA” selective activation of cell groups. On the other hand, the exact meaning of “CPA” selective activation needs further clarification. In our understanding there are two types of “CPA” selective activation.

- Type#1: After CPA execution, configurations of candidate PSCells for CPA are kept for the subsequent conditional PSCell change (without releasing the configuration after execution), and UE stays in DC scenario.

- Type#2: After CPA execution, configurations of candidate PSCells for CPA are kept for the subsequent conditional PSCell addition (without releasing the configuration after execution) when UE switches from DC to standalone scenario.

In the RAN2#121 meeting, Type#1 has been discussed and assumed by RAN2.

RAN2#121 Agreements: R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

Type#2 implies UE will switch frequently between DC scenario and stand-alone scenario, which doesn’t seem a practical scenario for Subsequent CPAC. In our understanding, the motivation for Subsequent CPAC is to reduce reconfiguration effort when UE stays in DC and moves among few PSCells and needs to switch PSCells frequently. In addition, switch between DC and standalone scenario also means the switch of bearer or radio resources between MN and SN, which means additional complex.

When UE disconnects from SN and enters stand-alone connection, configuration related to Subsequent CPAC is released.

It has been assumed that all three scenarios can be supported, namely SN initiated intra-SN Subsequent CPAC, MN initiated inter-SN Subsequent CPAC, and SN initiated inter-SN Subsequent CPAC.

RAN2#121 Agreements: Assume to support the following scenarios of SCG selective activation: - SN initiated intra-SN SCG selective activation - MN initiated inter-SN SCG selective activation - SN initiated inter-SN SCG selective activation

In case of SN initiated intra-SN Subsequent CPAC, in our view, some principles for legacy Rel16 intra SN CPC can be taken as the start point. For example, Rel16 intra SN CPC is a procedure without MN involvement, meaning the configuration of intra SN CPC is transparent to MN and can be configured by SRB3 directly.

Similar as Rel16 intra-SN CPC, Rel18 SN initiated intra-SN Subsequent CPAC is a procedure without MN involvement, which can be configured by SRB3.

In Release 17, coexistence of different conditional reconfigurations is supported, that UE can be configured with CHO, intra SN CPC and inter SN CPC at the same time. However, we don’t see the motivation to support Subsequent CPAC and legacy CPC at the same time, since they are more or less serving the same purpose. Thus, it is suggested that RAN2 does not support the coexistence of Subsequent CPAC and legacy CPC.

RAN2 does not support the coexistence of Subsequent CPAC and legacy CPC.

## 2.2 Reference Configuration

- RAN2#122 Agreement: The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

In terms of which network node provides the reference configuration, in our understanding, it could be decided by the initiating MN/SN node. In case of MN initiated inter-SN Subsequent CPAC, it should be MN that generates the reference configuration. And in case of SN initiated intra/inter-SN Subsequent CPAC, it should be the source SN that generates the reference configuration.

The reference configuration for Subsequent CPAC is generated by the initiating MN/SN node.

For inter SN Subsequent CPAC, when the MN provides the reference configuration to the target SN for delta configuration, it can reuse the existing fields in the CG-ConfigInfo inter node message, namely scg-RB-Config and sourceConfigSCG. The target SN does not need to understand if the received reference configuration is the initial source SCG configuration or some separate configuration generated by the initiating MN/source SN.

The existing fields in CG-ConfigInfo message, such as scg-RB-Config and sourceConfigSCG, are used to transfer the reference configuration from MN to target SN for delta configuration for Subsequent CPAC.

However, in case of SN initiated inter SN selective activation, in legacy CG-Config message, there seems to be no appropriate fields to convey the reference configuration. Thus, RAN2 is suggested to discuss if a new IE/field shall be introduced in CG-Config message to carry the reference configuration for SN initiated inter-SN selective activation.

RAN2 is suggested to discuss if a new IE/field shall be introduced in CG-Config message to carry the reference configuration for SN initiated inter-SN selective activation.

## 2.3 About MCG configuration

RAN2#121bis Agreements: The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations: - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included.

There are two FFSs left regarding the MCG configuration during Subsequent CPAC considering the MCG configuration could change as well during the Subsequent CPAC, namely:

- if reference configuration could also include a reference configuration for MCG.

- If each target SCG configuration could be associated with a corresponding MCG configuration.

First, it could be beneficial for RAN2 to confirm if MCG configuration could change during the Subsequent CPAC. Note that in Rel17 inter SN CPC, the MCG configuration could be different for different candidate SCGs. Thus, it seems straightforward to follow the same principle as in Rel17 inter SN CPC.

In Rel17 inter SN CPC, the MCG configuration could be different for different candidate SCGs.

RAN2 confirms that in Subsequent CPAC, MCG configurations (with the same PCell) could be different for different candidate SCGs.

During Subsequent CPAC, each target SCG configuration could be associated with a corresponding MCG configuration.

To better support the MCG configuration change during Subsequent CPAC, it seems beneficial to support reference configuration to support delta MCG configuration as well.

RAN2 supports MCG reference configuration for delta MCG configuration during Subsequent CPAC

## 2.5 Execution condition configuration

RAN2#121bis Agreements: For MN initiated inter-SN Subsequent CPAC, source MN generates the execution conditions for the initial CPAC. - FFS on the following options for subsequent CPC: - Option 1: Source MN generates the execution conditions for all subsequent CPC. - Option 2: Candidate SN may generate execution conditions for subsequent CPC. For SN initiated inter-SN Subsequent CPAC, source SN generates the execution conditions for the initial CPC. FFS if Candidate SN may generate/modify execution conditions for subsequent CPC - 3. The execution conditions associated with each candidate target PSCell. - a. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. - b. For SN initiated procedure, execution conditions based on events A3/A5 are supported. RAN2#122 Agreements: For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC. FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

It has been agreed in the last RAN2 meeting that at least for SN-initiated subsequent CPAC, candidate SN will generate execution conditions, e.g., when UE will leave a SCG belonging to the candidate SN. It is still open whether the same design should be adopted for the MN initiated Subsequent CPAC. In our understanding, we don’t see any critical reason to not adopt the same design. Instead, we see it’s beneficial to have a unified solution and reduce the specification effort.

For both MN initiated Subsequent CPAC, RAN2 also supports the candidate SN to generate execution condition for Subsequent CPAC, i.e., when UE leaves a SCG belonging to the candidate SN.

One issue with candidate SN generating execution condition would be that one candidate SN needs to know the existence of other candidate SCGs belonging to other candidate SNs, which is not supported in the legacy CPAC.

One possible solution is to have two rounds of configuration before sending the Subsequent CPAC configuration to UE.

- Round 1: Candidate SCG configuration

  - MN/SN requests a candidate SN to prepare candidate SCGs for Subsequent CPAC

  - Then, the candidate SN generates and provides the RRC reconfiguration for candidate SCG(s)

- Round 2: Execution condition configuration

  - After round 1, MN informs each candidate SN about the existence of other candidate SCGs prepared by other candidate SNs.

  - The candidate SN will then generate and provide the execution conditions for Subsequent CPAC when UE leaves a SCG belonging to the candidate SN.

A high level procedure is given in the figure below as one example.

To support candidate SN generating execution condition for Subsequent CPAC, RAN2 supports two rounds of configurations among network nodes before sending the Subsequent CPAC configuration to UE, 1. Candidate SCG configuration 2. Execution condition configuration.

It has been agreed to support A3 and A5 events for in SN initiated Inter-SN Subsequent CPAC. In our understanding the combination of A3 and A5 events should also be supported same as in legacy CHO/CPAC configuration. RAN2 is suggested to clarify.

In SN initiated Inter-SN Subsequent CPAC, RAN2 confirms the combination of A3 and A5 is supported as execution condition.

## 2.6	Security

SA3 has discussed and agreed the following mechanism to support the key update during the Subsequent CPAC.

S3-233200 SA3 wants to update on the state of the security analysis on selective SCG. SA3 has considered different alternative solutions to address the issue and concluded to have a SN Counter based solution for fresh key derivation for selective SCG activation. Following is the solution details from security perspective: 1. The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC. 2. The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change. SA3 is offering a joint conference call that can be used to discuss and to enhance the above described solution details.

From RAN2 perspective, it would be good to confirm the understanding and support the mechanism from RRC signalling perspective.

RAN2 supports configuring a list of sk-counters for one candidate SCG and UE shall use an unused sk-counter every time UE connects to this candidate SCG.

## 2.7	Others

Another aspect is after Subsequent CPAC execution and UE accesses to a target SCG successfully, how does UE send the SN RRC Reconfiguration Complete message to the network. Note that in legacy Rel16 intra-SN CPC and Rel17 CPAC, different approaches are used:

- For Rel16 intra-SN CPC, after CPC execution, the SN RRC Reconfiguration Complete message is conveyed in ULInformationTransferMRDC message to MN, if SRB3 is not configured. MN forwards to the SN.

- For Rel17 CPAC, after CPAC execution, the SN RRC Reconfiguration Complete message is conveyed in MN RRC Reconfiguration Complete message to MN. MN forwards to the corresponding target SN.

For Rel18 Subsequent CPAC, considering both intra-SN Subsequent CPAC and inter-SN Subsequent CPAC are going to be supported, the SN RRC Reconfiguration Complete message transmission after PSCell switch can follow similar approaches in the legacy. That is

- For intra-SN Subsequent CPAC, after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in ULInformationTransferMRDC message to MN, if SRB3 is not configured.

- For inter-SN Subsequent CPAC (no matter if it is MN initiated or SN initiated), after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in MN RRC Reconfiguration Complete message to MN.

For intra-SN Subsequent CPAC, after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in ULInformationTransferMRDC message to MN, if SRB3 is not configured.

For inter-SN Subsequent CPAC (no matter if it is MN initiated or SN initiated), after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in MN RRC Reconfiguration Complete message to MN.

In the LTM discussion, RAN2 agreed to support early TA acquisition. More specifically, UE could get the TA value for a certain candidate cell in advance and can thus skip the RACH procedure when switching to that candidate cell. As such the cell switching delay can be reduced.

Even though the exact solution for early TA acquisition is still under discussion, RAN2 can discuss, if time allowed, the early DL/UL sync approach introduced for LTM can be extend to the Subsequent CPAC scenario.

RAN2 can discuss early DL/UL sync for LTM can be extended to Subsequent CPAC if time allows.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	In Rel17 inter SN CPC, the MCG configuration could be different for different candidate SCGs.

Based on the discussion above, we propose:

Scenario:

Proposal 1	When UE disconnects from SN and enters stand-alone connection, configuration related to Subsequent CPAC is released.

Proposal 2	Similar as Rel16 intra-SN CPC, Rel18 SN initiated intra-SN Subsequent CPAC is a procedure without MN involvement, which can be configured by SRB3.

Proposal 3	RAN2 does not support the coexistence of Subsequent CPAC and legacy CPC.

Reference Configuration:

Proposal 4	The reference configuration for Subsequent CPAC is generated by the initiating MN/SN node.

Proposal 5	The existing fields in CG-ConfigInfo message, such as scg-RB-Config and sourceConfigSCG, are used to transfer the reference configuration from MN to target SN for delta configuration for Subsequent CPAC.

Proposal 6	RAN2 is suggested to discuss if a new IE/field shall be introduced in CG-Config message to carry the reference configuration for SN initiated inter-SN selective activation.

MCG configuration:

Proposal 7	RAN2 confirms that in Subsequent CPAC, MCG configurations (with the same PCell) could be different for different candidate SCGs.

Proposal 8	During Subsequent CPAC, each target SCG configuration could be associated with a corresponding MCG configuration.

Proposal 9	RAN2 supports MCG reference configuration for delta MCG configuration during Subsequent CPAC

Execution Condition Configuration:

Proposal 10	For both MN initiated Subsequent CPAC, RAN2 also supports the candidate SN to generate execution condition for Subsequent CPAC, i.e., when UE leaves a SCG belonging to the candidate SN.

Proposal 11	To support candidate SN generating execution condition for Subsequent CPAC, RAN2 supports two rounds of configurations among network nodes before sending the Subsequent CPAC configuration to UE, 1. Candidate SCG configuration 2. Execution condition configuration.

Proposal 12	In SN initiated Inter-SN Subsequent CPAC, RAN2 confirms the combination of A3 and A5 is supported as execution condition.

Security:

Proposal 13	RAN2 supports configuring a list of sk-counters for one candidate SCG and UE shall use an unused sk-counter every time UE connects to this candidate SCG.

Others:

Proposal 14	For intra-SN Subsequent CPAC, after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in ULInformationTransferMRDC message to MN, if SRB3 is not configured.

Proposal 15	For inter-SN Subsequent CPAC (no matter if it is MN initiated or SN initiated), after PSCell switch execution, the SN RRC Reconfiguration Complete message is conveyed in MN RRC Reconfiguration Complete message to MN.

Proposal 16	RAN2 can discuss early DL/UL sync for LTM can be extended to Subsequent CPAC if time allows.

# 4	Reference