3GPP TSG-RAN WG2 Meeting #123	R2-2308013

Toulouse, France, 21st – 25th August 2023

Agenda item:	 7.11.2.1

Source:	 Lenovo

Title:		Control plane aspects of multicast reception in RRC_INAVTICE

Document for:		Discussion and Decision

- Introduction

This contribution further discusses the remaining CP related issues of multicast reception in RRC_INACTIVE state.

- Discussion

  - Session Deactivation

- It is still FFS that whether PTM configuration of a deactivated multicast session should be provided in RRCRelease message.

- Currently, UE AS layer is not aware that whether a multicast session is deactivated or not. When the multicast session is deactivated, as specified in TS 23.247, there is no explicit "deactivation" indication to the UE, how the UE is changed to IDLE state is defined in TS 38.300.

7.2.5.3 MBS session deactivation procedure NOTE 2: There is no explicit "deactivation" indication to the UE, how the UE is changed to IDLE state is defined in TS 38.300 [9].

- In Rel-17, it is up to gNB implementation to release the multicast session and related MRBs upon receiving session deactivation from core network.

Observation 1: For a deactivated multicast session, the MRB may or may not be released according to NW implementation in Rel-17.

Observation 2: UE is not aware of the MC session status (active or inactive) in Rel-17.

- Following the principle, if the MRB have been released, the NW should not provide PTM configuration in RRCRelease message.  If the MRB have not been released, the NW should provide PTM configuration of the multicast session in RRCRelease message. New deactivated state indication can also be provided in the PTM configuration to notify UE not to monitor the corresponding G-RNTI.

For a deactivated session, the PTM configuration is optionally provided in RRCRelease message. A new deactivated state indication is provided in the PTM configuration to notify UE not to monitor the corresponding G-RNTI.

- For notification of session deactivation, it was agreed that:

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

This is assumed to have no/minor impact on RAN1/PHY

- As discussed in the email discussion [1]:

-	Solution 2.1: If PTM config of a MC session is absent, it means this MC session is deactivated.

-	Solution 2.2: New deactivated state indication is provided in PTM configuration per MC session.

As stated in observation 1, for a deactivated multicast session, the MRB may or may not be released according to NW implementation in Rel-17. If the MRB is released, a MRB release is provided in the PTM configuration in the MCCH. If MRB is not released, the PTM configuration is still provided in the MCCH by MRB Setup/modify We think either MRB release or MRB setup/modify of a deactivated MC session should be present in MCCH. Thus, solution 2.1 is not applicable.

For the former case, MRB release is provided in the MCCH and then the UE release the MRB and stop monitoring the corresponding G-RNTI. For the latter case, i.e., the gNB still prefer to keep the MRB of a deactivated MC session, a new deactivated state indication can also be provided in the PTM configuration to notify UE stop monitoring the corresponding G-RNTI.

For notification of deactivation of a MC session, either MRB release or MRB setup/modify can be used in MCCH. If MRB setup/modify is used, a new deactivated state indication is provided in the PTM configuration to notify UE stop monitoring the corresponding G-RNTI.

  - MRBs handling during RRC state transition

- In RAN2#122, it was agreed that:

As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH.

FFS how existing MRBs are handled.

- We think that PTM configuration in multicast MCCH is quite similar as the IE structure of broadcast MRB configuration. It would mean that the IE structure of a MRB in multicast MCCH is somehow different with MRB configuration in RRC dedicated signaling. It means that a new MRB is established for the multicast reception in RRC_INACTIVE. However, even though the IE structure are different between MCCH and RRC dedicated signalling, the PDCP configuration should be same at least in the serving cell.

- Currently, the old MRB used in RRC_CONNECTED is suspended when UE enters RRC_INACTIVE. In case of MRB suspend, the UE shall suspend PDCP entity of the multicast MRB, that includes that the UE shall stop and reset the t-reordering if running, and the UE shall keep the RX_NEXT and RX_DELIV. In order to support minimization of data loss during the RRC state transition from RRC_CONNECTED to RRC_INACTIVE, the UE can use the same PDCP entity for the MRB used for RRC_INACTIVE and RRC_CONNECTED states, which means the UE shall not perform PDCP suspend for the multicast MRB. If minimization of data loss is not required, the UE shall just establish a new MRB with new PDCP entity and suspends the old MRB that used for RRC_CONNECTED state.

The new MRB for multicast reception in RRC_INACTIVE uses the same PDCP entity with the old MRB that was used in RRC_CONNECTED to minimize the data loss, i.e., the UE shall not suspend the PDCP entity of the MRBs during RRC state transition.

  - Configured radio threshold for triggering RRC connection resumption

In RAN2#121bis-e meeting,

UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold, FFS how to specify the threshold/reception quality.

From our view, a radio condition threshold is sufficient for tiggering RRC connection resume. Network should have the flexibility to configure to trigger the RRC Connection resume. The radio condition threshold can be a RSRP or RSRQ value. Since different multicast session may have different QoS requirement, the threshold should be different for different multicast session which can be provided by MCCH.

A radio condition threshold (i.e. RSRP and/or RSRQ) per multicast session is provided in MCCH for UE returning RRC_CONNECTED for the multicast session reception.

Besides that, in order to avoid ping-pong, a time value that the threshold condition needs to be fulfilled for UE retuning RRC_CONNECTED for multicast reception should be also provided. Furthermore, a prohibited time for UE triggering to transit RRC_CONNECTED for multicast reception can also be provides to avoid the UE returning back RRC_CONNECTED unnecessary. For example, the UE Upon receiving the RRCRelease message, the UE enters RRC_INACTIVE state, the UE starts the prohibited timer. When prohibited timer is running, the UE shall not trigger state transition from RRC_INACTIVE to RRC_CONNECTED procedure, i.e., the UE only performs state transition from RRC_INACTIVE to RRC_CONNECTED when the timer expires.

- A time value that the threshold condition needs to be fulfilled for UE returning RRC_CONNECTED for multicast reception is configured to UE.

- A prohibited time for UE returning RRC_CONNECTED for multicast reception is configured to UE.

  - PTM configuration provision by RRCRelease without entering RRC_CONNECTED state

- In RAN2#121bis, it was agreed that

When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

- When UE moves to a cell of which the UEs has no PTM configuration of a MRB, the UE should perform RRC Resume procedure to acquiring the PTM configuration. Since we already agreed that the PTM configuration can be provided by RRCRelease message, it is not necessary for the UE entering RRC_CONNECTED state to get the PTM configuration for sake of signalling overhead. For example, the UE indicates PTM configuration request indication to the network in the RRCResumeRequest message. The PTM configuration request indication is used to indicate the RRCResumeRequest is only for acquiring the PTM configuration which implies there is no need to resume the UE into RRC_CONNECTED state. In response to the PTM configuration request indication in the RRCResumeRequest message, the network can directly provide the updated PTM configuration in the RRCRelease message without entering RRC_CONECTED state.

PTM configuration provision by RRCRelease without entering RRC_CONNECTED state should be supported (similar with RANU).

  - New Access Category and RRC Cause

- When a UE performs cell reselection to a neighbor cell that PTM configuration of the MRB is not available e.g. the PTM configuration of the MRB is not provided in the MCCH of the neighbor cell but the multicast session is supported by the cell, the UE needs to perform state transition to RRC_CONNECTED state and then NW can provide the PTM configuration by RRC dedicated signalling.

- Since the state transition to RRC_CONNECTED is triggered by the radio link condition, which is a AS triggered request, it is up to RRC determines the Access Category while NAS determines the Access Identity(ies). A new Access Category (e.g. access category 9) is defined for the RRC Resume procedure triggered for multicast reception and the RRC cause in the RRCResuemeRequest message is set to ‘multicast call’ to indicate the request is due to multicast reception.

A new access category and RRC cause are introduced in case that the RRC Resume procedure is triggered by multicast reception.

  - MBS Neighbor Cell List

In RAN2#121bis meeting, it was agreed that

The neighbour cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

In Rel-17, the MBS-NeighbourCellList IE is introduced for indicating a list of neighbour cells where ongoing MBS sessions provided via broadcast MRB in the current cell are also provided. This allows the UE, e.g. to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. Different with broadcast, the multicast transmission status in a neighbour cell may include:

-	multicast session is not provided by the neighbor cell either by PTM transmission or PTP transmission, e.g., the neighbour cell is out of multicast service area;

-	the multicast session is supported by the neighbour cell but PTM transmission has not been started e.g. it is the first UE moves to the cell for the multicast reception or only PTP transmission is used for a small group of UEs.

For the first case, same with broadcast, the UE may need to request unicast reception of the multicast service before moving to the neighbour cell. For the second case, since the PTM configuration is not provided by MCCH, the UE may need prepare to resume to RRC_CONNECTED state when cell reselection to the neighbor cell.

Neighbour cell list is enhanced to include:

-	Whether the multicast session is supported by the neighbour cell or not e.g., the neighbour cell is out of multicast service area;

-	In case that the multicast session is supported, whether PTM transmission is provided in the neighbour cell.

  - Service continuity during mobility

In Rel-17, if minimisation of data loss is applied for a given MRB, synchronisation of allocation of PDCP COUNT values is applied by derivation of the PDCP COUNT values by means of a DL MBS QFI Sequence Number provided on NG-U. Synchronisation in terms of MBS QoS flow to MRB mapping and PDCP SN size of the corresponding MRB among gNBs are achieved by means of network implementation.

- In order to support minimisation of data loss between cells for PTM reception in RRC_INACTIVE, the continuity of PDCP variables (e.g., RX_DELIV) of an MRB is needed among different cells if PDCP COUNT is synchronized. One of solution is to introduce an area of PDCP continuity (e.g., indication of a list of cells with synchronized PDCP COUNT in NCL):

-	When UE moves within the area, the PDCP continuity is performed, and the PDCP variables are not reset to initial value when UE moves one cell to another cell within the area.

-	when UE reselects to a cell that is not in the area of PDCP continuity, the UE releases the PDCP entity of the MRB and acquires MCCH from the new cell. And then perform PDCP establishment according to the MRB configuration in the MCCH. OR

-	when UE reselects to a cell that is not in the area of PDCP continuity, the UE releases the PDCP entity of the MRB and trigger RRCResume procedure for new PDCP configuration.

Support continuity of PDCP variables (e.g. RX_DELIV) of an MRB by MCCH when UE moving among cells to minimize the data loss e.g., by indication of a list of cells with synchronized PDCP COUNT in NCL.

- Another issue is the MRB ID may be different among different cells, so that the area of PDCP continuity may also include an associated MRB ID in each cell for the MRB. When UE reselects to a cell within the area, the UE identifies the MRB with the associated MRB ID and associate the MRB to the PDCP entity.

- One example is to introduce an association ID to associate the PDCP entity to different RLC bearer of each cell within the are of PDCP continuity. The association ID is provided in the area info of PDCP continuity of the MRB and also provided in the RLC bearer configuration in MCCH of each cell so that the UE can associate the PDCP entity to different RLC bearer of each cell.

RAN2 discusses how to solve the issue on which different MRB IDs may be allocated by different cells for a same MRB to support service continuity during mobility.

3	Conclusion

This contribution discusses CP aspects for multicast reception in RRC_INACTIVE.

Observation 1: For a deactivated multicast session, the MRB may or may not be released according to NW implementation in Rel-17.

Observation 2: UE is not aware of the MC session status (active or inactive) in Rel-17.

- For a deactivated session, the PTM configuration is optionally provided in RRCRelease message. A new deactivated state indication is provided in the PTM configuration to notify UE not to monitor the corresponding G-RNTI.

- For notification of deactivation of a MC session, either MRB release or MRB setup/modify can be used in MCCH. If MRB setup/modify is used, a new deactivated state indication is provided in the PTM configuration to notify UE stop monitoring the corresponding G-RNTI.

- The new MRB for multicast reception in RRC_INACTIVE uses the same PDCP entity with the old MRB that was used in RRC_CONNECTED to minimize the data loss, i.e., the UE shall not suspend the PDCP entity of the MRBs during RRC state transition.

- A radio condition threshold (i.e. RSRP and/or RSRQ) per multicast session is provided in MCCH for UE returning RRC_CONNECTED for the multicast session reception.

- A time value that the threshold condition needs to be fulfilled for UE returning RRC_CONNECTED for multicast reception is configured to UE.

- A prohibited time for UE returning RRC_CONNECTED for multicast reception is configured to UE.

- PTM configuration provision by RRCRelease without entering RRC_CONNECTED state should be supported (similar with RANU).

- A new access category and RRC cause are introduced in case that the RRC Resume procedure is triggered by multicast reception.

- Neighbour cell list is enhanced to include:

-	Whether the multicast session is supported by the neighbour cell or not e.g., the neighbour cell is out of multicast service area;

-	In case that the multicast session is supported, whether PTM transmission is provided in the neighbour cell.

- Support continuity of PDCP variables (e.g. RX_DELIV) of an MRB by MCCH when UE moving among cells to minimize the data loss e.g., by indication of a list of cells with synchronized PDCP COUNT in NCL.

- RAN2 discusses how to solve the issue on which different MRB IDs may be allocated by different cells for a same MRB to support service continuity during mobility.

Reference

[1] R2-2306870 Report of [AT122][603][MBS] Session deactivation for MC in INACTIVE (Apple) Apple