3GPP TSG-RAN WG2 #123	R2-2308022

Toulouse, France, 21st – 25th August 2023

Agenda item:	   7.16.2.3

Source:		Lenovo

Title:		Discussion on gNB/LMF awareness of UE side/part model/functionality

Document for:		Discussion and Decision

# Introduction

In the past RAN2 meeting, the following agreements have been made regarding the model transfer/delivery. The seven AI model transfer/delivery methods on the table have their own pros and cons as discussed in the email discussion [1].

We Use the wording “model transfer/delivery” model delivery that serves the use cases in the SI is within RAN2 scope, regardless other aspects. Agreed: - Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following: - Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP). Table: relations between solutions and use cases Solutions; Applicable use cases. Solution 1a, 1b; CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 1a and 1b.. Solution 2a, 2b; CSI feedback enhancement Beam management Note: No specific considerations for Positioning accuracy enhancement for Solution 2a and 2b.. Solution 3a, 3b; Positioning accuracy enhancement. Solution 4; CSI feedback enhancement Beam management Positioning accuracy enhancement. - Note: the solutions use case relation is preliminary (work in progress), and the purpose is to have better understanding on what to further analyse

# Discussion

In this paper, we try to further discuss one particular issue that the entity responsible for AI model/functionality monitoring/selection/etc. might be transparent to the AI model transfer/delivery.

For example, in case of AI based CSI/BM, when a UE-side model or two-sided model is used, gNB could be responsible for AI model/functionality monitoring and the corresponding AI model selection/(de)activation/switch/fallback decision making. And for AI based positioning, LMF could make the AI model selection/etc. decision for UE side model. is also discussed in the email discussion [Post122][060][AIML] Mapping of functions.

Table 1: The mapping of functions to physical entities for CSI compression with two-sided model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | gNB, OAM, OTT server, UE, [FFS: CN] |
| b) | Model transfer/delivery | For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->gNB&UE, or UE->gNB, [FFS: CN->gNB&UE] For training Type 3: - For UE part of two-sided model: OTT server->UE, [FFS: CN->UE]; - For NW part of two-sided model: OAM->gNB, [FFS: CN->gNB]; |
| c) | Inference | NW part of two-sided model: gNB UE part of two-sided model: UE |
| d) | Model/functionality monitoring | NW-side: NW monitors the performance UE-side: UE monitors the performance and reports to NW |
| e) | Model/functionality control (selection, (de)activation, switching, updating, fallback) | gNB, [FFS: UE] |

Table 2: The mapping of AI/ML functions to physical entities for beam management with UE-side model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | UE-side OTT server, UE, [FFS: gNB, OAM, CN] |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: gNB->UE, or OAM->UE, or CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE (UE monitors the performance, and may report to gNB), gNB (gNB monitors the performance) |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB if monitoring resides at UE or gNB, UE if monitoring resides at UE |

Table 3: The mapping of functions to physical entities for positioning with UE-side model (case 1 and 2a)

| Use case | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | LMF, UE-side OTT server, UE, [FFS: OAM, CN] |
| b) | Model transfer/delivery | LMF->UE, or UE-side OTT server->UE, [FFS: OAM->UE, CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE, LMF |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | UE if monitoring resides at UE, LMF if monitoring resides at UE or LMF |

For AI based CSI/BM, when a UE-side model or two-sided model is used, gNB could be responsible for AI model/functionality monitoring and the corresponding AI model selection/(de)activation/switch/fallback decision making.

For AI based positioning, when a UE-side model is used, LMF could be responsible for AI model/functionality monitoring and make the AI model selection/(de)activation/switch/fallback decision.

For the above mentioned two cases, when solution 2a, 2b or 4 is used, gNB or LMF is transparent to the AI model transfer/delivery. In order for the gNB/LMF to perform AI monitoring and corresponding model/functionality selection/(de)activation/switch/fallback decision, additional procedure is needed for the gNB/LMF to be aware of the use of AI model/functionality at UE side.

Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data.

Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

One simple solution could be that after UE receives the UE side model or UE part of two-sided model, UE informs gNB/LMF with necessary meta information. As such, gNB and LMF can still be aware of the usage of UE side/part model/functionality and performs AI monitoring/selection/etc. An exemplary procedure is shown in the figure below.

We also notice that a concept of AI model/functionality identification procedure from RAN1 could be used to serve such purpose, e.g., in model identification type B1, UE may initiate a model identification procedure which will align the understanding of AI models between UE and NW.

RAN1#113 Agreements Agreement For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any). - Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification. - FFS: Spec impact to other WGs - Type B: Model is identified via over-the-air signaling, - Type B1: - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - FFS: details of steps - Type B2: - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - FFS: details of steps - Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary.

For AI based CSI/BM, to support model transfer Solution 2a/2b/4, RAN2 studies approaches for the gNB to be aware of the UE side/part model/functionality, e.g., by UE notification after the model transfer via UE initiated AI model/functionality identification procedure.

For AI based positioning, to support model transfer Solution 4, RAN2 studies approaches for the LMF to be aware of the UE side model/functionality, e.g., by UE notification after the model transfer via UE initiated AI model/functionality identification procedure.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	For AI based CSI/BM, when a UE-side model or two-sided model is used, gNB could be responsible for AI model/functionality monitoring and the corresponding AI model selection/(de)activation/switch/fallback decision making.

Observation 2	For AI based positioning, when a UE-side model is used, LMF could be responsible for AI model/functionality monitoring and make the AI model selection/(de)activation/switch/fallback decision.

Based on the discussion above, we propose:

Proposal 1	For AI based CSI/BM, to support model transfer Solution 2a/2b/4, RAN2 studies approaches for the gNB to be aware of the UE side/part model/functionality, e.g., by UE notification after the model transfer via UE initiated AI model/functionality identification procedure.

Proposal 2	For AI based positioning, to support model transfer Solution 4, RAN2 studies approaches for the LMF to be aware of the UE side model/functionality, e.g., by UE notification after the model transfer via UE initiated AI model/functionality identification procedure.

# 4	Reference

[1] R2-2302268	Report of Offline 027 model transfer delivery (Huawei)	Huawei

[2] R2-2305680	Discussion on AI framework and functionality mapping	Lenovo