3GPP TSG-RAN WG2 Meeting #123 meeting	R2-2308023

Toulouse, France, August 21-25, 2023

Agenda item:		7.5.2

Source:		Lenovo

Title:		Discussion on PDU sets and data burst awareness in RAN

Document for:		Discussion and Decision

- Introduction

- RAN#99 has updated the XR WID in RP-230786 including XR awareness:

Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

- In this contribution we further discuss the XR-awareness parameters to use in RAN.

- Discussion

2.1	UL assistant information

RAN2#122 meeting has discussed UL assistant information and reached the following agreements:

- UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter. - Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side. - UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information. - RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity). - Do not use PIN delay budget request for jitter reporting for XR services.

The jitter range information may be expressed by example of +/- N ms. For example, if the data arrives 2ms earlier compared to an expected periodical arrival time, the jitter value is -2ms, while if the data arrives 3ms later relative to an expected timing, the jitter value is 3 ms. UE transmits the UL jitter range information e.g., maximum value (e.g., 3 ms) associated with each UL periodicity for a QoS flow, to NG-RAN, the NG-RAN consider the jitter range is (+3ms, -3ms) relative to the periodical arrival time.

Proposal 1: UE transmits the UL maximum jitter range value relative to the periodical arrival time to NG-RAN.

In IIOT topic, the clock synchronization is a pre-condition for the IIOT deployment case. It is reasonable that the burst arrival time in the TSCAI is coded based on the timing information of the 5G internal system clock. However, the clock synchronization is not a pre-condition for the XR deployment case. Therefore, the burst arrival time reported to the gNB is coded similar as the timingOffset IE, which indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds.

SL-TrafficPatternInfo-r16::= SEQUENCE { trafficPeriodicity-r16 ENUMERATED {ms20, ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000}, timingOffset-r16 INTEGER (0..10239), sl-QoS-FlowIdentity-r16 SL-QoS-FlowIdentity-r16 } timingOffset This field indicates the estimated timing for a packet arrival in a sidelink logical channel. Specifically, the value indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds..

Proposal 2: UE transmits the UL burst arrival time similar as timeoffset-16, which indicates the timing offset with the respect to subframe#0 of SFN#0 in milliseconds to NG-RAN.

In case of NR-DC, the MCG and SCG may not be time synchronized. For example, PCell and PSCell are time desynchronized. The UE may calculate the BAT according to the timing of PCell in MCG and report it to the network. Since the PSCell in SN is not time synchronized with PCell in MN, the reported BAT cannot be used by SN directly.

In case of handover, the UE calculates the BAT according to the timing in the source cell and report it to the source gNB before handover. The source gNB forwards the reported BAT to the target gNB. However, the reported BAT cannot be used by the target gNB directly if the target cell is not timing synchronized with the source cell.

According to above issue analysis, it is proposed RAN2 to discuss how to handle these cases above.

Proposal 3: RAN2 to discuss how to report the UL burst arrival time to MCG/SCG in case of NR-DC and to target cell in case of handover.

The UL periodicity of a QoS flow has been included in the TSCAI, it is not so necessary to further report this to the gNB. However, if UE is aware of the periodicity and report it together with jitter range information as a supplement is also acceptable.

Proposal 4: UE transmits the UL periodicity together with the jitter range to NG-RAN.

2.2	UL End of Data Burst

RAN2#121b and RAN2#122 meeting has discussed UL end of data burst indication and reached the following:

- FFS on whether EoDB signalling is needed. - Reuse existing mechanisms (e.g. (Padding) BSR with BS value equal to zero) as implicit End of Data Burst (EoDB) indicator for the RAN. - On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

Like the purpose of the downlink EoDB, if network can be aware of the end of a UL data burst, network can determine to terminate UE’s DRX active time early for the purpose of power saving.

In our understanding, the padding BSR e.g., indicating data volume=0 can be used as an implicit indication of EoDB. Some new trigger condition for the transmission of a “zero-BSR” or the new BSR cancel condition may be required in some cases listed:

- Case 1: if the UL grant is not sufficient to additionally accommodate any padding BSR MAC CE plus subheader, no padding BSR is triggered.

- Case 2: if the UL grant is not sufficient to accommodate the triggered BSR, all triggered BSR may be cancelled if all triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader.

- Case 3: if the UL grant is only sufficient to additionally accommodate padding short truncated BSR for an LCH with highest priory.

In any case we see some benefit that gNB is made aware of the end of a data burst to optimize UE power saving. However, in above cases, if no “zero-BSR” is received by network, the network does not easily determine that end of transmission of a data burst.

Proposal 5: RAN2 to discuss new trigger conditions or new cancel conditions of BSR to indicate “zero-BSR” to the gNB as an indication of end of data burst for UL.

2.3	DL Data Burst

According to definition in SA2, a data burst is a set of multiple PDUs generated and sent by the application in a short period of time and a Data Burst can be composed by one or multiple PDU Sets. During a Data Burst, periods of data transmission inactivity should not be assumed. And the duration of Data Bursts may vary. For DL, RAN is aware of the last PDU indication of a data burst according to the End of Data Burst indication in the last PDU of a data burst from UPF, it is helpful to timely determine all PDUs of a data burst were received from UPF. After successfully transmitting the data burst to UE, NW can timely indicate UE enter C-DRX inactive mode to save power.

Proposal 6: Confirm that NW can send UE to DRX inactive mode according to the End of Data Burst indication.

2.4	PDU set information over Xn and F1 interfaces

In the following we discuss XR-awareness information provided between RAN nodes in case of DC, CU-DU split and handover case.

    - Handover and dual connectivity

During handover procedure, the PDU set QoS parameter and characteristics may need to be transmitted from source gNB to target gNB. There is a case that only a part of the PDUs of a PDU set has been successfully transmitted in the source gNB, while the other PDUs of the PDU set will be forwarded and transmitted by the target gNB. In this case, it is beneficial to be aware of the remaining 5G PSDB for transmitting the remaining PDUs of the PDU set for more accurate QoS management and scheduling for target gNB. The PDU set information may be also needed for the forwarding data.

For dual connectivity, the similar information as handover may also needed between MN and SN.

    - CU-DU split

When receiving QoS flow from UPF, the CU performs QoS flow to DRB mapping. After the mapping, the CU may need to provide the PDU set QoS parameter and characteristics of the DRB to the DU via control plane signaling and PDU set information of the DRB to DU via GTP-U extension header since the DU is responsible for the scheduling.

Proposal 7: The PDU set QoS parameter and characteristics and PDU set information may also need to be transmitted between source gNB and target gNB, MN and SN, or CU and DU.

- Conclusion

In this contribution, we discuss XR-awareness. According to above observations, we have the following proposals:

Proposal 1: UE transmits the UL maximum jitter range value relative to the periodical arrival time to NG-RAN.

Proposal 2: UE transmits the UL burst arrival time similar as timeoffset-16, which indicates the timing offset with the respect to subframe#0 of SFN#0 in milliseconds to NG-RAN.

Proposal 3: RAN2 to discuss how to report the UL burst arrival time to MCG/SCG in case of NR-DC and to target cell in case of handover.

Proposal 4: UE transmits the UL periodicity together with the jitter range to NG-RAN.

Proposal 5: RAN2 to discuss new trigger conditions or new cancel conditions of BSR to indicate “zero-BSR” to the gNB as an indication of end of data burst for UL.

Proposal 6: Confirm that NW can send UE to DRX inactive mode according to the End of Data Burst indication.

Proposal 7: The PDU set QoS parameter and characteristics and PDU set information may also need to be transmitted between source gNB and target gNB, MN and SN, or CU and DU.

- References

RAN2#122 chairman note.

R2-2305684, Discussion on PDU sets and data burst awareness in RAN, Lenovo