3GPP TSG-RAN WG2 Meeting #123	R2- 2308029

Toulouse, France, August 21-25, 2023                       revision of R2- 2305720

Agenda item:		7.20.2

Source:		Lenovo

Title:		Discussion on the UE-initiated RACH procedure in multi-TRP operation

Document for:		Discussion and Decision

# Introduction

Some agreements on multi-TRP have been achieved in RAN2#122 [1].

Agreement

Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

RAN2 confirm the following working assumption as an agreement:

- Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles:

    - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs).

- The following information can be indicated by the MAC CE (for joint DL/UL TCI mode):

    - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs,

    - if  the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

FFS for the separate DL/UL TCI mode.

In this contribution, we discuss the cases for obtaining of the two TAs in multi-DCI multi-TRP operation.

# Discussion

In legacy, a RACH is initiated by a PDCCH order, by the MAC entity itself, or by RRC. A RACH can be initiated to obtain the TA when the UL data arrive at the UE in RRC_CONNECTED state while the UL is unsynchronized. Considering multi-DCI multi-TRP operation, the UE shall be in the RRC_CONNECTED state. Referring to the legacy procedure to maintain the UL TA, RRC configures the timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned. Also, the gNB can determine whether the TA is maintained according to the timeAlignmentTimer in legacy. Since two TAs are introduced in multi-TRP operation, the timeAlignmentTimer per TAG can be introduced correspondingly in order to flexibly control the UL time alignment.

Observation 1: The gNB can determine whether the UL time per TRP is aligned e.g. according to the expiry of TAT per TAG.

One TA of two TAs is valid

It has been agreed in RAN1 that for multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases. Then, the gNB can trigger a PDCCH ordered CFRA to update the TA for the TRP if the gNB identifies that the uplink time for one TRP is not aligned and determines to recover the UL time alignment.

On the other side, the SR configuration is not restricted by the TRP. In other words, the SR is not configured to be associated with any TRP. Then the UE can transmit SR if only one TA for one TRP is maintained when UL data arrives. The gNB can determine whether to recover the UL time alignment for the unsynchronized TRP, for example based on the radio condition and the BSR.

Observation 2: UE can transmit SR upon UL data arrival when only one TA of two TAs is maintained.

Observation 3: The gNB can trigger the UE to obtain the TA for the unsynchronized TRP via PDCCH order.

Neither TA is valid

The gNB cannot identify UL data arrive and the UE cannot transmit SR if both of the UL for the TRPs are unsynchronized. In legacy, the UE initiates a RACH for this case. The UE-initiated RACH procedure can also work when both of the UL for the TRPs are unsynchronized.

Proposal 1: RACH procedure can be initiated by UE when both of the uplink time for the TRPs are not aligned and UL data arrives.

RACH procedure can be initiated by UE when both TATs to two TAs expire, and UL data arrives. In RACH procedure, a preamble will be transmitted in a RACH association related to one TRP. Then, the serving cell can calculate the TA value for the TRP receiving the preamble. In one RACH, the TA for the second TRP cannot be obtained. Therefore, if UE wants to keep synchronization towards two TRP, two RACH procedures are needed. However, the DL/UL transmission can be resume once one of two TRP can be recovered. The first issue is which TRP should be recovered first if both TRPs are unsynchronized.

Observation 4: one RACH procedure can be used to recover only one TRP.

Proposal 2: RAN2 to discuss which TRP should be recovered first when RACH procedure is triggered because of UL data arrival and uplink time not aligned.

# Conclusion

In this contribution, we propose the followings for obtaining of the two TAs in multi-DCI multi-TRP operation in Rel-18 NR:

Observation 1: The gNB can determine whether the UL time per TRP is aligned e.g. according to the expiry of TAT per TAG.

Observation 2: UE can transmit SR upon UL data arrival when only one TA of two TAs is maintained.

Observation 3: The gNB can trigger the UE to obtain the TA for the unsynchronized TRP via PDCCH order.

Proposal 1: RACH procedure can be initiated by UE when both of the uplink time for the TRPs are not aligned and UL data arrives.

Observation 4: one RACH procedure can be used to recover only one TRP.

Proposal 2: RAN2 to discuss which TRP should be recovered first when RACH procedure is triggered because of UL data arrival and uplink time not aligned.

# References

RAN2#122 chairman notes.