3GPP TSG RAN WG2 Meeting #123               	R2-2308161

Toulouse, France, 21 – 25 August 2023

Agenda Item:	7.9.2

Source: 			Sony

Title:				Discussions on DRX for Sidelink UE to UE Relay

Document for:	Discussion

# Introduction

In this document we discuss the UE to UE Relay and that the power consumption of the Relay node is important. Sidelink UE to Network Relay links can be configured with sidelink DRX but not UE to UE Relay links. Based on that we think that the DRX scheme for UE to UE Relay links should also be considered.

There have not been any recent agreements related to UE power consumption when U2U Relay sidelinks are used. Especially when a Relay UE is a Relay (U2U or U2N) with several Remote UEs, we have not discussed the impact on the Relay UE. Since power consumption is important and relevant, also for the Relay UE, we propose that we discuss it.

# Discussion

  - DRX for sidelink U2U Relay

    - Background

In this contribution we discuss the power consumption of UE-UE relay when configured with sidelink DRX. The usage of DRX for UE to NW relay was agreed to be included in Rel-17 similar as for sidelink unicast communication, and in objective 4 in the Rel-18 WID [1], based on company contributions in the RAN plenary meeting, RAN#96 in June 2022.

The situation for a Relay UE, with more than one Remote UE connected to it, is different from two UEs with unicast communication, therefore this UE needs to be treated with special care. The timing of the DRX when a Relay UE is connected to multiple Remote UEs has been discussed for the U2N Relays. This discussion should continue also taking the U2U Relay into account.

    - UE-to-UE Relay configured with DRX

Currently SL DRX is not specified for the U2U sidelink relay, but it is specified for U2N Relays as well as unicast communication.  In the legacy procedure in Sidelink for unicast communication the Tx UE decides the Sidelink DRX configuration.

Assuming a specific UE acts as a U2U relay. This Relay UE may have several Remote UEs connected. The Relay UE may be in any RRC state on the Uu interface: Out of Coverage, RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED state.

DRX is currently not discussed for U2U Relay. When a UE acts as a relay UE for UEs rarely sending data this UE should not need to listen continuously to all SL communication. In order to save power, DRX should be specified for U2U Relay UEs as well as the U2N case.

Observation 1: Currently the SL DRX is applicable for UE-to-UE sidelink communication and UE to NW Relay UE sidelink communication. It is however not discussed for the UE-to-UE Relay sidelink communication. This communication is equally power sensitive as the other cases.

Proposal 1: Add support of SL DRX for UE-to-UE Relay sidelink communication.

    - SL DRX timing for UE-to-UE Relay UE configured with DRX

When the Relay UE is acting as a relay for several Remote UEs where some Remote UEs uses DRX the UE needs to be awake during active time on the Uu interface to the gNB, if it is in coverage, as well as on the active times of each of the PC5 links. When these DRX timings to the different UEs are aligned the power consumption of the Relay UE can be improved substantially compared with if they are distributed over time. If the remote UEs start to transmit when connecting to the Relay UE the remote UEs decide the DRX timing of that link, thereby the Relay UE needs to be awake at different times for different Remote UEs. The Remote UEs are either in out of coverage state or may have a long eDRX period if they are power sensitive. When the Remote UE is only connected on sidelink to the Relay UE the exact timing of the sidelink DRX is not as important for the Remote UE as it is for the Relay UE. Based on that it is beneficial if the Relay UE can decide the DRX timing to align its DRX cycles. In Rel-17 SL-DRX, the timing is decided by the Tx UE which in many cases is the Remote UE who wants to transmit data via the Relay UE.

Observation 2: The Power consumption of a Sidelink Relay UE with more than one Remote UE connected would benefit if the Relay UE can configure the DRX timing.

Proposal 2: Support that Relay UEs decides the timing of the DRX active time if it has limited power resources.

# Conclusion

Based on the discussion above we would like to have a discussion of the proposals and possible solutions for Sidelink DRX above for UE-to-UE Relay.

Below our observations

Observation 1: Currently the SL DRX is applicable for UE-to-UE sidelink communication and UE to NW Relay UE sidelink communication. It is however not discussed for the UE-to-UE Relay sidelink communication. This communication is equally power sensitive as the other cases.

Observation 2: The Power consumption of a Sidelink Relay UE with more than one Remote UEs connected would benefit if the Relay UE can configure the DRX timing.

Our proposals are stated here

Proposal 1: Add support of SL DRX for UE-to-UE Relay sidelink communication.

Proposal 2: Support that Relay UEs decides the timing of the DRX active time if it has limited power resources. That can be part of the timing negotiation.

# References

- RP-223501, Revised WID on NR sidelink relay enhancements