3GPP TSG-RAN WG2 Meeting #123	R2-2308227

Toulouse, France, 21 – 25 August 2023

Agenda item:	4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Reference point for UTC timing in SIB16(-NB)

WID/SID:	LTE_NBIOT_eMTC_NTN - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In the post-email discussion 102 for RAN2#122 ([Post122][102][IoT NTN] UTC reference point) the following question was discussed [R2-2306926]

Question 1: Do companies agree that the reference point of the propagation delay for timing info in SIB16(-NB) should be the uplink time synchronization reference point (i.e., RP)?

However, no conclusion was reached besides

Proposal 1: Postpone the discussion of this CR to the next meeting.

In this contribution we provide our view on the topic.

# 2	Reference point for UTC timing in SIB16(-NB)

The SIB16(-NB) includes the timeInfoUTC information element, which defines the UTC time with a granularity of 10 ms. Furthermore, there is an optional information element timeReferenceInfo which includes the field time. That field has a granularity of 0.25 µs with reference to the network i.e., there is no compensation for the RF propagation delay from network-side. See TS 36.331:

SystemInformationBlockType16 information element -- ASN1START SystemInformationBlockType16-r11 ::= SEQUENCE { timeInfo-r11 SEQUENCE { timeInfoUTC-r11 INTEGER (0..549755813887), dayLightSavingTime-r11 BIT STRING (SIZE (2)) OPTIONAL, -- Need OR leapSeconds-r11 INTEGER (-127..128) OPTIONAL, -- Need OR localTimeOffset-r11 INTEGER (-63..64) OPTIONAL -- Need OR } OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ timeReferenceInfo-r15 TimeReferenceInfo-r15 OPTIONAL -- Need OR ]] } -- ASN1STOP timeInfoUTC Coordinated Universal Time corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which SystemInformationBlockType16 is transmitted. The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). NOTE 1. This field is excluded when estimating changes in system information, i.e. changes of timeInfoUTC should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1. TimeReferenceInfo information elements -- ASN1START TimeReferenceInfo-r15 ::= SEQUENCE { time-r15 ReferenceTime-r15, uncertainty-r15 INTEGER (0..12) OPTIONAL, -- Need OR timeInfoType-r15 ENUMERATED {localClock} OPTIONAL, -- Need OR referenceSFN-r15 INTEGER (0..1023) OPTIONAL -- Cond TimeRef } ReferenceTime-r15 ::= SEQUENCE { refDays-r15 INTEGER (0..72999), refSeconds-r15 INTEGER (0..86399), refMilliSeconds-r15 INTEGER (0..999), refQuarterMicroSeconds-r15 INTEGER (0..3999) } -- ASN1STOP time, timeInfoType This field indicates time reference with 0.25 us granularity. The indicated time is referenced at the network, i.e., without compensating for RF propagation delay. The indicated time in 0.25 us unit from the origin is refDays*86400*1000*4000 + refSeconds*1000*4000 + refMilliSeconds*4000 + refQuarterMicroSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from the origin of the time field. If timeInfoType is not included, the origin of the time field is 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If timeInfoType is set to localClock, the interpretation of the origin of the time is unspecified and left up to upper layers. If time field is included in SystemInformationBlockType16, this field is excluded when estimating changes in system information, i.e. changes of time should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1.

Observation 1: The high-accuracy timeReferenceInfo information elements are transmitted by the network without RF propagation delay compensation.

An initial question is what companies expect to do with the SIB16(-NB), when considering that GNSS is a mandatory component of NTN release 17 [RP-211601]: “GNSS capability in the UE is taken as a working assumption for both NB-IoT and eMTC devices.” The GNSS can also provide UTC time information with high granularity.

Observation 2: The purpose of changing the reference point for the UTC timing in SIB16(-NB) is unclear, because IoT NTN devices are equipped with GNSS, which can provide similar or even more accurate timing information.

The SIB16 has a NR counterpart in SIB9. The SIB9 also provides a timeReferenceInfo information element, but with an even finer granularity of 10 ns. In order to achieve this accuracy, the network can provide the UE with a propagation delay estimate based on the RAN RTT-based measurement (similar to the multi-RTT functionality of the positioning method). This information is obviously provided on a UE-basis and not in the SIB.

Such propagation delay signalling is not feasible in NTN, because the UE-specific propagation delay changes over time due to the satellite movement.

Observation 3: To achieve the accuracy of timeReferenceInfo UE-specific signalling on the propagation delay can be provided from NW to UE in NR for terrestrial networks, but it is not feasible for NTN.

It may be considered whether the UE can estimate the propagation delay towards the reference point instead.

If the reference point is in the eNB, the kmac is a limiting factor, because it has a 1 ms granularity as per the SIB31 definition in TS 36.331.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms.

Thus, it is sufficient for the timeInfoUTC (a factor 10 better), but insufficient for the timeReferenceInfo.

Furthermore, the UE would still need to determine the propagation delay on the service link and the part of the feeder link between the satellite and the reference point (independent of whether it is in the eNB or some arbitrary place).

During the RAN1 release 17 discussions the accuracy of the satellite assistance information was discussed. For example, R1-2201011 notes the ephemeris after 60 seconds will results in a satellite position estimation error of 9.3 m - 33.9 m for the position & velocity vector option (for NUM6x6 and Kepler propagation models in the UE) and 4.2 m – 30.6 m for the orbital option (for NUM6x6 and Kepler propagation models in the UE). Thus, the accuracy in terms of timing is 14 ns – 113 ns. Likewise, there will be an error in the common TA estimation (covering the feeder link delay) and the UE GNSS position. For example, R1-2109165 estimates the common TA error is 8 µs just 20 seconds after receiving the SIB containing the common TA parameters (for a satellite at 60 degrees elevation angle relative to the UE). The estimates are made using a second order approximation without considering the quantization error introduced when providing the parameters in the system information. This means it is challenging for the UE to compensate the propagation delay with sufficient accuracy to fully exploit the accuracy of the timeReferenceInfo.

This challenge is also reflected in the transmit timing error requirement (TS 36.133) which for NB-IoT over NTN is  97∙Ts=97∙115000∙2048=3.16 µs. This is more than a factor 10 higher than the time granularity of the timeReferenceInfo in SIB16. For comparison, the NB-IoT over TN timing error limit is 80∙Ts i.e. the additional error allowed in NTN is 17∙115000∙2048=0.55 µs, which is still twice as large as the granularity of the timeReferenecInfo.

Observation 4: The RAN4 transmit timing error requirement for NB-IoT over NTN is 3.16 µs because of the DL timing estimation error based on ephemeris, common TA and GNSS position.

In summary, the UE is allowed to have significantly worse transmit timing accuracy than the timeReferenceInfo and it is clear that the UE may not be able to compensate properly for the propagation delay as is expected when using timeReferenceInfo. Therefore, it is clear that UE can only apply the timeInfoUTC, which 10 ms accuracy, because the propagation delay estimate is more accurate than this granularity.

Observation 5: The combined accuracy of ephemeris, common TA, and UE GNSS position is significantly worse than the timeReferenceInfo and therefore the UE may not be able to apply timeReferenceInfo.

Observation 6: The UE can determine the timeInfoUTC with sufficient accuracy because the potential inaccuracies in the propagation delay estimate are significantly smaller.

Since the UE is not able to properly compensate for the propagation delay and therefore not able to apply the timeReferenceInfo with an accuracy corresponding to the accuracy of the timeReferenceInfo, there is no benefit in RAN2 defining a shift of the reference point for the SIB16 including the timeReferenceInfo. Independent of the reference point location, the UE can apply the timeInfoUTC and thus it is pointless to start shifting the reference point from eNB to RP, when it does not change the UE’s understanding of the timeInfoUTC content. Finally, it is also worth noting that a change of the reference point would have high impact on the eNB implementation, because the eNB continuously need to calculate what the time is in the reference point before transmitting the information from the eNB, and possibly result in the optional field timeReferenceInfo would not be transmitted.

Observation 7: A shift of the reference point for SIB16(-NB) will have large impact on eNB implementation.

Proposal 1: RAN2 to agree to maintain the legacy reference point in the network for SIB16(-NB).

RAN2 may further discuss whether a note is needed to clarify that the UE can estimate the propagation delay in order to apply the timeInfoUTC. A text proposal for TS 36.331 is provided in the appendix.

Proposal 2: RAN2 to discuss to add a note to SIB16(-NB) as follows “In an NTN cell, the UE should compensate for the service link and feeder link propagation delays to obtain accurate timing via timeInfoUTC.”

Proposal 3: RAN2 to agree the TP as included in the Appendix.

# 3	Conclusion

This document has made the following observations:

Observation 1: The high-accuracy timeReferenceInfo information elements are transmitted by the network without RF propagation delay compensation.

Observation 2: The purpose of changing the reference point for the UTC timing in SIB16(-NB) is unclear, because IoT NTN devices are equipped with GNSS, which can provide similar or even more accurate timing information.

Observation 3: To achieve the accuracy of timeReferenceInfo UE-specific signalling on the propagation delay can be provided from NW to UE in NR for terrestrial networks, but it is not feasible for NTN.

Observation 4: The RAN4 transmit timing error requirement for NB-IoT over NTN is 3.16 µs because of the DL timing estimation error based on ephemeris, common TA and GNSS position.

Observation 5: The combined accuracy of ephemeris, common TA, and UE GNSS position is significantly worse than the timeReferenceInfo and therefore the UE may not be able to apply timeReferenceInfo.

Observation 6: The UE can determine the timeInfoUTC with sufficient accuracy because the potential inaccuracies in the propagation delay estimate are significantly smaller.

Observation 7: A shift of the reference point for SIB16(-NB) will have large impact on eNB implementation.

And proposed the following:

Proposal 1: RAN2 to agree to maintain the legacy reference point in the network for SIB16(-NB).

Proposal 2: RAN2 to discuss to add a note to SIB16(-NB) as follows “In an NTN cell, the UE should compensate for the service link and feeder link propagation delays to obtain accurate timing via timeInfoUTC.”

Proposal 3: RAN2 to agree the TP as included in the Appendix.

# Appendix

Text proposal for TS 36.331

### 6.3.1 System information blocks #### – SystemInformationBlockType16 The IE SystemInformationBlockType16 contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time. NOTE: The UE may use the time information for numerous purposes, possibly involving upper layers e.g. to assist GPS initialisation, to synchronise the UE clock (a.o. to determine MBMS session start/ stop). SystemInformationBlockType16 information element -- ASN1START SystemInformationBlockType16-r11 ::= SEQUENCE { timeInfo-r11 SEQUENCE { timeInfoUTC-r11 INTEGER (0..549755813887), dayLightSavingTime-r11 BIT STRING (SIZE (2)) OPTIONAL, -- Need OR leapSeconds-r11 INTEGER (-127..128) OPTIONAL, -- Need OR localTimeOffset-r11 INTEGER (-63..64) OPTIONAL -- Need OR } OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ timeReferenceInfo-r15 TimeReferenceInfo-r15 OPTIONAL -- Need OR ]] } -- ASN1STOP SystemInformationBlockType16 field descriptions. dayLightSavingTime It indicates if and how daylight saving time (DST) is applied to obtain the local time. The semantics is the same as the semantics of the Daylight Saving Time IE in TS 24.301 [35] and TS 24.008 [49]. The first/leftmost bit of the bit string contains the b2 of octet 3, i.e. the value part of the Daylight Saving Time IE, and the second bit of the bit string contains b1 of octet 3.. leapSeconds Number of leap seconds offset between GPS Time and UTC. UTC and GPS time are related i.e. GPS time -leapSeconds = UTC time.. localTimeOffset Offset between UTC and local time in units of 15 minutes. Actual value = field value * 15 minutes. Local time of the day is calculated as UTC time + localTimeOffset.. timeInfoUTC Coordinated Universal Time corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which SystemInformationBlockType16 is transmitted. The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). NOTE 1. This field is excluded when estimating changes in system information, i.e. changes of timeInfoUTC should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1.. NOTE 1: The UE may use this field together with the leapSeconds field to obtain GPS time as follows: GPS Time (in seconds) = timeInfoUTC (in seconds) - 2,524,953,600 (seconds) + leapSeconds, where 2,524,953,600 is the number of seconds between 00:00:00 on Gregorian calendar date 1 January, 1900 and 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). NOTE 2: In an NTN cell, the UE should compensate for the service link and feeder link propagation delays to obtain accurate timing via timeInfoUTC.