3GPP TSG-RAN WG2 Meeting #123                                                                        R2-2308278

Toulouse, France, August 21-25, 2023

Agenda Item	:	7.5.3

Title	:	XR-specific power saving enhancement

Source	:	Google Inc.

Document for	:	Discussion and Decision

# Introduction

In this Tdoc, we discuss the DRX support of XR frame rates corresponding to non-integer periodicities.

In RAN2#121bis-e, the following agreement has been reached:

RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration.

In RAN2#122, the following agreement has been reached:

Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications.

In this Tdoc, we share our views on the signaling details for the use of rational number DRX cycles for XR.

# C-DRX Enhancement

In the MAC specification [TS 38.321], the formulas to start On-Duration timer are:

- For long DRX cycle: [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset , where SFN is the current System Frame Number, and the subframe number is the current subframe number.

- For short DRX cycle : [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle) .

In R2-2304709, Option 1 has been promoted as a solution for supporting DRX cycles expressed as rational numbers.

The following changes to TS 38.321 have been proposed in R2-2304709:

1> if the Short DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-ShortCycle)) = floor ((drx-StartOffset) modulo (drx-ShortCycle)): 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX cycle is used for a DRX group, and floor ([(SFN × 10) + subframe number] modulo (drx-LongCycle)) = drx-StartOffset: NOTE: If drx-ShortCycle or drx-LongCycle is not an integer, the modulo operation should be implemented by a method that does not introduce rounding errors. For example, (drx-StartOffset) modulo (drx-ShortCycle) can be implemented as drx-StartOffset – drx-ShortCycle × floor (drx-StartOffset / drx-ShortCycle). (The rest of the legacy text is omitted…)

The modulo definition: A mod B = A - B * floor(A/B), was proposed to implement the modulo operation without introducing rounding errors.

Hence, floor ([(SFN × 10) + subframe number] modulo (drx-ShortCycle)) can be written as

floor ([(SFN × 10) + subframe number] modulo (drx-ShortCycle)) = floor([(SFN × 10) + subframe number] - drx-ShortCycle * floor ([(SFN × 10) + subframe number] / drx-ShortCycle ) )

Assuming drx-ShortCycle = F fps, then floor ([(SFN × 10) + subframe number] / drx-ShortCycle ) can be implemented as:

Formula 1:

floor([(SFN × 10) + subframe number] - drx-ShortCycle * floor ([(SFN × 10) + subframe number] / ( 1/F *1000) ) ) = floor ((drx-StartOffset) modulo (drx-ShortCycle))

Formula 2:

floor([(SFN × 10) + subframe number] - drx-ShortCycle * floor ([(SFN × 10) + subframe number] /1000*F ) ) = floor ((drx-StartOffset) modulo (drx-ShortCycle))

Note 1: results above are obtained with MATLAB and reproduced with both short format precision (4 digits after the decimal point) and long format precision (15 digits after the decimal point).

Note 2: The error is calculated as XRstart-StartOnDuration. Where  XRstart is the expected start time of the XR traffic (e.g. for FPS = 60, XRstart=[0,  16.66, 33.33, 50,  66.66, … ]) and the StartOnDuration is the subframe index verifying Formula 1 or Formula 2.

Observation 1: The proposed method to implement the modulo operation (A mod B = A - B * floor(A/B) ) introduces rounding errors.

Proposal 1: The proposed method to implement the modulo operation operation (A mod B = A - B * floor(A/B) ) introduces rounding error due to the floating point error. This error can be reduced by carefully considering the order of the operations inside the floor calculation.

Formula 3:

floor([(SFN × 10) + subframe number] - drx-ShortCycle * floor ([(SFN × 10) + subframe number]*F /1000 ) ) = floor ((drx-StartOffset) modulo (drx-ShortCycle))

Formula 3 however seems to address the precision issue by multiplying the subframe number by the FPS before the division by 1000. This issue can also be addressed with UE implementation; however, it is preferred to address the issue in the specifications to avoid different UE implementations.

# Conclusion

In this contribution, we made the following proposals

Observation 1: The proposed method to implement the modulo operation (A mod B = A - B * floor(A/B) ) introduces rounding errors.

Proposal 1: The proposed method to implement the modulo operation operation (A mod B = A - B * floor(A/B) ) introduces rounding error due to the floating point error. This error can be reduced by carefully considering the order of the operations inside the floor calculation.

# References

- S4aV220921 “Reply LS on further details on XR traffic” 3GPP TSG-WG SA4 VIDEO SWG call, E-meeting October 2022.